Prepared by the experts from CLEPA

GRRF AEBS/LDWS Informal Working Group

13th Meeting, 22nd March 2011

6.9.	   False reaction test

6.9.1.	Two stationary vehicles, of category M1 AA saloon, shall be positioned:

(a)	with the vehicle [centre line in the centre of a lane / located at the borderline closest to the lane of travel of the subject vehicle] to the right and left of the subject vehicle lane,

(b)	[so as to face in the same direction of travel as the subject vehicle,]

(b)	with a distance of 5.2 m between them1,

(c)	with the rear of each vehicle aligned with the other,

where all three lanes are 3.5m wide.

6.9.2.	The subject vehicle shall travel for a distance of at least 60m, at a constant speed of 50 ± 2 km/h, in a straight line, in the centre of the centre lane before passing between the 2 stationary vehicles.

During the test there shall be no adjustment of any subject vehicle control other than slight steering adjustments to counteract any drifting.

6.9.3.	The AEBS shall not provide a collision warning and shall not initiate the emergency braking phase."

1 The point of reference of each stationary vehicle for establishing the distance between
  the two stationary vehicles, shall be determined in accordance with ISO 612-1978
  paragraph 6.2.

Justification:

The use of 3 lanes of a specified width and the positioning of the stationary vehicles in the centre of the outer lanes means that the distance between the stationary vehicles is variable.

By specifying a specific value for the distance between the stationary vehicles there is standard test requirement.