Amendment to ECE/TRANS/WP29/GRRF/2011/23, 24, 25 and26

A.   Proposal

5.1.1 of GRRF/2011/23, 24, 25 and26 ,amend to read

5.1.1. Any vehicle fitted with an AEBS-M complying with the definition of paragraph 2.1.1. shall meet the performance requirements contained in paragraphs 5.1. to 5.6.2. of this Regulation and shall be equipped with an anti-lock braking function in accordance with the performance requirements of Annex 13 of Regulation No.13 [and a Vehicle Stability Function in accordance with the performance requirements of Annex 21 of Regulation No.13].

Annex 3 of ECE/TRANS/WP29/GRRF/2011/24 and 25, amend to read

Warning and activation test requirements – pass/fail values

| Class | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stationary target |  |  | Moving target |  |  |  |
|  |  | Timing of warning modes |  | Speed reduction (ref. paragraph 6.4.4.) | Timing of warning modes |  | Speed reduction (ref. paragraph 6.5.3.) | Target speed (ref. paragraph 6.5.1.) |
|  |  | At least 1 haptic or acoustic (ref. paragraph 6.4.2.1.) | At least 2 (ref. paragraph 6.4.2.2.) |  | At least 1 haptic or acoustic (ref. paragraph 6.5.2.1.) | At least 2 (ref. paragraph 6.5.2.2.) |  |  |
| 1 | M3 and Ｎ3 1/ | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | Not less than 10 km/h | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | No impact | 32 ± 2 km/h |
| 2 | M2 and N2 |  |  |  |  |  |  |  |

1/　 Class 2 requirement is applied to vehicles with pneumatic-hydraulic braking system (AoH)

and vehicles with hydraulic braking system

Annex 3 of ECE/TRANS/WP29/GRRF/2011/23 and 26, amend to read

| Class | A | B |  | C |  | D | E |  | F |  | G |  | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stationary target |  |  |  |  | Moving target |  |  |  |  |  |  |
|  |  | Timing of warning modes |  |  | Speed reduction (ref. paragraph 6.4.4.) |  | Timing of warning modes |  |  | Speed reduction (ref. paragraph 6.5.3.) |  | Target speed (ref. paragraph 6.5.1.) |  |
|  |  | At least 1 haptic or acoustic (ref. paragraph 6.4.2.1.) | At least 2 (ref. paragraph 6.4.2.2.) |  |  |  | At least 1 haptic or acoustic (ref. paragraph 6.5.2.1.) | At least 2 (ref. paragraph 6.5.2.2.) |  |  |  |  |  |
| 1 | M3 and N3 1/ | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase |  | Not less than 20 km/h [2/] |  | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase |  | No impact |  | 12 ± 2 km/h [3/] |  |
| 2 | M2 and N2 |  |  |  |  |  |  |  |  |  |  |  |  |

1/　Class 2 requirement is applied to vehicles with hydraulic braking system.

[2/  Not less than 10km/h is applied for speed reduction to vehicles with pneumatic-hydraulic braking system (AoH) .]

[3/  32 ± 2km/h is applied for target speed to vehicles with pneumatic-hydraulic braking system (AoH) .]

B.   Justification

With regards to the necessity of equipment of EVSC and (Rear) Air Suspension for AEBS, there is a different opinion among different countries and regions on reflecting their market. It would be appropriate to decide this judgment　on　national or regional basis. Therefore the relevant prescriptions are deleted.

With regards to the different requirement considering the capability of brake system, the basic classification by vehicle category is introduced instead of brake system in column A. And some foot notes are added to adjust the discrepancy between vehicle category and brake system.