Proposed amendments to Annex 3 of document ECE/TRANS/WP.29/GRRF/2011/23
(AEBS – collision avoidance regulation)

Introducing comment:

There is a lack of experience with regard to the driver/vehicle behaviour characteristics of AEBS equipped N2 category vehicles, and a significant difference in vehicle dynamics between the higher and lower ends of the weight range. Therefore, while CLEPA can support an AEBS requirement – the same as for N3 category vehicles – at the higher end of the weight range, it can not support an AEBS requirement including an emergency braking phase for vehicles at the lower end of the weight range, i.e. those with a vacuum hydraulic braking system. Should it be required to include such vehicles in the regulation, it is recommended that the mandatory requirement should only be for a warning. Any automatic emergency braking would be optional.

Proposed amendment:

Annex 3

Warning and activation test requirements – pass/fail values

| A | B | C | D | E | F | G | H |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stationary target |  |  | Moving target |  |  |  |  |
|  | Timing of warning modes |  | Speed reduction (ref. paragraph 6.4.4.) | Timing of warning modes |  | Speed reduction (ref. paragraph 6.5.3.) | Target speed (ref. paragraph 6.5.1.) |  |
|  | At least 1 haptic or acoustic (ref. paragraph 6.4.2.1.) | At least 2 (ref. paragraph 6.4.2.2.) |  | At least 1 haptic or acoustic (ref. paragraph 6.5.2.1.) | At least 2 (ref. paragraph 6.5.2.2.) |  |  |  |
| M3 and N3 vehicles with a pneumatic braking system [pneumatic rear axle suspension] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | Not less than [10/20/50 km/h] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | No impact | [12 ± 2 km/h] | 1 |
| M3 vehicles with a Vacuum-hydraulic braking system | No restriction regarding timing and warning modes 1 | No restriction regarding timing and warning modes 1 | 0 km/h 2 | No time restriction | Not later than 1.4 s. before start of emergency braking phase 3 | 0 km/h 4 | 12 ± 2 km/h | 2 |
| M2 and N2 vehicles with a pneumatic braking system [pneumatic rear axle suspension] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | Not less than [10/20/50 km/h] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | No impact | [12 ± 2 km/h] | 3 |
| M2 and , N2 , [M3 and N3]vehicles with an pneumatic-hydraulic braking system (AoH) [pneumatic rear axle suspension] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | Not less than [10/20/50 km/h] | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | No impact | [32 ± 2 km/h] 12 ± 2 km/h | 4 |
| M2 and N2 vehicles with a Vacuum-hydraulic braking system | No restriction regarding timing and warning modes 1 | No restriction regarding timing and warning modes 1 | 0 km/h 2 | No time restriction | Not later than 1.4 s. before start of emergency braking phase 3 | 0 km/h 4 | 12 ± 2 km/h | 5 |

Notes: 1 For zero speed reduction, start of emergency braking phase is time to collision. In this case, paragraph 6.4.3. does not apply.

2 Speed reduction optional.

3 For zero speed reduction, start of emergency braking phase is time to collision.

4 In this case, paragraph 6.5.3. does not apply. Speed reduction optional.