Proposed single AEBS regulation 
(document ECE/TRANS/WP.29/GRRF/2011/25)
 and its proposed concurrently running 01 series of amendments 
(document ECE/TRANS/WP.29/2011/26)

By this document, OICA is keen to correct the formula put forward by the expert from CLEPA in document AEBS/LDWS-14-08.

Correction to the formula:

CLEPA used an incorrect formula for calculating the energy reduction:

Correct formula:

m = vehicle mass [kg]
v0 = initial vehicle speed [m/s]
v1 = final vehicle speed [m/s]

| Vehicle speed [km/h] | 80 | 32 | 12 |  | 68 | 48 |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle mass [kg] | 40000 | 40000 | 40000 |  | 40000 | 40000 |
| Vehicle speed [m/s] | 22,22222 | 8,888889 | 3,333333 |  | 18,88889 | 13,33333 |
| Kinetic energy [kJ] | 9876,543 | 1580,247 | 222,2222 |  | 7135,802 | 3555,556 |
| Reduced kinetic energy (from 80 km/h) [kJ] | 0 | 8296,296 | 9654,321 |  |  |  |
| Reduced kinetic energy (from 80 km/h) [%] | 0 | 84% | 98% |  |  |  |

OICA Conclusion:

Correct kinetic energy reduction:

Step 1:         8296 kJ, 84% reduction

Step 2:         9654kJ, 98% reduction

Step 2 is 16% better than step1   (Not 100% better)

(9654-8296)/8296 = 16%

---------