Amendment to ECE/TRANS/WP29/GRRF/2011/23, 24, 25 and26

A. Proposal

Annex 3 of ECE/TRANS/WP29/GRRF/2011/24 and 25, amend to read

Warning and activation test requirements – pass/fail values

| Class | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stationary target |  |  | Moving target |  |  |  |
|  |  | Timing of warning modes |  | Speed reduction (ref. paragraph 6.4.4.) | Timing of warning modes |  | Speed reduction (ref. paragraph 6.5.3.) | Target speed (ref. paragraph 6.5.1.) |
|  |  | At least 1 haptic or acoustic (ref. paragraph 6.4.2.1.) | At least 2 (ref. paragraph 6.4.2.2.) |  | At least 1 haptic or acoustic (ref. paragraph 6.5.2.1.) | At least 2 (ref. paragraph 6.5.2.2.) |  |  |
| 1[1/] | M3 and Ｎ3 and N2＞[8 / 7.5]t | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | Not less than 10 km/h | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase | No impact | 32 ± 2 km/h |
| 22/ | M2 and N2≦[8 / 7.5]t | [No restriction regarding timing and warning modes] or [Blank] | [No restriction regarding timing and warning modes] or [Blank] | [0 km/h] or [Blank] | [No time restriction] or [Blank] | [Not later than 1.4 s. before start of emergency braking phase] or [Blank] | [0 km/h] or [Blank] | [32 ± 2 km/h] or [Blank] |

[1/  Class 1 requirement is also applied with regards to vehicles, which belong to N2 of equal or less than [8 / 7.5]t, with pneumatic braking system.]

2/　 Class 2 requirement is also applied with regards to vehicles, which belong to M3, N3 and N2 of more than [8 / 7.5]t, with [pneumatic-hydraulic braking system (AoH) and] hydraulic braking system.

Annex 3 of ECE/TRANS/WP29/GRRF/2011/23 and 26, amend to read

| Class | A | B |  | C |  | D | E |  | F |  | G |  | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stationary target |  |  |  |  | Moving target |  |  |  |  |  |  |
|  |  | Timing of warning modes |  |  | Speed reduction (ref. paragraph 6.4.4.) |  | Timing of warning modes |  |  | Speed reduction (ref. paragraph 6.5.3.) |  | Target speed (ref. paragraph 6.5.1.) |  |
|  |  | At least 1 haptic or acoustic (ref. paragraph 6.4.2.1.) | At least 2 (ref. paragraph 6.4.2.2.) |  |  |  | At least 1 haptic or acoustic (ref. paragraph 6.5.2.1.) | At least 2 (ref. paragraph 6.5.2.2.) |  |  |  |  |  |
| 1[1/] | M3, N3 and N2＞[8 / 7.5]t | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase |  | Not less than 20 km/h |  | Not later than 1.4 s. before start of emergency braking phase | Not later than 0.8 s. before start of emergency braking phase |  | No impact |  | 12 ± 2 km/h |  |
| 22/ | M2 and N2≦[8 / 7.5]t | [No restriction regarding timing and warning modes] or [Blank] | [No restriction regarding timing and warning modes] or [Blank] |  | [0 km/h] or [Blank] |  | [No time restriction] or [Blank] | [Not later than 1.4 s. before start of emergency braking phase] or [Blank] |  | [0 km/h] or [Blank] |  | [12 ± 2 km/h] or [Blank] |  |

[1/  Class 1 requirement is also applied with regards to vehicles, which belong to N2 of equal or less than [8 / 7.5]t, with pneumatic braking system.]

2/　Class 2 requirement is also applied with regards to vehicles, which belong to M3, N3 and N2 of more than [8 / 7.5]t, with hydraulic braking system.