- HARMONIZED PROVISIONS CONCERNING PNEUMATIC TYRES

- FOR PASSENGER AND LIGHT TRUCK (COMMERCIAL) VEHICLES

Transmitted by the Experts from the ETRTO, JATMA, and RMA

This document summarizes the proposal of amendment to the document TYREGTR-11-02 - (ETRTO, JATMA, RMA) Harmonized provisions concerning pneumatic tyres for passenger and light truck (commercial) vehicles in preparation of the next meeting of the informal group on TYRE gtr on September 16, 2011.

____________

Global Technical Regulation

For

Passenger and Light Truck (Commercial) Tyres

HARMONIZED PROVISIONS CONCERNING PNEUMATIC TYRES

FOR PASSENGER AND LIGHT TRUCK (COMMERCIAL) VEHICLES

CONTENTS

# STATEMENT TECHNICAL RATIONALE AND JUSTIFICATION

Action item: CP to review and agree on document TYREGTR-11-03

# II. 	TEXT OF THE REGULATION

# SCOPE

Action item from the minutes of the last meeting:

The following text was approved “The exemption list will contain only T-Type tyres, tyres having a nominal rim diameter code below or equal to 10 and LT/C tyres with tread depth of greater than 14.3 mm if data are provided by RMA for justification.

- LT or C Tyres with tread-depth of greater than 14.3 mm (18/32 in).”

unless data are provided by RMA to justify keeping exemption on LT or C tyres with tread depth of greater than 14.3mm

RMA response: Proposed requirements for LT/C tyres, including those with tread depth greater than 14.3 mm (18/32 in), are not included in the scope of Phase I, but will be addressed in Phase II.  This will include applicable parts of FMVSS 119 requirements for LT, and C, including for tyres of tread depth greater than 14.3 mm (18/32 in).  US government representative will need to address the above noted issue at that time and is expected that inclusion of applicable parts from FMVSS 119 will be recommended.

# DEFINITIONS

Action item from the minutes of the last meeting:

“Deflected section height”: add the date after the ISO reference.

Proposal from Tyre Industry:

"Deflected section height" is the difference between the deflected radius, measured from the centre of the rim to the surface of the drum, and one half the nominal rim diameter as defined in ISO 4000-1: 2010;

Action item from the minutes of the last meeting: "Load capacity variation with speed": add missing paragraphs 6.2.4 and 6.2.5 from Reg.54.

Proposal from Tyre Industry:

This will be added to paragraph 3.16, Endurance test requirements.

Action item from the minutes of the last meeting: “Load range” definition to be added from FMVSS139.

Proposal from Tyre Industry is to add the following:

"Load range" means a letter (B, C, D, or E) used to identify a given LT size tyre with its load classification and inflation limits;

Action item from the minutes of the last meeting:  “T-Type tyres” definition to be added to understand what is exempted.

Proposal from Tyre Industry is to add the following:

"T-type temporary use spare tyre" means a type of temporary use spare tyre designed for use at inflation pressures higher than those established for standard and reinforced tyres;

# REQUIREMENTS

## Marking

Action item from the minutes of the last meeting: Administrative problem to obtain plant code from NHTSA should be clarified (permanent representative in USA): need to be clarified by NHTSA.

## Other Sidewall Markings

- Action item from the minutes of the last meeting:  “Load range” marking definition to be added from FMVSS139 in a new paragraph 3.3.1.2.3.4.

- Proposal from Tyre Industry:

- 3.3.1.2.3.4	   In the case of LT and C type tyres, the words "Load Range" or "LR" followed by the letter designating the tyre load range "B, C, D, or E".

Action item from the minutes of the last meeting: Maximum inflation pressure (FMVSS139): NHTSA to confirm the content of the following paragraphs 3.3.2 and 3.3.3.

    - For passenger tyres, each tyre must be labelled with its maximum permissible inflation pressure in kPa (psi) and must be labelled with its maximum load rating in kilograms (lbs)..  For standard load and light load tyres:  240 kPa (35 psi), 250 kPa (36 psi), 300 kPa (44 psi) or 350 kPa (51 psi).  For reinforced or extra load tyres:  280 kPa (41 psi), 290 kPa (42 psi) or 340 kPa (50 psi).

    - In the case of LT or C type tyres, the maximum load rating and corresponding inflation pressure of the tyre, shown as follows:

"Max load single ___kg (___lb) at ___kPa (___psi) cold"

"Max load dual ___kg (___lb) at ___kPa (___psi) cold"

For LT and C type tyres rated for single fitment only, mark as follows:

"Max load ___kg (___lb) at ___kPa (___psi) cold"

- Each tyre must be labelled with its maximum load rating, in kilograms (lbs). pursuant to the maximum permissible inflation pressure in the preceding paragraph 3.3.4. above.

## Physical Dimensions of Tyres

Action item from the minutes of the last meeting: Paragraph 3.5.2.1: exclusion of rim protectors from overall width definition in UNECE but included in FMVSS139; need to be checked carefully by NHTSA and why not having a tolerance of 8 mm, as in Reg. 30 is required( See paragraph 3.5.5.1.1. below)?

Proposal from Tyre Industry:

      - The overall width of the tyre is defined as the average of four measurements of its width at the widest point, including any markings or protective ribs.

Action item from the minutes of the last meeting: Paragraph 3.5.3 and 3.5.4: remove “theoretical”.

Proposal from Tyre Industry: After recheck, TI considers that the word “theoretical” should remain for a better understanding. Therefore the text should read:

    - The theoretical section width shall be calculated by the following formula:

S = S1 + K(A-A1),

where:

S is the "theoretical section width" expressed in mm;  ........

    - Outer diameter of tyre

  - The theoretical outer diameter of the tyre shall be calculated by the following formula:

D = d + 2H, where:

D is the theoretical outer diameter in millimetres,  .........

Action item from the minutes of the last meeting: Paragraph 3.5.5.1., replace approved rim by measuring rim; tyre industry to clarify what to do in case the measuring rim is not available.

Proposal from Tyre Industry: After recheck, TI considers preferable to keep approved rims because the formula in § 3.5.3 takes the rim width into account.

3.5.5.1 Mount the tyre on one of the  approved rims mentioned in the appropriate Standards Manual.

3.5.6.1	Overall width

Action item from the minutes of the last meeting: “Overall width” tolerance in § 3.5.6.1.1. - the text needs to be simplified.

Proposal from Tyre Industry:

The tyre overall width may exceed the theoretical section width defined in paragraph 3.5.3 above by 4%.

Action item from the minutes of the last meeting: exclusion of rim protectors from overall width definition in UNECE but included in FMVSS139; need to be checked carefully why not a tolerance of 8 mm, as in Reg 30 is required.

Proposal from Tyre Industry: to add the following paragraph.

        - In addition, if the tyre has rim protectors (see definition in paragraph 2), the figure as increased by the above tolerance may be exceeded by 8 mm.

## Strength Test for Passenger Car Tyres

Action item from the minutes of the last meeting: Paragraph 3.6.2.5: confirmation for the changes in the text is needed from NHTSA.

## Tubeless Tyre Bead Unseating Resistance Test for Passenger Car Tyres

New proposal from Tyre Industry: This whole section has been replaced by the old version of the test. Indeed, the new test is not suitable for large tyres. A certain number of parameters needs to be redefined before this new test would be suitable for all dimensions.

    - Requirements

The following requirements apply to all radial ply tyres using the blocks referred to in the test procedure described in this section.

      - Each tubeless tyre shall meet the requirements for minimum force, in Newtons, for bead unseating resistance, specified in one of the tables below.

      - For tubeless radial ply tyres the applied force required to unseat the tyre bead at the point of contact, in relation to the nominal section width of the tyre, shall not be less than:

| Nominal Section Width S (mm) | Minimum Force (N) |
| --- | --- |
| S < 160 | 6 670 |
| 160 < S < 205 | 8 890 |
| S > 205 | 11 120 |

| Nominal Section Width S (code) | Minimum Force (N) |
| --- | --- |
| S < 6.00 | 6 670 |
| 6.00 < S < 8.00 | 8 890 |
| S > 8.00 | 11 120 |

    - Preparation of tyre

      - Wash the tyre and dry it at the beads.  Mount it without lubricant or adhesive on a clean, painted test rim.  The rim contour shall be one of those specified for the fitment of the test tyre.

      - Inflate the tyre to the pressure specified in the table shown below:

| Test Type | Passenger Tyres |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | kPa |  |  |  |  |
|  | 240 | 280 | 300 | 340 | 350 |
| Bead Unseating Test Pressure | 180 | 220 | 180 | 220 | 180 |

    - Test Procedure

      - Mount the assembly on a fixture as shown in Figure 2, below, and force the bead unseating block shown in Figure 3 or Figure 4 against the tyre sidewall as required by the geometry of the fixture.

      - Position the bead unseating block against the tyre sidewall at a horizontal distance “A” as shown in Figure 2 and Table 1, below.

      - Apply a force through the block to the tyre outer sidewall at a rate of 50 mm/min + 2.5 mm/min.

      - Increase the force until the bead unseats or until the prescribed value shown in paragraph 3.7.1.2. is reached.

      - Repeat the test at least four times at places approximately equally spaced around the tyre circumference.

- Figure 2 :

- Bead unseating fixture

| Table of A dimension for different rim codes |  |  |
| --- | --- | --- |
| Rim code | mm | Inches |
| 20 | 345 | 13.50 |
| 19 | 330 | 13.00 |
| 18 | 318 | 12.50 |
| 17 | 305 | 12.00 |
| 16 | 292 | 11.50 |
| 15 | 279 | 11.00 |
| 14 | 267 | 10.50 |
| 13 | 254 | 10.00 |
| 12 | 241 | 9.50 |
| 11 | 229 | 9.00 |
| 10 | 216 | 8.50 |
| 320 | 216 | 8.50 |
| 340 | 229 | 9.00 |
| 345 | 235 | 9.25 |
| 365 | 248 | 9.75 |
| 370 | 254 | 10.00 |
| 390 | 279 | 11.00 |
| 415 | 292 | 11.50 |

Table 1:  List of “A” Dimensions

Figure 3: Bead Unseating Block

Figure 4:  Bead Unseating Block

*	*		*

## Strength Test for LT/C Tyres (from FMVSS 119)

Editorial correction: Tables in § 3.14.1 and 3.14.2 were incorrect and have to be replaced by the tables below. Editorial correction have been also made in § 3.14.3.1 below.

    - Requirements

When tested according to the procedure described in this section, LT/C tyres must have an average strength of not less than the values shown in the table below:

| Load Range | Minimum breaking energy |  |
| --- | --- | --- |
|  | Joules (J) | Inch-pounds (in-lbs) |
| B | 293 | 2600 |
| C | 361 | 3200 |
| D | 514 | 5440 |
| E | 576 | 5100 |

    - Preparation of tyre

Mount the tire on a model rim assembly and inflate it to the pressure corresponding to the maximum load, or maximum dual load where there is both a single and dual load marked on the tire. If the tire is tubeless, a tube may be inserted to prevent loss of air during the test in the event of puncture.

Condition it at room temperature for at least 3 hours and readjust the inflation pressure if necessary.

    - Test Procedure

      - Force a 19.05 mm (0.75 inch) diameter cylindrical steel plunger with a hemispherical end perpendicularly into the tread rib as near to the centerline as possible, avoiding penetration into the tread groove, at the rate of 50 mm (2 inches) per minute.

3.15	Tubeless Tyre Bead Unseating Resistance for LT/C Tyres (from FMVSS 109)

Editorial correction: Editorial corrections have to be made in § 3.15.2.3 below to read:

      - Mount the wheel and tyre in a fixture shown in Figure 6, and force the bead unseating block shown in Figure 7 or Figure 8 against the tyre sidewall as required by the geometry of the fixture.

| Table of A dimension for different rim codes |  |  |
| --- | --- | --- |
| Rim code | mm | Inches |
| 20 | 345 | 13.50 |
| 19 | 330 | 13.00 |
| 18 | 318 | 12.50 |
| 17 | 305 | 12.00 |
| 16 | 292 | 11.50 |
| 15 | 279 | 11.00 |
| 14 | 267 | 10.50 |
| 13 | 254 | 10.00 |
| 12 | 241 | 9.50 |
| 11 | 229 | 9.00 |
| 10 | 216 | 8.50 |
| 320 | 216 | 8.50 |
| 340 | 229 | 9.00 |
| 345 | 235 | 9.25 |
| 365 | 248 | 9.75 |
| 370 | 254 | 10.00 |
| 390 | 279 | 11.00 |
| 415 | 292 | 11.50 |

Figure 6:  Bead Unseating Fixture (all dimension in mm) and table of “A” dimensions

Figure 7:  Diagram of Bead Unseating Block (all dimensions in mm)

Figure 8: Diagram of Bead Unseating Block (all dimensions in mm)

    - Test Procedure

      - Apply a load through the block to the tyre's outer sidewall at the distance specified in Figure 6 for the applicable wheel size at a rate of 50 mm (2 inches) per minute, with the load arm substantially parallel to the tyre and rim assembly at the time of engagement.

      - Increase the load until the bead unseats or the applicable value specified in 3.15.1 is reached.

      - Repeat the test at least four places equally spaced around the tyre circumference.

## Load/Speed Endurance Test for LT/C Tyres (from UN ECE Regulation 54)

Action item from the minutes of the last meeting: "Load capacity variation with speed": add missing paragraphs 6.2.4 and 6.2.5 from Reg.54 and renumber following paragraphs.

Proposal from Tyre Industry:

    - Requirements

      - Each type of LT/C tyre shall undergo at least one load/speed endurance test carried out by the procedure described below.

      - A LT/C tyre which, after undergoing the endurance test, does not exhibit any tread separation, ply separation, cord separation, chunking or broken cords shall be deemed to have passed the test.

      - The outer diameter of the tyre, measured six hours after the load/speed endurance test, must not differ by more than  3.5 per cent from the outer diameter as measured before the test.

      - If the load/speed combinations for the tyre is given in the table in Appendix 5, the endurance test prescribed in 3.16. above need not be carried out for load and speed values other than the nominal values.

      - Where application is made for the approval of a type of pneumatic tyre which has a load/speed combination in addition to the one that is subject to the variation of load with speed given in the table in Appendix 5, the endurance test prescribed in paragraph 3.16. above shall also be carried out on a second tyre of the same type at the additional load/speed combination.

Renumber former paragraphs 3.16.1to 3.16.3. to read 3.16.2 to 3.16.4