| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/1014 21 April 2004 ENGLISH Original: ENGLISH and FRENCH |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

CORRIGENDUM 1 TO SUPPLEMENT 2 TO REGULATION No. 113

(Headlamps emitting a symmetrical passing beam)

Note:	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-sixth session, following the recommendation by WP.29 at its one-hundred-and-thirty-second session.  It is based on document TRANS/WP.29/2004/10, not amended (TRANS/WP.29/992, para. 79).

__________

# Paragraph 6.3.2.2., correct to read:

"6.3.2.2.	Starting from point HV, horizontally to the right and left, the illumination shall be not less than 12 lux for Class B and C headlamp, and 24 lux for Class D headlamp to a distance of 1125 mm and not less than 3 lux for Class B and C headlamp, and 6 lux for Class D headlamp to a distance of 2250 mm.

In the case of a Class C and D headlamp, the intensities shall conform to the tables A or B in annex 3.  Table A applies in the case where a primary driving beam is being produced with a single light source.  Table B applies in the case where the driving beam is being produced by a secondary driving beam headlamp operated with a harmonized passing beam headlamp or a primary driving beam headlamp."

____________