| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/1020 21 September 2004 ENGLISH Original: ENGLISH AND FRENCH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

#### CORRIGENDUM 1 TO SUPPLEMENT 16 TO THE 04 SERIES OF AMENDMENTS TO REGULATION No. 16

Note :  The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-seventh session, following the recommendation by WP.29 at its one-hundred-and-thirty-third session.  It is based on document TRANS/WP.29/2004/30, as amended (TRANS/WP.29/1016, paras. 54 and 83.).

__________

Insert a paragraph 8.1.2.1. to read:

"8.1.2.1.	However, for outboard seating positions, other than front, of vehicles of the category N1 shown in annex 16 and marked with the symbol Ø, the installation of a lap belt of type Br4m or Br4Nm is allowed, where there exists a passage between a seat and the nearest side wall of the vehicle intended to permit access of passengers to other parts of the vehicle.  A space between a seat and the side wall is considered as a passage, if the distance between that side wall, with all doors closed, and a vertical longitudinal plane passing through the centre line of the seat concerned - measured at the R-point position and perpendicularly to the median longitudinal plane of the vehicle - is more than 500 mm."

Annex 16, amend to read:

| Annex 16 MINIMUM REQUIREMENTS FOR SAFETY-BELTS AND RETRACTORS Vehicle Category | Forward facing seating positions |  |  |  | Rear facing Seating positions |
| --- | --- | --- | --- | --- | --- |
|  | Outboard seating positions |  | Centre seating position |  |  |
|  | Front | Other than front | Front | Other than front |  |
| M1 | Ar4m | Ar4m | Ar4m | Ar4m | B, Br3, Br4m |
| M2 < 3.5 t | Ar4m, Ar4Nm | Ar4m, Ar4Nm | Ar4m, Ar4Nm | Ar4m, Ar4Nm | Br3, Br4m, Br4Nm |
| M2 > 3.5 t M3 | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm |
|  | See para. 8.1.7. for conditions when a lap belt is permitted | See para. 8.1.7. for conditions when a lap belt is permitted | See para. 8.1.7. for conditions when a lap belt is permitted | See para. 8.1.7. for conditions when a lap belt is permitted |  |
| N1 | ##### Ar4m, Ar4Nm | Ar4m, Ar4Nm, Br4m, Br4Nm Ø | B, Br3, Br4m, Br4Nm or A, Ar4m, Ar4Nm * 1/ | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm |
|  |  | Para. 8.1.2.1. lap belt permitted if seat is inboard of a passageway | Para. 8.1.6. lap belt permitted if the windscreen is not in the reference zone. |  |  |
| N2 N3 | B, Br3, Br4m, Br4Nm or A, Ar4m, Ar4Nm * | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm, or A, Ar4m, Ar4Nm * | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm |
|  | Para. 8.1.6. lap belt permitted if the windscreen is outside the reference zone and for the driver's seat. |  | Para. 8.1.6. lap belt permitted if the windscreen is not in the reference zone. |  |  |

| 1/ Erratum to Supplement 12 to the 04 series of amendments, applicable "ab initio." A: three-point (lap and diagonal) belt | B: 2-point (lap) belt | r: retractor | m: emergency locking retractor with multiple sensitivity |
| --- | --- | --- | --- |
| 3: automatically locking retractor | 4: emergency locking retractor | N: higher response threshold | (see Regulation No. 16, paras. 2.14.3. and 2.14.5.) |
| *: Refers to para. 8.1.6. of this Regulation 2/ | Ø: Refers to para. 8.1.2.1.of this Regulation Regulation | ●: refers to para. 8.1.7. of this Regulation 2/ |  |

TRANS/WP.29/1020

page 3

2/     Erratum to Revision 4, applicable "ab initio."

Note: 	In all cases all S-type belts may be fitted in place of all possible A or B type belts, provided their anchorages comply with Regulation No. 14.

Where a harness belt has been approved as a S-type belt according to this Regulation, using the lap belt strap, the shoulder belt straps and possibly one or more retractors, one or two additional crotch straps including their attachments for their anchorages may be provided by the manufacturer/applicant. These additional anchorages need not meet the requirements of Regulation No. 14 (Erratum to Supplement 14 to the 04 series of amendments, applicable "ab initio.")"

___________