| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/1030 21 September 2004 ENGLISH Original: ENGLISH AND FRENCH |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

CORRIGENDUM 1 TO SUPPLEMENT 4 TO REGULATION No. 65

(Special warning lamps)

Note :  The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-seventh session, following the recommendation by WP.29 at its one-hundred-and-thirty-third session.  It is based on document TRANS/WP.29/2004/48, not amended (TRANS/WP.29/1016, para. 83.).

__________

Paragraph 1.7., amend to read:

"1.7.		"reference centre of the special warning lamp" means:

-	for a rotating or stationary flashing lamp (category T), the centre of the light source,

- for a directional flashing lamp (category X), the intersection of the axis of reference with the exterior light-emitting surface; it is specified by the manufacturer of the special warning lamp.  In the absence of such specification, it means the centre of the light source."

Paragraph 1.8., amend to read:

"1.8.		"reference axis of the special warning lamp" means:

- for a rotating or stationary flashing lamp (category T), a vertical axis passing through the reference centre of the lamp,

- for a directional flashing lamp (category X), a horizontal axis parallel to the median longitudinal plane of the vehicle.

The manufacturer of the special warning lamp shall indicate the position of the special warning lamp in relation to the reference axis."

___________