|  | United Nations | ECE/TRANS/WP.29/2012/68 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 27 July 2012 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

158th session

Geneva, 13-16 November 2012

Item 4.7.7 of the provisional agenda

1958 Agreement – Consideration of draft amendments

to existing Regulations submitted by GRE

Proposal for Supplement 19 to Regulation No. 23 (Reversing lamps)

Submitted by the Working Party on Lighting and Light-Signalling*

The text reproduced below was adopted by the Working Party on Lighting and Light-Signalling (GRE) at its sixty-seventh session to introduce into the Regulation provisions for the use of light-emitting diode (LED) light sources according to the draft Regulation (Regulation No. XXX: ECE/TRANS/WP.29/2010/44 and Corr.1) and to correct and harmonize the provisions concerning approval markings. It is based on ECE/TRANS/WP.29/GRE/2011/9, as amended by para. 25 and Annex III to the report and ECE/TRANS/WP.29/GRE/2012/9, as amended by para. 29 of the report (see ECE/TRANS/WP.29/GRE/67, paras. 25 and 29). It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration.

Paragraph 1.4., amend to read:

"1.4.	Reversing / manoeuvring lamps of different types" means …

(b)	The characteristics … category of light source, light source module, etc.);

A change of the colour of the light source or the colour of any filter does not constitute a change of type."

Paragraph 1.5., amend to read:

"1.5.	References made… for type approval.

References made in this Regulation to standard (étalon) LED light source(s) and to Regulation No. XXX shall refer to Regulation No. XXX and its series of amendments in force at the time of application for type approval."

Paragraph 2.2.2., amend to read:

"2.2.2.	a brief … light sources:

(a)	The category …; and/or

(b)	The category or categories of LED light source(s) prescribed; this LED light source category shall be one of those contained in Regulation No. XXX and its series of amendments in force at the time of application for type approval; and/or

(c)	the light source module specific identification code."

Paragraph 3.2., amend to read:

"3.2.	with the … indicating:

(a)	The category or categories of light source(s) prescribed; and/or

(b)	The light source module specific identification code."

Paragraph 4.7., amend to read:

"4.7. 	Annex 2 gives examples of approval marks for a single lamp (Figure 1) and for grouped, combined or reciprocally incorporated lamps (Figure 2) with all the additional symbols referred to above. The letters A and R may be mingled."

Paragraphs 5.4. to 5.4.3., amend to read:

"5.4. 	In the case of replaceable light source(s):

5.4.1. 	Any category or categories of light source(s) approved according to Regulation No. 37 and/or Regulation No. XXX may be used, provided that no restriction on the use is made in Regulation No. 37 and its series of amendments in force at the time of application for type approval or in Regulation No. XXX and its series of amendments in force at the time of application for type approval.

5.4.2. 	The design of the device shall be such that the light source can be fixed in no other position but the correct one.

5.4.3. 	The light source holder shall conform to the characteristics given in IEC Publication 60061. The holder data sheet relevant to the category of light source used, applies."

Paragraph 7.1.1., amend to read:

"7.1.1.	In the case of a …standard light source of the category prescribed for the device, supplied with the voltage:

(a)	In the case of filament lamp(s), that is necessary to produce the reference luminous flux required for that category of filament lamp,

(b)	In the case of LED light sources of 6.75 V, 13.5 V or 28.0 V; the luminous flux value produced shall be corrected. The correction factor is the ratio between the objective luminous flux and the mean value of the luminous flux found at the voltage applied."

Annex 2, Figure 1, amend to read:

"Annex 2

Examples of Arrangements of Approval Marks

# Figure 1

# Marking for single lamps

Model A	optional:

|  | AR |
| --- | --- |

Annex 3, paragraph 3.2., amend to read:

"3.2.		For replaceable light source(s):

When equipped with light source(s) at 6.75 V, 13.5 V or 28.0 V, the luminous intensity values produced shall be corrected. For filament lamps the correction factor is the ratio between the reference luminous flux and the mean value of the luminous flux found at the voltage applied (6.75 V, 13.5 V or 28.0 V).

For LED light sources the correction factor is the ratio between the objective luminous flux and the mean value of the luminous flux found at the voltage applied (6.75 V, 13.5 V or 28.0 V).

The actual luminous fluxes of each filament lamp and/or LED light source used shall not deviate more than 5 per cent from the mean value.

Alternatively and in case of filament lamps only, a standard filament..."

Annex 4, paragraph 1.2.2., amend to read:

"1.2.2.		If, in the case of a … standard light source."

Annex 5, paragraphs 1.2.2., amend to read:

"1.2.2.		If, in the case of a … standard light source."

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106, ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.