Addendum to ECE/TRANS/WP.29/2012/4

Paragraph 4.2.2.2.2, amend to read:

“4.2.2.2.2.	Test program (brake drum high load test).

Table A11/4.2.2.2.2.

| Test provision | High load test |
| --- | --- |
| "Bedding in" procedure | 200 brake applications Initial speed: 60 km/h Final speed: 5 km/h dm alternating between 1 m/s2 and 2 m/s2 Initial temperature :  200°C (beginning at room temperature) Alternatively bedding may be omitted if the applicant for approval does not consider it to be necessary |
| Type of braking | Braking to less than 5 km/h |
| Total number of brake applications | 150 |
| Initial brake drum temperature at each brake application |  100 °C |
| Brake applications from to | 60 km/h  5 km/h |
| Brake torque set to produce a deceleration of | 6 m/s² |
| Cooling (also deviating from paragraph 3.2.3. of this annex) | Permitted |