Proposal of new sound limit values to the draft 03 series of amendments 
to UN regulation No. 51

| Vehicle Category | Sub-category | Limit value (Phase 1st) | Limit value (Phase 2nd) |
| --- | --- | --- | --- |
| M1* | GVW ＜ 2 500kg or seats number ＜ 6 | 72 | Reserved |
|  | Others | 73 |  |
| M2 | GVW＜3 500 kg | 74 |  |
|  | Others | 76 |  |
| M3 | Vehicle length ＜ 7 500mm | 78 |  |
|  | 7 500mm ≤Vehicle length ＜ 10 000mm | 80 |  |
|  | 10 000mm ≤Vehicle length | 81 |  |
| N1 | ---- | 74 |  |
| N2 | GVW＜5 000 kg | 78 |  |
|  | Others | 79 |  |
| N3 | GVW＜ 17 000kg | 81 |  |
|  | Others | 82 |  |

Additional items (All the three additional items can be accumulated.):

1. All vehicle categories use more than one drive axle during the test, the limit values +1 dB;

2. All vehicle categories with more than two axles, the limit values +1 dB;

3. * M1 categories vehicles with the PMR values higher than 120 kW/t, the limit values +1 dB.

The analysis of limit value submitted by China, Japan and limit values of ECE R51-02 and GB 1495-2002

| Vehicle Category | Sub-category | Limit value dB(A) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Suggestion of China (Phase 1st) | Suggestion of Japan (Phase 1st) | ECE R51-02 series | GB 1495-2002 (2005 edition) |
| M1*[1] | GVW ＜ 2 500kg or seats number ＜ 6 | 72+1+1 | 72+1 / 73+1 / 75+1 | 74+1+1+1(+2) | 74+1+1+1(+2) |
|  | Others | 73+1+1 |  |  |  |
| M2[2] | GVW＜3 500 kg | 74+1 | 72+1 / 74+1 | 76+1 / 77+1 | 76+1 / 77+1 |
|  | Others | 76+1 | 75+1 | 78+1 / 80+2 | 80+1 / 83+2 |
| M3[3] | Vehicle length ＜ 7 500mm | 78+1 | 76+2+2 / 79+2 / 80+2 |  |  |
|  | 7 500mm ≤Vehicle length ＜ 10 000mm | 80+1 |  |  |  |
|  | 10 000mm ≤Vehicle length | 81+1+1 |  |  |  |
| N1 | ---- | 74+1 | 74+1 / 72+1 / 74+1 | 76+1 / 77+1 | 76+1 / 77+1 |
| N2 | GVW＜5 000 kg | 78+1 | 77+1 / 78+1 | 77+1 / 78+1 / 80+2 | 81+1 / 83+1 / 84+2 |
|  | Others | 79+1 |  |  |  |
| N3 | GVW＜ 17 000kg | 81+1 | 80+2 / 82+2 |  |  |
|  | Others | 82+1+1 |  |  |  |

Additional items (All the three additional items can be accumulated):

1. All vehicle categories use more than one drive axle during the test, the limit values +1 dB;

2. All vehicle categories with more than two axles, the limit values +1 dB;

3. * M1 categories vehicles with the PMR＞120 kW/t, the limit values +1 dB.

Appendix

Explanations for the sub-categories suggestion from China

In the view point of human being and customers, the vehicle means what ?

Used for carring goods or persons？How many goods and persons it can take for me？

Purpose

And Utility

Characteristic of complete vehicles

How many seats the vehicle has ? and what’s the mass and dimension parameters? (other parameters  of complete vehicles )

Engine displacement, fuel consumption, engine manufacturer,  engine power, engine torque……

Which kind of  transmission ? Manual or automotive ? How many Gears?

tyre type ? tyre dimension ?

The main assembly

The safty performance,  the economic performance, the environment performance, the dynamic performance.

Performance of vehicles

The vehicle Category M: M1: M2: M3 ; are all divided by seats number and maximum  weight of vehicles.

Category N: N1: N2: N3 are all divided by maximum weight of vehicles.

So the sub-categories should not go against the overall plan and total vehicle categories.

Figure 1 Thinking models - HUMANBEING CENTER

Conclusion:

- I have ever browsed the Germany Constitution (Chinese edition), the first part is all about the human rights and the equality between different persons, when we decide the sub-categories we should better choose the view point of human being. Use the utility, purpose, maximum weight, seat number or the cargo capacity. This should be considered as the mainstream thinking models.

- We can not use the mechanical thinking models instead of the human thinking models. There is a famous America movie called < Spider Man>, and there is a famous dialogue “with great power comes great responsibility”, that does not mean “with great money comes great engine, and with great engine power comes great pass-by noise”, the equality of the regulations must been ensured firstly.

- The government of China does not encourage the high engine displacement and high energy consumption vehicles.

- The noise is a kind of pollution, and is now a more and more serious question in China. Some noise can not been avoid for the normal running of urban and some noise is just been made for fun or just shows the owners of vehicles are special, we should treat them separately, avoid and forbidden the non-necessary noise first, manage and improve other traffic noise like buses and coaches, and finally have a better acoustic environment.

- The vehicles which frequently appear in the urban areas should been managed more strictly in the noise field, so the limit values for all M categories should been more strict.

Engine arrangement,

engine displacement,

engine power or engine torque;

manual or automatic transmission ;

one axle dirve, two wheel drive or all wheel drive  (contains power divider)

In the view point of noise source, the vehicle means what ?

Contains engine (exhausted pipe \ air filter) \ transmission \ transmission shaft \ power divider \ final drive

Powertrain system

Drive system

Contains vehicle frame \ suspensions \ axles \ tyres

Vehicle body

Wind noise (not obvious in pass-by noise test)

Additional consideration

Additional consideration

axles number, tyres type, tyres number

Figure 2 Thinking models –NOISE SOURCE

Conclusion:

- Only the mainstream is not enough, so maybe we need some additional considerations, in the view point of noise source. We need some more consideration for the more than one axle drive vehicles, high engine performance vehicles, the axle number and tyres number.

- The engine power is just one parameter of engine, and we should not only consider the sub-categories of complete vehicles with a single parameter of engine. That means we should not use the part instead of the whole.

- The theoretical basis of ECE R51 – 03 Series and ISO 362-1:2007 is that the engine noise is not the only main noise source of vehicles, so the new test method considers the noise of the powertrain and the drive systems. So, when we set the limit values we should also consider the structure, arrangement and main noise source of complete vehicles.

- I have ever browsed the Constitution of Japan (English edition), it says “We believe that no nation is responsible to itself alone, but that laws of political morality are universal”, I agree with it, so we should consider the sub-categories and limit values in the view point of all the human being but not only the benefit of any single country or any single region.

- Although there are nearly no passenger cars with PMR values higher than 120kW/t manufactured in China, but there are also some realistic problems, the noise of sports and luxury cars have high engine power and high noise. And there was a German philosopher called Georg Wilhelm Friedrich Hegel ever said “All that is real is rational; and all that is rational is real”, so maybe we can control the noise of this kind of vehicle with some additional items, but not set all the vehicles sub-categories using the engine power method. We can have some other better and more neutral methods for the sub-categories.

[1] There are nearly no passenger cars with PMR values higher than 120 kW/t manufactured in China.

[2] There are nearly no M2 / M3 / N2 vehicles with more than one drive axle manufactured in China.

[3] The M3 categories with the length ＞13 meters alsways have more than two axles manufactured in China