Proposal for Supplement 41 to the 03 series of amendments to Regulation No. 37 (Filament lamps)

Note:	The text reproduced below was prepared by the expert from the International Electrotechnical Commission (IEC) in order to clarify a provision concerning category PR27/7W in Regulation No. 37.  The modifications to the current text of Regulation No. 37 are marked in bold characters.

I.	Proposal

Sheet PR27/7W/1, the table, the cap designation, amend to read:

“...

Cap WU2.5x16q in accordance with IEC Publication 60061 (sheet 7004-104D-1)

...”

II.	Justification

A mistake was discovered only recently in the IEC cap designation of category PR27/7W.  Cap designations are well defined in publication IEC 60061-4, sheet 7007-1-6.  In this case of a dual-filament light source, 4 contacts are needed and indicated in the drawing in sheet PR27/7W/1.  The designation rules prescribe for 4 contacts the use of the character “q” in the designation (part “c”).