| Transmitted by the expert from OICA | Informal document GRE-68-16-rev 1 (68th GRE, 16-18 October 2012, agenda item 4(a)) |
| --- | --- |

Proposal for amendments to ECE/TRANS/WP.29/GRE/2012/29

(Supplement 11 to the 04 series of amendments to Regulation No. 48)

(Supplement XX to the 05 series of amendments to Regulation No. 48)

(Supplement YY to the 06 series of amendments to Regulation No. 48)

I.	Proposal

A. Amendment to the 04 series of amendment to Regulation No. 48

Paragraph 6.1.7.2., amend to read:

"6.1.7.2.	It shall always be possible to switch the main-beam headlamps ON and OFF manually and to manually switch OFF the automatic control of the main-beam headlamps.

Moreover, the switching OFF, of the main-beam headlamps and of their automatic control, shall be by means of a simple and immediate manual operation; (multi-step operations, such as the use of sub-menus is not allowed)."

Paragraph 6.22.7.1.3., amend to read:

"6.22.7.1.3.	It shall always be possible to switch the main-beam headlamps, adaptive or non-adaptive, ON and OFF manually and to manually switch OFF the automatic control.

Moreover, the switching OFF, of the main-beam headlamps and of their automatic control, shall be by means of a simple and immediate manual operation; (multi-step operations, such as the use of sub-menus is not allowed)."

B. Amendment to the 05 and 06 series of amendment to Regulation No. 48

Paragraph 6.1.7.23., amend to read:

"6.1.7. 23 	It shall always be possible to switch the main-beam headlamps 		ON and OFF manually and to manually switch OFF the 			automatic control of the main-beam headlamps.

Moreover, the switching OFF, of the main-beam headlamps 		and of their automatic control, shall be by means of a simple 		and immediate manual operation; (multi-step operations, such 		as the use of sub-menus is not allowed)."

Paragraph 6.22.7.1.3., amend to read:

"6.22.7.1.3.	It shall always be possible to switch the main-beam headlamps, 		adaptive or non-adaptive, ON and OFF manually and to 			manually switch OFF the automatic control.

Moreover, the switching OFF, of the main-beam headlamps 		and of their automatic control, shall be by means of a simple 		and immediate manual operation; (multi-step operations, 		such as the use of sub-menus is not allowed)."

II.	Justification

The prohibition of sub-menus is considered fully in line with the objective of the proposal ECE/TRANS/WP.29/GRE/2012/29.

At the same time the proposal should not restrict the current and future design of control and display systems. Therefore we propose to delete the “multi-step operations”.

_________________