GFV-20-05

INFORMAL GROUP ON GASEOUS FUEL VEHICLES

Within the UN GRPE (WP29)

Correction Factor for G-25 Weighting Process

Please submit new work items to:

Andre Rijnders, Chairman (RDW, Netherlands) arijnders@rdw.nl

Acting secretariat(s)

Jeffrey Seisler (NGV Global/Clean Fuels Consulting) jseisler@cleanfuelsconsulting.org

Salvatore Piccolo (AEGPL/Federchimica) s.piccolo@federchimica.it

Name of Organisation submitting Amendment/Work Item

AEB/LANDI RENZO

Person submitting Item

Alberto Castagnini

Francesco Cagnolati

Address/phone/email coordinates

via dell'Industria 20

42025 Cavriago (RE) Italy

Ph: +39 0522 494401

mail: acastagnini@aeb-tech.com

Regulation name and reference number

Regulations 115 & 83.

Name of Amendment/Work Item

Correction factor for G25 weighing process

Specific language for Amendment/Work Item

English

Rationale: (Why is it important/required?)

Documents ECE-TRANS-WP29-GRPE-2012-07 (referring to R115) and ECE-TRANS-WP29-GRPE-2012-06 (referring to R83) introduce the calculation of NG/biomethane energy ratio during a type I cycle by weighing the NG/biomethane mass consumed. The formulas introduced respectively in Annex 6B (ECE-TRANS-WP29-GRPE-2012-07) and Annex 12 - Appendix 2 (ECE-TRANS-WP29-GRPE-2012-06) are affected by an error when G25 reference fuel is used. The error is due to the presence of inert gas (N2) in the G25 reference fuel, varying from 12 to 16% in molar fraction units.

This proposal describes a modification to previous formulas in order to take into account the presence of inert gas and providing the correct energy ratio.

Analysis/testing or data requirements to support the Amendment/Work item

When G25 reference fuel is used and its composition is known, the mass of inert gas in it can be calculated.

In general, R83 fixes the allowed ranges for both CH4 molar fraction and N2 molar fraction in reference fuels. If we assume xCH4 the molar fraction of CH4 and xN2 the molar fraction of N2, we have:

xCH4 = nCH4 / nTOT 		(for G25: 0.84 ≤ xCH4 ≤ 0.88)

xN2 = nN2 / nTOT			(for G25: 0.12 ≤ xN2 ≤ 0.16)

where:

nCH4 is the number of moles of CH4

nN2 is the number of moles of N2

nTOT is the total number of moles

Since the mass of CH4 and N2 expressed in grams can be calculated as follows:

mCH4 = nCH4 * PMCH4

mN2 = nN2 * PMN2

where:

PMCH4 is the molar mass of CH4  = 16.042 g/mol

PMN2 is the molar mass of N2 = 28.02 g/mol

Combining previous equations:

mCH4 = xCH4 * nTOT* PMCH4

mN2 = xN2 * nTOT * PMN2

The weighing process will measure the total mass of G25 mTOT:

mTOT = mCH4 + mN2

mTOT = nTOT * (xCH4 * PMCH4 + xN2 * PMN2)

but for the calculation of the energy ratio only the mass of CH4 should be used. Thus the weighting process will overestimate the mass by a factor:

mTOT / mCH4 = (xCH4 * PMCH4 + xN2 * PMN2) / (xCH4 * PMCH4)

or vice versa the measured mass must be reduced by a factor:

cf = mCH4 / mTOT = (xCH4 * PMCH4) / (xCH4 * PMCH4 + xN2 * PMN2).

For G25, in the worst case of minimum quantity of CH4 and maximum quantity of N2, the factor is

cfG25 = 0.75.

(continue)

Proposal of modification of doc. ECE-TRANS-WP29-GRPE-2012-07

In annex 6B, replace par 2 with the following:

2.	Calculation of the CNG energy ratio

The fuel consumption value shall be calculated from the emissions of hydrocarbons, carbon monoxide, and carbon dioxide determined from the measurement results assuming that only CNG is burned during the test.

The CNG ratio of the energy consumed in the cycle is then determined as follows:

GCNG = MCNG*cf *100/(FCmean*dist*d)*100%

Where:

GCNG: the CNG energy ratio;

MCNG: the CNG mass consumed during the cycle (kg);

FCmean: the mean fuel consumption calculated in accordance with paragraph. 6.2.2.4.3.2.;

dist: distance travelled during the cycle (km);

d: density d=0.654kg/m3.

cf: correction factor, assuming the following values:

cf = 1 							in case of G20 reference fuel

cf = (xCH4 * PMCH4) / (xCH4 * PMCH4 + xN2 * PMN2)	in case of G25 reference fuel

where:

xCH4 is the molar fraction of CH4

xN2 is the molar fraction of N2

PMCH4 is the molar mass of CH4  = 16.042 g/mol

PMN2 is the molar mas of N2 = 28.02 g/mol

If the exact composition of G25 is not known, a value of cf = 0.75 can be assumed.

Proposal of modification of doc. ECE-TRANS-WP29-GRPE-2012-06

In Annex 12 - Appendix 2, replace par. 2 with the following:

2. 	Calculation of the CNG energy ratio

The fuel consumption value shall be calculated from the emissions of hydrocarbons, carbon monoxide, and carbon dioxide determined from the measurement results assuming that only CNG is burned during the test.

The CNG ratio of the energy consumed in the cycle is then determined as follows:

GCNG = MCNG* cf *100/(FCnorm*dist*d)

Where:

GCNG: the CNG energy ratio;

ECE/TRANS/WP.29/GRPE/2012/6

MCNG: the CNG mass consumed during the cycle (kg);

FCnorm: the fuel consumption calculated in accordance with Paragraph 1.4.3., letter (c), of Annex 6 to Regulation No. 101;

dist: distance travelled during the cycle (km);

d: density d = 0.654kg/m3.

cf: correction factor, assuming the following values:

cf = 1 							in case of G20 reference fuel

cf = (xCH4 * PMCH4) / (xCH4 * PMCH4 + xN2 * PMN2)	in case of G25 reference fuel

where:

xCH4 is the molar fraction of CH4

xN2 is the molar fraction of N2

PMCH4 is the molar mass of CH4  = 16.042 g/mol

PMN2 is the molar mas of N2 = 28.02 g/mol

If the exact composition of G25 is not known, a value of cf = 0.75 can be assumed.