| Submitted by the expert from OICA | Informal document GRSG-102-18-Rev.1 (102nd GRSG, 16-20 April 2012 agenda item 8) |
| --- | --- |

Proposal for amendments to Regulation No. 67
(Equipment for liquefied petroleum gas (LPG))

The modifications to the proposal ECE/TRANS/WP.29/GRSG/2012/7 are marked in blue.

Paragraphs 6.15.2.1. and 6.15.2.2., amend to read:

"6.15.2.1.	The electrical connections inside the boot and passengers compartment shall comply with protection degree class IP 40 according to IEC Standard 60529-1989+A1:1999.

6.15.2.2.	All other electrical connections shall comply with protection degree class IP 54 according to IEC Standard 60529-1989+A1:1999.[ISO 20653-2006 (currently being revised)]  "

Paragraph 6.15.13.1.1., amend to read:

"6.15.13.1.1.	In case the …… protection degree class IP 40 according to IEC Standard 60529-1989+A1:1999."

Paragraph 17.11.4., amend to read:

"17.11.4.	Electrical cables shall …… with protection degree class IP 40 according to IEC Standard 60529-1989+A1:1999. All other electrical connections shall comply with protection degree class IP 54 according to IEC Standard 60529-1989+A1:1999."

Annex 10,

Paragraph 2.1.2.3.1., amend to read (including the insertion of a new footnote 1/):

"2.1.2.3.1.	The bend test shall be carried out in accordance with standards ISO 7438:2003 2005 1/ and ISO 7799:2000 and ISO 7799:1985 and Euronorm EN 910 ISO 5173:2009+Amd 1:2011 for welded parts.  The bend tests shall be carried out on the inner surface in tension and the outer surface in tension.

____________________

1/	ISO 4009, referred to in this standard, need not be applied.

Paragraph 2.6.1., renumber the reference to and footnote 1/ to read footnote 2/.

__________