|  | United Nations | ECE/TRANS/WP.29/GRSP/2012/21 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 25 September 2012 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Fifty-second session

Geneva, 11–14 December 2012

Item 15 of the provisional agenda

Regulation No. 44 (Child restraint systems)

Proposal for Supplement 7 to the 04 series of amendments

Submitted by the expert from France*

The text reproduced below was prepared by the expert from the France on behalf of the Technical Services Group (TSG) on UN Regulation No. 44. It aims to clarify the lap strap interaction with the dummy and its assessment as well as improving the overturning test procedure in light of recent findings. The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Contents, list of Annexes,

Insert a new Annex 23, to read:

"Annex 23	-	Load application device"

Text of the Regulation,

Paragraph 2.8.1., amend to read:

2.8.1.	"lap strap" means a strap which, either in the form of a complete belt or in the form of a component of such a belt, passes across the front of, and restrains, directly or not, the child's pelvic region pelvis;

Paragraph 6.2.1.5., amend to read:

"6.2.1.5.	For all forward-facing group I restraints incorporating an integral harness belt system to prevent submarining, either by impact or through restlessness, a crotch strap shall be required. on all forward-facing group I restraints incorporating an integral harness belt system.  With the crotch strap attached, and in its longest position if adjustable, it shall not be possible to adjust the lap strap to lie above the pelvis in either the 9 kg or the 15 kg dummy."

Paragraph 6.2.2., amend to read:

"6.2.2.	For groups I, II and III, all restraint devices utilizing a "lap strap" must positively guide the "lap strap" to ensure that the loads transmitted by the "lap strap" are transmitted through the pelvis. The assembly shall not subject weak parts of the child's body (abdomen, crotch, etc.) to excessive stresses."

Insert new paragraphs 6.2.2.1. and 6.2.2.2., to read:

"6.2.2.1.	With the crotch strap attached, and in its longest position if adjustable, it shall not be possible to adjust the lap strap to lie above the pelvis of both the smallest and largest dummy within the mass groups covered by the approval. For all forward-facing restraints, it shall not be possible to adjust the lap strap to lie above the pelvis of both the smallest and largest dummy within the mass groups covered by the approval."

6.2.2.2.	During the dynamic test, as prescribed in paragraph 8.1.3., the lap belt shall not wholly pass beyond or move over the pelvic structure of the dummy, during the period prior to maximum horizontal head excursion. Assessment shall be carried out using high speed camera."

Paragraph 6.2.4., amend to read

"6.2.4.	The assembly shall not subject weak parts of the child's body (abdomen, crotch, etc.) to excessive stresses. The design shall be such that compression loads shall not be imposed on the crown of the child's head in the event of a collision."

Paragraph 7.1.3.1., amend to read

"7.1.3.1.	The child restraint shall be tested as prescribed in paragraph 8.1.2.; the manikin shall not fall out of the device during the whole test and, when the test seat is in the upside down position the manikin's head shall not move more than 300 mm from its original position in a vertical direction relative to the test seat."

Paragraph 8.1.2.2., amend to read:

"8.1.2.2.	The restraint shall be fastened to the test seat or vehicle seat. The whole seat shall be rotated around a horizontal axis, contained in the median longitudinal plane of the seat through an angle of 360 540° +/- 5° at a speed of 2-5 degrees/second and stopped in this position.  For the purposes of this test, devices intended for use in specific cars may be attached to the test seat described in Annex 6."

Insert new paragraphs 8.1.2.3. to 8.1.2.5., to read:

"8.1.2.3.	At this static inverted position a total mass equivalent to [5] times that of the dummy shall be applied vertically downwards in a plane perpendicular to the axis of rotation to the dummy utilizing the load application device described in Annex 23. The load shall be applied at a rate not exceeding [400 mm/min]. Maintain the prescribed maximum load  for a duration of [30 -0/+5] seconds.

8.1.2.4.	Remove the load at a rate not exceeding [400] mm/min and measure displacement.

8.1.2.5.	Rotate the whole seat for 180° to return to the starting position."

Paragraph 8.1.2.3.and 8.1.2.4.(former), renumber as paragraphs 8.1.2.6. and 8.1.2.7. and amend to read:

"8.1.2.36.	This test cycle shall be carried out again rotating in the reverse direction. after having replaced, if necessary, the manikin in its initial position. With the rotational axis in the horizontal plane and at 90° to that of the two earlier tests, the procedure shall be repeated in the two directions of rotation.

8.1.2.47.	These tests shall be carried out using both the smallest and the largest appropriate manikin of the group or groups for which the restraining device is intended.

Any adjustment of the dummy or child restraint systems during the complete test cycle is not allowed."

Insert a new Annex 23, to read:

"Annex 23

Load application device

"

II.	Justification

1.	Paragraph 2.8.1.: Consistency with paragraph 6.2.2 and clarification of the meaning of "lap strap" in relation to devices used to restrain the child and ensure loading through the pelvis.

2.	Paragraph 6.2.1.5.: Deals with submarining, while the second part of the paragraph covers requirement of lap strap position which is considered more relevant to paragraph 6.2.2. Assessment test method has been also clarified.

3.	Paragraph 6.2.2.: Deals with effective lap belt positioning. Requirements about this item have been removed from 6.2.1.5 and 6.2.4 and collected in one paragraph. Due to complexity of the verification assessment, two subparagraphs have been prepared: 6.2.2.1 provides guidance for the static assessment of lap strap positioning, while 6.2.2.2 deals with dynamic assessment. Consequently also 6.2.4 has been modified.

4.	Paragraphs 7.1.3. to 8.1.2.7. and Annex 23: The present test procedure does not adequately reflect the circumstances of real world rollover crashes. The procedure has been improved to reduce the existing differences between the test procedure and real accidents. A vertical quasi-static load is applied to simulate the dynamic behaviour of the child restraint systems and the dummy.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.