I.	Comments on the proposed amendments to GTR No 9 in working document ECE/TRANS/WP.29/AC.3/31

Concerning the proposal to insert new paragraphs 3.30. and 3.31.:

"3.30.	”Measuring point” for the headform test means a point on the vehicle’s outer surface selected to be tested. The central axis of the impactor and the measuring point are in a plane parallel to the vertical longitudinal median plane of the vehicle (see Figure 6A). The first contact point may differ from the measuring point as a result of the geometry of the bonnet top (see Figure 6B)."

[The measuring point is sometimes referred to as "test point" or "selected impact point" in respective regulatory texts for pedestrian protection.]

3.31.	”Measuring point” for the lower legform to bumper test and upper legform to bumper test means a point on the vehicle’s outer surface selected to be tested. The central axis of the impactor and the measuring point are in a plane parallel to the vertical longitudinal median plane of the vehicle. The first contact point may differ from the measuring point as a result of the geometry of the vehicle front (see Figure 6C)."

It is not clear if these additions are indeed meant to be definitions. Furthermore, the text is not clear as a point in the vertical longitudinal plane of the vehicle is described. As the headform is aimed towards the rear of the vehicle in a downward direction and the legform only in a rearward direction, the only relevant planes are either the horizontal and/or transverse vertical planes.

Concerning the proposal to amend Paragraph 5.2.4.3.:

"5.2.4.3. 	The areas of "HIC1000 zone" and "HIC1700 zone" may consist of several 			parts, with the number of these parts not being limited. The determination of 			the impacted zone is done by the measuring first contact point, irrespective 			of the position of the first contact point of the headform with the "bonnet 			top"."

This is quite a departure from the original formal text and we would like to ensure that all Contracting Parties can agree to this proposed change.

II.	Justification

1.	The European Commission has raised concerns about this document during the 50th session of GRSP.

2.	The current text of 5.2.4.3. leads to a very clear situation without the need for further interpretation, however, this is not in line with current practise of manufacturers and technical services. The aim to modify this text is to align it with existing type-approval testing practises, however the implication on self-certification testing is not entirely clear.

3.	Provided that the relevant Contracting Parties can indeed support the proposal in general, the items which surely need to be addressed are those of the unclear definitions.