| Transmitted by the chairman of the informal group GTR9-PH2 | Informal document GRSP-52-31 (52nd GRSP, 11 – 14 December 2012, agenda item 4(a)) |
| --- | --- |

Draft Third progress report of the informal group on Phase 2 of gtr No. 9

(IG GTR9 - PH2)

The text reproduced below was submitted by the chairman of the informal group on phase 2 of gtr no. 9 and proposes the 3rd progress report on the activities and work of this informal group (ECE/TRANS/WP.29/AC.3/24).

I.		Objective of this proposal

1.	The representatives of Japan and Germany proposed the development of Phase 2 of gtr No. 9. The main objective is the development of a draft proposal to amend the UN global technical regulation No. 9 - Phase 2 on pedestrian safety by introducing the flexible pedestrian legform impactor (FlexPLI) as a single harmonized test tool in order to enhance the level of protection for lower legs of pedestrians.

The work of the informal group shall not be limited to draft proposals to amend Gtr No. 9, but shall cover the development of a complementary draft proposal to amend the draft UN Regulation on pedestrian safety.

The group may also review proposals to improve and / or clarify aspects of the legform test procedure.

II.	Background

2.	At the 154th session of the World Forum for Harmonization of Vehicle Regulations and the 32nd session of the Executive Committee of the 1998 Agreement it was agreed to set up an informal group to solve the pending issues for the incorporation of the FlexPLI in Phase 2 of the UN Gtr No. 9 and in the draft UN Regulation on pedestrian safety, subject to the submission to WP.29 and to AC.3 of appropriate terms of references (document ECE/TRANS/WP.29/1091, paras. 36 and 100).

Japan and Germany have informed the World Forum for Harmonization of Vehicle Regulations and the Executive Committee of the 1998 Agreement that this informal group named IG GTR9 - Phase 2 will be co-sponsored by Japan and Germany.

3.	The informal group has started its work on 03 November 2011 with a Constitutional Meeting in Bonn, Germany, to draft a document on the terms of references, the operating principles, the schedule and the work plan. At this occasion the participants agreed to the proposal of Japan and Germany that the informal group will be managed by Germany (chair), Japan (vice-chair) and OICA (secretary).

4.	At the 155th session of the World Forum for Harmonization of Vehicle Regulations and the 33rd session of the Executive Committee of the 1998 Agreement Japan and Germany have informed delegates about the ongoing activities of the informal group on UN Gtr No. 9 Phase 2 (document WP.29-155-35). The delegates were informed about the constitutional meeting and the management of the group. The information was given that the first meeting was planned for 01 and 02 December 2011 to start the technical discussion and to finalize the draft terms of references as well as the work plan for submission to GRSP in December 2011.

5.	The first meeting of the informal group was held on 01 and 02 December 2011 in Geneva, Switzerland. The technical discussion was started and the draft document on the terms of references, the operating principles, the schedule and the work plan for submission to GRSP in December 2011 was finalised. The first progress report was submitted to GRSP in December 2011 and to the World Forum for Harmonization of Vehicle Regulations for the 156th session and to the Executive Committee of the 1998 Agreement for its 34th session in March 2012. At the 156th session the World Forum endorsed in principle the above mentioned terms of references, pending the adoption of the report of the December 2011 session of GRSP. AC.3 endorsed in principle the terms of reference of the informal working group and requested the secretariat to distribute document ECE/TRANS/WP.29-156-11 with an official symbol for June 2012.

6.	The second meeting of the informal group took place on 28 and 29 March 2012 in Osaka, Japan. The discussion was focused on the technical aspects including the accident and benefit analysis. Main priority was also given to discuss the activities on the further development of the certification procedures. For one further work item a task force was initiated to work on the bumper test area for the lower legform impact.

7.	The second progress report was submitted to GRSP in May 2012 and to the World Forum for Harmonization of Vehicle Regulations for the 157th session and to the Executive Committee of the 1998 Agreement for its 35th session in June 2012. During these sessions the first progress report (ECE/TRANS/WP.29/2012/58) and the terms of references including the operating principles, the schedule and the work plan were endorsed. The second progress report (document ECE/TRANS/ WP.29-157-21) was distributed with an official symbol for November 2012.

8.	The third meeting of the informal group was held on 29 and 30 May 2012 in Paris, France. Main topics discussed during the meeting were related to accident data on pedestrian injuries, the cost benefit assessment and the setup of certification corridors.

9.	The fourth meeting of the informal group took place on 17 to 18 September 2012 in Washington DC, United States of America. The group carried on the discussions of the third meeting, while the main focus was given to the finalisation of certification corridors and the cost benefit assessment for the introduction of the FlexPLI. Further priority was given to agree on the plan for an international vehicle test programme with the FlexPLI.

10.	The fifth meeting was held on 6 and 7 December 2012 in Bergisch Gladbach, Germany.

III.	Subjects for review and tasks to be undertaken (terms of reference)

11.	The informal group has set up an activity list to address remaining items for the introduction of the FlexPLI:

- Review and consideration of remaining items

  - Review of Flex-TEG activities  to reach common understanding

  - Assessment of biofidelity (comparison of FlexPLI and EEVC lower legform impactor)

  - Assessment of benefit and costs (injury reduction, additional benefit compared to EEVC lower legform impactor)

  - Technical specifications (drawings) and PADI (user manual)

  - Evaluation of durability

  - Test procedure (rebound phase, best practice, velocity measurement etc.)

  - Certification tests

  - Review and exchange of test results

  - Evaluation of reproducibility and repeatability

  - Evaluate and decide on performance / injury criteria and threshold values

  - Evaluation of vehicle countermeasures (assessment of technical feasibility)

- Develop a draft proposal to amend UN Gtr No. 9 - Phase 2

- Develop a complementary draft proposal to amend draft UN Regulation on Pedestrian Safety (including a recommendation for transitional provisions based on item 1).

IV	History of the discussions

12.	Benefit and costs

At the third and fourth meeting the pedestrian experts reviewed again the information from JASIC on the benefit of the FlexPLI. The Alliance of Automobile Manufacturers in the United States of America has undertaken an investigation of the methodology that was presented by JASIC.

One major concern of the Alliance was that the data used does not correctly reflect the current accident situation in the United States due to the outdated dataset and the assumptions made for pedestrian lower limb injuries on the national level by using regional US traffic accident data as a basis for the benefit calculation.

During the meeting no agreement on a common position was reached. The item will stay on the agenda and some participants offered to supply additional information for the next meeting of the informal group.

13.	Biofidelity

This item was re-discussed. The informal group has received additional information on the superior performance of the FlexPLI compared to the current lower legform impactor.

The discussion on the limitations of the FlexPLI in assessing knee injuries was adressed by Japan in presenting information which showed for the Flex-PLI a slightly lower correlation ratio with the human model regarding medial collateral ligament (MCL) elongation, however it has a very good correlation ratio for the anterior cruciate ligament (ACL) elongation as well as tibia bending moment. The topic was closed pending for the submission of new information regarding this subject.

14.	Accident analysis

NHTSA informed delegations about a research project in the USA to investigate the accident situation for pedestrians using the Pedestrian Crash Data Study (PCDS) and the German In-Depth Accident Study (GIDAS). Submission of the final document was announced for end of October 2012. The discussion on the accident analysis is planned to be finalised at the fifth meeting of the informal group.

15.	TF-RUCC (certification procedures)

Japan gave a status report of the activities of the task force reviewing and updating the certification corridors. A round robin certification test series confirmed a stable performance of the legform impactors. The task force has finalised the work and succeeded in proposing updated certification corridors to be used for the calibration of the flexible legform impactors on the assembly and component level.

The corridors were agreed by the informal group as final. It was also indicated that an evaluation of the stability of performance of the flexible legform impactors will be done during vehicle testing.

16.	Technical specifications and user manual

Humanetics presented information on the updated version of the user manual that is going to be distributed to the informal group. The updated manual will be available before the fifth meeting of the informal group.

The chairman informed the participants about the on-going activity at WP.29 to set up a Special Resolution No. 2 in future as a repository for dummies and other test devices used in UNECE regulations. It was noted that the representatives of the UK and the USA will together make a proposal at the upcoming WP.29 session in November 2012 to set up a resolution, named mutual resolution, which can be used for both legislative frameworks of the UNECE regulations, the 1958 and the 1998 agreement.

Humanetics confirmed that they are going to provide drawings of the FlexPLI in November 2012. The informal group agreed to start the discussion and the procedure of reviewing the drawings at the next meeting.

17.	Review and exchange of test results

The informal group discussed the plan to start an international vehicle round robin test programme. A work plan and guidelines for the testing were agreed. It is planned to finalise vehicle testing end of January 2013.

First results of this test programme were presented by BASt. The legform impactors showed a repeatable performance, while the measurements were found to be at a lower level compared to the test results with former flexible legform impactors tested with the same cars. NHTSA indicated that this could have an influence on the discussion about thresholds and injury criteria. Japan explained that results from former flexible legform impactors conditions should not be used for the evaluation but the results with the updated flexible legform impactor.

The Concept Tech GmbH presented information on the influence of friction with regard to the test device used for inverse testing. The informal group is planning to resolve this issue at the next meeting by introducing limits for the friction of test devices.

OICA announced that a proposal will be submitted for the fifth session of the informal group to introduce requirements for the definition of the vehicle setup in terms of riding height.

18.	Test procedure

Within the informal group a task force bumper test area (TF-BTA) was established after the second meeting of the informal group based on a request of the the European Commission. The necessity was shown to improve the procedure for the lower legform test, as the size of the area of the bumper tested is quite limited due to some design features on the front of some vehicles that interact with the current test procedure. The expert of the European Commission has become the chairman of this task force.

A first web meeting of the task force took place on 4 September 2012, where a work plan and an action list to be worked on were discussed (not finalized). The next meeting will be held on 5 December 2012. The task force is expected to forward if possible a proposal to update the lower legform test procedure within the GTR No. 9 to the informal group.

19.	Activity list and schedule

The informal group has reviewed the activity list in terms of identifying outstanding issues to be resolved at the next meetings. One main issue which was not yet covered is the topic performance and injury criteria. The discussion on this subject will be started at the 5th meeting of the informal group.

According to the overall schedule Japan and Germany will provide a first draft informal document for the discussion at the next meeting of the informal group and the GRSP session in December 2012.

V.		Work schedule

20.	Work schedule

May 2011	Proposal of Draft ToR to GRSP (informal document)

June 2011	GRSP to seek consent of WP.29 and AC.3 to mandate new informal group on pedestrian protection

03 November 2011	Constitutional meeting of the IG GTR9-PH2 
(Bonn, DE)

November 2011	Report to WP.29 on activities of IG

01/02 December 2011	First meeting of the IG GTR9-PH2 
(Geneva, CH)

December 2011	Progress-Report to GRSP, submission of Draft ToR to WP.29

March 2012	Progress-Report to WP.29 and adoption of ToR by WP.29 / AC.3

28 and 29 March 2012	Second meeting of the IG GTR9-PH2
(Osaka, JP)

May 2012	Progress-Report to GRSP

29 and 30 May 2012	Third meeting of the IG GTR9-PH2
(Paris, FR)

June 2012	Progress-Report to WP.29

17 to 19 Sept. 2012	Fourth meeting of the IG GTR9-PH2
(Washington DC, USA)

November 2012	Progress-Report to WP.29

06 and 07 December 2012	Fifth meeting of the IG GTR9-PH2
(Bergisch Gladbach, DE)

December 2012	Progress report and submission of informal draft documents to GRSP

March 2013	Sixth meeting of the IG GTR9-PH2

May 2013	Submit formal proposal to GRSP, adoption by GRSP,
submit formal proposal to WP.29

November 2013	Adoption by WP.29

VI.		Documents

21.	Documents for the meetings

| ECE/TRANS/WP.29/GRSP/2011/13 | Japan | Proposal for Amendment 2 to global technical regulation No. 9 (Pedestrian safety |
| --- | --- | --- |
| GRSP-49-38 | Japan and Germany | Draft terms of reference for the informal group on pedestrian safety phase 2 (IG PS2) |
| ECE/TRANS/WP.29/1091 |  | Reports of the World Forum for Harmonization of Vehicle Regulations on its 154th session, Administrative Committee of the 1958 Agreement on its forty-eighth session, Executive Committee of the 1998 Agreement on its thirty-second session, Administrative Committee of the 1997 Agreement on its eighth session |
| GTR9-C-01 | Chair | Agenda for the constitutional meeting of the Informal Group on Pedestrian Safety Phase 2 (IG PS2) |
| GTR9-C-01r1 | Chair | Agenda for the constitutional meeting of the Informal Group on Pedestrian Safety Phase 2 (IG PS2) |
| GTR9-C-02 | Chair/Secretary | Minutes of the Constitutional Meeting of the Informal Group on Global Technical Regulation No. 9 – Phase 2 (IG GTR9-PH2) |
| GTR9-C-02r1 | Chair/Secretary | Minutes of the Constitutional Meeting of the Informal Group on Global Technical Regulation No. 9 – Phase 2 (IG GTR9-PH2) |
| GTR9-C-03 | Japan/Germany | Informal document GRSP-49-38: Draft Terms of Reference for the Informal Group on Pedestrian Safety Phase 2 (IG PS2) |
| GTR9-C-04 | Japan | History of Development of the FlexPLI |
| GTR9-C-05 | Japan | Review of the FlexPLI TEG Activities |
| GTR9-C-06 | OICA | Comments on the Draft Terms of Reference for the Informal Group on Pedestrian Safety Phase 2 (28/10/2011) |
| GTR9-C-07 | Chair | Final Operating Principles and Terms of Reference for the IG GTR9-PH2 |
| GTR9-C-07r1 | Chair | Final Operating Principles and Terms of Reference for the IG GTR9-PH2 |
| GTR9-C-08 | Secretary | TEG document matrix |
| GTR9-1-01 | Chair/ Secretary | Agenda for the 1st meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) |
| GTR9-1-01r1 | Chair/Secretary | Agenda for the 1st meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) |
| GTR9-1-02 | Chair/ Secretary | Minutes of the 1st meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Draft |
| GTR9-1-02r1 | Chair/ Secretary | Minutes of the 1st meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Final |
| GTR9-1-03 | Japan | Work plan of Task Force Review and Update of Certification Corridors (TF-RUCC) |
| GTR9-1-03r1 | Japan | Work plan of Task Force Review and Update of Certification Corridors (TF-RUCC) |
| GTR9-1-04 | OICA | FlexPLI Version GTR Prototype SN-02 - Durability Assessment |
| GTR9-1-05 | JASIC | Technical Discussion - Biofidelity |
| GTR9-1-05r1 | JASIC | Technical Discussion – Biofidelity (revised) |
| GTR9-1-06 | JASIC | Technical Discussion - Injury Criteria |
| GTR9-1-06r1 | JASIC | Technical Discussion - Injury Criteria (revised) |
| GTR9-1-07 | JASIC | Technical Discussion - Benefit |
| GTR9-1-07r1 | JASIC | Technical Discussion – Benefit (revised) |
| GTR9-1-08 | Humanetics | FlexPLI GTR status 1 / 2 December 2011 |
| GTR9-1-08r1 | Humanetics | FlexPLI GTR status 1 / 2 December 2011 |
| GTR9-1-09 | Japan/ Germany | Informal document WP.29-155-35: Report to the November session of WP.29 on the activities of the IG GTR9-PH2 |
| GTR9-1-10c1 | Humanetics | Flex-GTR changes list since prototype built, status 02 Dec. 2010 (corrected) |
| GTR9-1-11 | Bertrandt | Pendulum Test Scatter |
| GTR9-1-12 | United States | Informal document GRSP-49-23: Update on Pedestrian Leg Testing |
| GTR9-2-01 | Chair/ Secretary | Agenda for the 2nd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Draft |
| GTR9-2-01r1 | Chair/ Secretary | Agenda for the 2nd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Final |
| GTR9-2-02 | Chair/ Secretary | Minutes of the 2nd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Draft |
| GTR9-2-02r1 | Chair/ Secretary | Minutes of the 2nd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Final |
| GTR9-2-03 | BASt | Proposal for a Modification of the Bumper Test Area for Lower and Upper Legform to Bumper Tests |
| GTR9-2-04 | BASt | Robustness of SN02 prototype test results |
| GTR9-2-05 | BASt | Comparison of Filter Classes for FlexPLI |
| GTR9-2-06 | OICA | Technical Specification and PADI |
| GTR9-2-07 | JASIC | Technical Discussion – Benefit; Updated Version of Document GTR9-1-07r1 (Note: The document was not presented since a revision 1 of the document was already available for the meeting) |
| GTR9-2-07r1 | JASIC | Technical Discussion – Benefit; Updated Version of Document GTR9-1-07r1 |
| GTR9-2-08 | Humanetics | Flex PLI GTR meeting actions |
| GTR9-2-09 | Humanetics | FLEX PLI GTR –FE v2.0IG FLEX GTR9-PH2 |
| GTR9-2-10 | OICA | FlexPLI Comparison (Impactors: SN02, SN04, IND-Impactor - Test experiences) |
| GTR9-2-11 | Chair | Informal document WP.29-156-11: First progress report of the informal group on Phase 2 of gtr No. 9 |
| GTR9-2-12 | JASIC | Re-examination of Number of Pedestrians by Injury Severity |
| GTR9-2-13 | Humanetics | FLEX PLI Update for Alliance of Automobile Manufacturers |
| GTR9-2-14 | JARI | Updated Japan Progress Report: Review and Update Certification Test Corridors and Test Methods (added pendulum Test data) |
| GTR9-3-01 | Chair/ Secretary | Agenda for the 3rd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) - Draft |
| GTR9-3-02 | Chair/ Secretary | Minutes of the 3rd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Draft |
| GTR9-3-02r1 | Chair/ Secretary | Minutes of the 3rd meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Final |
| GTR9-3-03 | Germany | Informal document GRSP-51-15: Draft second progress report of the informal group on Phase 2 of UN GTR No. 9 (IG GTR9 - PH2) |
| GTR9-3-04 | Humanetics | Flex PLI GTR User Manual Rev. C |
| GTR9-3-05 | BASt | FlexPLI Prototype SN04 Robustness Test results |
| GTR9-3-06 | BASt | Draft future work plan - testing |
| GTR9-4-01 | Chair/ Secretary | Agenda for the 4th meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) - Draft |
| GTR9-4-01r1 | Chair/ Secretary | Agenda for the 4th meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) - Final |
| GTR9-4-02 | Chair/ Secretary | Minutes of the 4th meeting of the Informal Group on Global Technical Regulation No. 9 - Phase 2 (IG GTR9-PH2) – Draft |
| GTR9-4-03 | Secretary | Status of activity list items |
| GTR9-4-04 | United Kingdom | EEVC WG10 report, Nov. 1994 |
| GTR9-4-05 | United Kingdom | EEVC WG10 report, 15.ESV paper, May 1996 |
| GTR9-4-06 | United Kingdom | EEVC WG17 report, Dec. 1998, update Sep. 2002 |
| GTR9-4-07 | Chair of TF-RUCC | Activity report TF-RUCC |
| GTR9-4-08 | Chair of TF-RUCC | Guideline to conduct FlexPLI round robin car tests, smooth and effectively |
| GTR9-4-09 | Humanetics | Round robin certification test results |
| GTR9-4-10 | Chair | Draft document “Special resolution No2” (WP.29-157-16) |
| GTR9-4-11 | Concept Tech | Investigation of the influence of friction within the inverse certification test setup |
| GTR9-4-12 | Alliance / JP | Letter of JP Research report on benefit assessment of the FlexPLI |
| GTR9-4-13 | Alliance / JP | JP Research presentation on benefit assessment of the FlexPLI |
| GTR9-4-14 | BASt | Comparison of FlexPLI performance in vehicle tests with prototype and series production legforms |
| GTR9-4-15 | Chair | Informal document WP.29-157-21: Second progress report of the informal group on Phase 2 of gtr No. 9 |
| GTR9-4-16 | BASt | Pedestrian lower extremity injury risk |
| GTR9-4-16r1 | BASt | Pedestrian lower extremity injury risk – updated information |
| GTR9-4-17 | Vice-Chair | FlexPLI round robin car test schedule |
| GTR9-4-18 | BASt | FlexPLI vs. EEVC WG17 PLI, benefit estimation |
| GTR9-4-19 | NHTSA | Overview of pedestrian protection activities for USA |
| GTR9-4-20 | JASIC | Validation of Pedestrian Lower Limb Injury Assessment using Subsystem Impactors (IRCOBI paper Sept. 2012) |
| GTR9-4-21 | USCAR / Alliance | USCAR/OSRP Pedestrian Lower Leg Response Research test series |
| GTR9-4-22 | IG GTR9-PH2 | Checklist for Vehicle Testing |
| TF-RUCC-2-03 | BASt | FlexPLI Inverse Certification Corridors-Further Test Results |
| TF-RUCC-2-04 | ACEA | Comments on Inverse Certification Test Procedure |
| TF-RUCC-2-05 | Humanetics | Humanetics Inverse and Round Robin Leg Preparation |
| TF-RUCC-2-06 | BGS Boehme & Gehring | Status of the FlexPLI – Inverse Certification |
| TF-RUCC-2-07r1 | TF-RUCC chair | Japan Progress Report: Review and Update Certification Test Corridors and Test Methods |