Economic Commission for Europe

Inland Transport Committee

Working Party on the Transport of Dangerous Goods

Joint Meeting of the RID Committee of Experts and the
Working Party on the Transport of Dangerous Goods				26 September 2013

Proposal

23.	Modify 4.3.2.2.1 as follows (new text underlined):

4.3.2.2.1	The following degrees of filling shall not be exceeded in tanks intended for the carriage of liquids at ambient temperatures:

(a)	for flammable substances, for environmentally hazardous substances and for the flammable environmentally hazardous substances, without additional risks (e.g. toxicity or corrosivity), in tanks with a breather device or with safety valves (even where preceded by a bursting disc):

Degree of filling =1001+α(50 - tF)% of capacity

(b)	for toxic or corrosive substances (whether flammable or environmentally hazardous or not) in tanks with a breather device or with safety valves (even where preceded by a bursting disc):

Degree of filling =981+α(50 - tF)% of capacity

(c)	for flammable substances, for environmentally hazardous substances and for slightly toxic or corrosive substances (whether flammable or environmentally hazardous or not) in hermetically closed tanks without a safety device:

Degree of filling =971+α(50 - tF)% of capacity

(d)	for highly toxic, toxic, highly corrosive or corrosive substances (whether flammable or environmentally hazardous or not) in hermetically closed tanks without a safety device:

Degree of filling =951+α(50 - tF)% of capacity