|  | INF 28 |  |
| --- | --- | --- |
| Economic Commission for Europe Inland Transport Committee Working Party on the Transport of Dangerous Goods Joint Meeting of Experts on the Regulations annexed to the European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways (ADN) (ADN Safety Committee) Twenty-third session Geneva, 26 - 30 August 2013 Item 3 (b) of the provisional agenda Implementation of ADN: Special authorizations, derogations and equivalents |  | 12 August 2013 |

LNG as fuel - Introduction Eiger-Nordwand

Transmitted by the Government of the Netherlands

I.	Introduction

Danser Group has planned to retrofit the current pusher-barge combination Eiger-Nordwand with a dual fuel (LNG-diesel) engine. This will be the first container vessel and also the first retrofit to use a duel fuel engine. This document will give a first introduction of this project. A formal application for a derogation will be submitted to the January 2014 meeting of the Safety Committee.

II.	Eiger-Nordwand

The Eiger-Nordwand is a pusher-barge combination (348 TEU) to be refitted with a dual fuel (LNG-diesel) engine. The LNG-fuel tank (60m3) is planned to be placed in the cargo area adjacent to the engine room. This will reduce the cargo area by about 6 TEU.

Loading and unloading operations are also foreseen to take place above the LNG fuel tank. Therefore the fuel tank will be protected by a reinforced structure capable of withstanding the impact of a falling container in case the container is not attached to the crane correctly.