|  | United Nations | ECE/TRANS/WP.29/GRPE/2013/9 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 22 March 2013 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Sixty-sixth session

Geneva, 3-7 June 2013

Item 4(b) of the provisional agenda

UN Regulations Nos. 68 (measurement of the

maximum speed, including electric vehicles),

83 (emissions of M1 and N1 vehicles),

101 (CO2 emissions/fuel consumption) and

103 (replacement pollution control devices)

Proposal for Supplement 3 to the 01 series of amendments to Regulation No. 101 (CO2 emissions/fuel consumption)

Submitted by the expert from the European Commission*

The text reproduced below was prepared by the expert from the European Commission to align the requirements of UN Regulation No. 101 with those of European Union Regulation (EC) Nos. 459/2012 and 630/2012.

The modifications to the original English text are marked using track changes. The same modifications in the French and Russian versions are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 2.16.1., amend to read:

"2.16.1.	"Hybrid electric vehicle (HEV)" means a vehicle, including vehicles which draw energy from a consumable fuel only for the purpose of re-charging the electrical energy/power storage  device, that, for the purpose of mechanical propulsion, draws energy from both of the following on-vehicle sources of stored energy/power:

(a)	a consumable fuel;

(b)	a battery, capacitor, flywheel/generator or other electrical energy/power storage device;"

Insert new paragraphs 2.20. and 2.21., to read:

"2.20.	"Flex fuel H2NG vehicle" means a flex fuel vehicle that can run on different mixtures of hydrogen and NG/biomethane;

2.21.	"Hydrogen fuel cell vehicle" means a vehicle powered by a fuel cell that converts chemical energy from hydrogen into electric energy, for propulsion of the vehicle."

Footnote 2, amend to read:

"2	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.2/Amend.1 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htm."

Paragraph 5.1., split in paragraphs 5.1. and 5.1.1. to read:

"5.1.	General

5.1.1.	The components liable to affect the emissions of CO2 and fuel consumption or the electric energy consumption shall be so designed, constructed and assembled as to enable the vehicle, in normal use, despite the vibrations to which it may be subjected, to comply with the provisions of this Regulation."

Insert new paragraphs 5.1.2. to 5.1.3., to read:

5.1.2.	The daytime running lamps of the vehicle as defined in Section 2 of Regulation No 48 shall be switched on during the test cycle. The vehicle tested shall be equipped with the daytime running lamp system that has the highest electrical energy consumption among the daytime running lamp systems, which are fitted by the manufacturer to vehicles in the group represented by the type-approved vehicle. The manufacturer shall supply appropriate technical documentation to the type-approval authorities in this respect.

5.1.3.	Table A illustrates the application of the test requirements for type approval of a vehicle.

# Table A

Application of the test requirements: CO2 emissions, fuel consumption, electric energy consumption and electric range

| Vehicles with positive ignition engines including hybrids |  |  | Test? |
| --- | --- | --- | --- |
| Mono fuel | Petrol (E5) |  | Yes |
|  | LPG |  | Yes |
|  | NG/Biomethane |  | Yes |
|  | Hydrogen |  | Yes |
| Bi-fuel1 | Petrol (E5) | LPG | Yes (both fuels) |
|  | Petrol (E5) | NG/Biomethane | Yes (both fuels) |
|  | Petrol (E5) | Hydrogen | Yes (both fuels) |
| Flex-fuel1 | Petrol (E5) | Ethanol (E85) | Yes (both fuels) |
|  | NG/Biomethane | H2NG | Yes (both fuels) |
| Vehicles with compression ignition engines including hybrids |  |  | Test? |
| Flex fuel | Diesel (B5) | Biodiesel | Yes (B5 only)2 |
| Mono fuel | Diesel (B5) |  | Yes |
| Other vehicles |  |  | Test? |
| Pure electric vehicles |  |  | Yes |
| Hydrogen Fuel cell vehicles |  |  | Yes |
| 1 When a bi-fuel vehicle is combined with a flex fuel vehicle, both test requirements are applicable. 2 This provision is temporary, further requirements for biodiesel shall be proposed later on. |  |  |  |

# "

Paragraph 5.2.3. and 5.2.4., amend to read (leaving footnote 3 unchanged):

"5.2.3.	Fuel consumption values must be expressed in litres per 100 km (in the case of petrol, LPG, ethanol (E85) and diesel), in m3 per 100 km (in the case of NG/biomethane and H2NG) or in kg per 100 km (in the case of hydrogen) and are calculated according to paragraph 1.4.3. of Annex 6. The results will be rounded to the first decimal place.

5.2.4.	For the purpose of the calculation mentioned in paragraph 5.2.3., the fuel consumption shall be expressed in appropriate units and the following fuel characteristics shall be used:

(a)	Density: measured on the test fuel according to ISO 3675 or an equivalent method. For petrol, diesel, biodiesel and ethanol (E85 and E75) the density measured at 15 ºC will be used; for LPG and natural gas/biomethane a reference density will be used, as follows:

0.538 kg/litre for LPG

0.654 kg/m3 for NG 3;

(b)	Hydrogen-carbon ratio:  fixed values will be used which are:

C1H1.89 O0.016 for petrol;

C1H1.86 O0.005 for diesel;

C1H2.525 for LPG (liquefied petroleum gas);

CH4 for NG (natural gas) and biomethane;

C1H2.74 O0.385 for ethanol (E85);

C1 H2.61 O0.329 for ethanol (E75)."

Paragraph 11., amend to read:

"11.	Production definitively discontinued

…"

Annex 4, items 7.1.2.1. to 7.1.2.3., amend to read (inserting also a new footnote 6):

"7.1.2.1.	Fuel consumption (urban conditions):	l/100 km or m3/100 km or kg/100 km 6

7.1.2.2.	Fuel consumption (extra-urban conditions):	l/100 km or m3/100 km or kg/100 km 6

7.1.2.3.	Fuel consumption (combined):	l/100 km or m3/100 km or kg/100 km 6

____________

6	Delete where not applicable (there are cases where nothing needs to be deleted when more t	han one entry is applicable)."

Annex 6

Paragraph 1.1., amend to read:

"1.1.	Emissions of carbon dioxide (CO2) and fuel consumption of vehicles powered by an internal combustion engine only shall be determined according to the procedure for the Type I test as defined in Annex 4a of Regulation No. 83 in force at the time of the approval of the vehicle."

Paragraph 1.3., amend to read:

"1.3.	In addition to the conditions specified in Annex 4a of Regulation No. 83...:"

Paragraph 1.4.1., amend to read:

"1.4.1.	The mass emission of CO2, expressed in g/km, shall be calculated from the measurement results using the provisions defined in  paragraph 6.6. of Annex 4a to Regulation No. 83 in force at the time of the approval of the vehicle."

Paragraphs 1.4.2. and 1.4.3., amend to read:

"1.4.2.	The fuel consumption values shall be calculated from the emissions of hydrocarbons, carbon monoxide, and carbon dioxide determined from the measurement results using the provisions defined in paragraph 6.6. of Annex 4a to  Regulation No. 83 in force at the time of the approval of the vehicle.

1.4.3.	The fuel consumption, expressed in litres per 100 km (in the case of petrol, LPG, ethanol (E85) and diesel), in m3 per 100 km (in the case of NG/biomethane and H2NG) or in kg per 100 km (in the case of hydrogen) is calculated by means of the following formulae:

…

(f)	For vehicles with a compression ignition engine fuelled with diesel (B5):

FC = (0.116 / D)  [(0.861 ·HC) + (0.429 ·CO) + (0.273 ·CO2)];

(g)	For vehicles with a positive ignition engine fuelled by H2NG:

(h)	For vehicles fuelled by gaseous hydrogen:

Under previous agreement with the type-approval authority, and for vehicles fuelled either by gaseous or liquid hydrogen, the manufacturer may choose as alternative to the method above, either the formula

FC = 0.1 · (0.1119 · H2O + H2)

or a method according to standard protocols such as SAE J2572.

In these formulae:

FC	=	the fuel consumption in litre per 100 km (in the case of petrol, ethanol, LPG, diesel or biodiesel) in m3 per 100 km (in the case of natural gas and H2NG) or in kg per 100 km in the case of hydrogen.

HC	=	the measured emission of hydrocarbons in g/km

CO	=	the measured emission of carbon monoxide in g/km

CO2	=	the measured emission of carbon dioxide in g/km

H2O	=	the measured emission of H2O in g/km

H2	=	the measured emission of H2 in g/km

A	=	quantity of NG/biomethane within the H2NG mixture, expressed in per cent volume

D	=	the density of the test fuel

In the case of gaseous fuels this is the density at 15 °C.

d	=	the theoretical distance covered by a vehicle tested under the Type I test in km.

p1	=	pressure in gaseous fuel tank before the operating cycle in Pa;

p2	=	pressure in gaseous fuel tank after the operating cycle in Pa;

T1	=	temperature in gaseous fuel tank before the operating cycle in K.

T2	=	temperature in gaseous fuel tank after the operating cycle in K.

Z1	=	compressibility factor of the gaseous fuel at p1 and T1

Z2	=	compressibility factor of the gaseous fuel at p2 and T2

V	=	inner volume of the gaseous fuel tank in m3

The compressibility factor shall be obtained from the following table:

|  |  | T (K) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 100 | 200 | 300 | 400 | 500 | 600 | 700 | 800 | 900 |
| p (bar) | 33 | 0.859 | 1.051 | 1.885 | 2.648 | 3.365 | 4.051 | 4.712 | 5.352 | 5.973 | 6.576 |
|  | 53 | 0.965 | 0.922 | 1.416 | 1.891 | 2.338 | 2.765 | 3.174 | 3.57 | 3.954 | 4.329 |
|  | 73 | 0.989 | 0.991 | 1.278 | 1.604 | 1.923 | 2.229 | 2.525 | 2.81 | 3.088 | 3.358 |
|  | 93 | 0.997 | 1.042 | 1.233 | 1.47 | 1.711 | 1.947 | 2.177 | 2.4 | 2.617 | 2.829 |
|  | 113 | 1 | 1.066 | 1.213 | 1.395 | 1.586 | 1.776 | 1.963 | 2.146 | 2.324 | 2.498 |
|  | 133 | 1.002 | 1.076 | 1.199 | 1.347 | 1.504 | 1.662 | 1.819 | 1.973 | 2.124 | 2.271 |
|  | 153 | 1.003 | 1.079 | 1.187 | 1.312 | 1.445 | 1.58 | 1.715 | 1.848 | 1.979 | 2.107 |
|  | 173 | 1.003 | 1.079 | 1.176 | 1.285 | 1.401 | 1.518 | 1.636 | 1.753 | 1.868 | 1.981 |
|  | 193 | 1.003 | 1.077 | 1.165 | 1.263 | 1.365 | 1.469 | 1.574 | 1.678 | 1.781 | 1.882 |
|  | 213 | 1.003 | 1.071 | 1.147 | 1.228 | 1.311 | 1.396 | 1.482 | 1.567 | 1.652 | 1.735 |
|  | 233 | 1.004 | 1.071 | 1.148 | 1.228 | 1.312 | 1.397 | 1.482 | 1.568 | 1.652 | 1.736 |
|  | 248 | 1.003 | 1.069 | 1.141 | 1.217 | 1.296 | 1.375 | 1.455 | 1.535 | 1.614 | 1.693 |
|  | 263 | 1.003 | 1.066 | 1.136 | 1.207 | 1.281 | 1.356 | 1.431 | 1.506 | 1.581 | 1.655 |
|  | 278 | 1.003 | 1.064 | 1.13 | 1.198 | 1.268 | 1.339 | 1.409 | 1.48 | 1.551 | 1.621 |
|  | 293 | 1.003 | 1.062 | 1.125 | 1.19 | 1.256 | 1.323 | 1.39 | 1.457 | 1.524 | 1.59 |
|  | 308 | 1.003 | 1.06 | 1.12 | 1.182 | 1.245 | 1.308 | 1.372 | 1.436 | 1.499 | 1.562 |
|  | 323 | 1.003 | 1.057 | 1.116 | 1.175 | 1.235 | 1.295 | 1.356 | 1.417 | 1.477 | 1.537 |
|  | 338 | 1.003 | 1.055 | 1.111 | 1.168 | 1.225 | 1.283 | 1.341 | 1.399 | 1.457 | 1.514 |
|  | 353 | 1.003 | 1.054 | 1.107 | 1.162 | 1.217 | 1.272 | 1.327 | 1.383 | 1.438 | 1.493 |

In the case that the needed input values for p and T are not indicated in the table, the compressibility factor shall be obtained by linear interpolation between the compressibility factors indicated in the table, choosing the ones that are the closest to the sought value."

Annex 8

Paragraph 1.1., amend to read:

"1.1.	This annex defines the specific provisions regarding type-approval of a hybrid electric vehicle (HEV) as defined in paragraph 2.16.1. of this Regulation."

Paragraphs 1.4.1. to 1.4.3., amend to read:

"1.4.1.	For vehicles with a manual transmission the driving cycle described in paragraph 6.1. of Annex 4a  to Regulation No. 83 in force at the time of approval of the vehicle shall be used, including the prescribed gear shifting points.

1.4.2.	For vehicles with a special gear shifting strategy the gear shifting points prescribed in paragraph 6.1. of Annex 4a  to Regulation No. 83 are not applied.  For these vehicles the driving cycle specified in paragraph 6.1.3.2. of Annex 4a  to Regulation No. 83 in force at the time of approval of the vehicle shall be used.  Concerning gear shifting points, these vehicles shall be driven according to the manufacturer’s instructions, as incorporated in the drivers’ handbook of production vehicles and indicated by a technical gear shift instrument (for drivers information).

1.4.3.	For vehicles with an automatic transmission the driving cycle specified in paragraph 6.1.3.2. of Annex 4a to Regulation No. 83 in force at the time of approval of the vehicle shall be used."

Paragraph 3.2.3.4., amend to read:

"3.2.3.4.	The exhaust gases shall be analysed according to Annex 4a of Regulation No. 83 in force at the time of approval of the vehicle."

Paragraph 3.3.2.4., amend to read:

"3.3.2.4.	The exhaust gases shall be analysed according to Annex 4a of Regulation No. 83 in force at the time of approval of the vehicle."

Paragraph 4.2.4.4., amend to read:

"4.2.4.4.	The exhaust gases shall be analysed according to Annex 4a of Regulation No. 83 in force at the time of approval of the vehicle."

Paragraph 4.3.2.4., amend to read:

"4.3.2.4.	The exhaust gases shall be analysed according Annex 4a of Regulation No. 83 in force at the time of approval of the vehicle."

Annex 9, paragraph 4.2.2.1.1., amend to read:

"4.2.2.1.1.	The applicable test sequence and accompanying gear shift prescription, as defined in paragraph 1.4. of Annex 8, is applied on a chassis dynamometer adjusted as described in Appendices 1, 6 and 7 of Annex 4a to Regulation No. 83, until the end of the test criteria is reached.

…"

Annex 10, paragraph 3.2.1., amend to read:

"3.2.1.	Preparation of the vehicle, if required, for the emissions test during a regeneration phase, may be completed using the preparation cycles in paragraph 6.3. of Annex 4A to Regulation No. 83 or equivalent engine test bench cycles, depending on the loading procedure chosen in paragraph 3.1.2. above."

II.	Justification

1.	The text aims to align the requirements of UN Regulation No. 101 with those of European Union Regulation (EC) Nos. 459/2012 and 630/2012.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.