|  | United Nations | ECE/TRANS/WP.29/GRRF/2013/18 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 July 2013 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-fifth session

Geneva, 17-19 September 2013

Item 7(b) of the provisional agenda

Tyres – Regulation No. 117

Proposal for corrigendum to Regulation No. 117 (Tyres, rolling resistance, rolling noise and wet grip)

Submitted by the experts from the European Tyre and Rim Technical Organisation*

The text reproduced below was prepared by the experts from the European Tyre and Rim Technical Organisation (ETRTO) in order to correct ECE/TRANS/WP.29/2012/54. The modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Annex 7

Paragraph 3.1.1., amend to read:

"3.1.1.	Test course

The braking tests shall be done on a flat test surface of sufficient length and width, with a maximum 2 per cent gradient, covered with packed snow.

The snow surface shall be composed of a hard packed snow base at least 3 cm thick and a surface layer of medium packed and prepared snow about 2 cm thick.

The air temperature, measured about one meter above the ground, shall be between -2 °C and -15 °C; the snow temperature, measured at a depth of about one centimetre, shall be between -4 4 °C and -15 °C.”

Appendix 2

Paragraph 2.1., amend the table to read:

"

|  | At start of tests | At end of tests | Specification |
| --- | --- | --- | --- |
| weather |  |  |  |
| ambient temperature |  |  | -2 °C to -15 °C |
| snow temperature |  |  | -4 4 °C to -15 °C |
| CTI index |  |  | 75 to 85 |
| other |  |  |  |

"

II.	Justification

The snow temperature range for the snow grip test in the document ECE/TRANS/WP.29/2012/54 e is prescribed as 4°C to ‐15°C.

This is not the correct range.

As a reference, in ASTM 1805‐06 (Standard Test Method for Single Wheel Driving Traction in a Straight Line on Snow‐ and Ice‐Covered Surfaces) the range is ‐4°C to ‐15°C.

Amendments as shown in documents ECE-TRANS-WP29-GRRF-2011-29 are correct:

Annex 7, paragraph 3.1.1., amend to read (footnote 1 remains unchanged):

"3.1.1.	Test course

The braking tests shall be done on a flat test surface of sufficient length and width, with a maximum 2 per cent gradient, covered with packed snow.

The snow surface shall be composed of a hard packed snow base at least 3 cm thick and a surface layer of medium packed and prepared snow about 2 cm thick.

Both The air temperature, measured about one meter above the ground, shall be between -2°C and -15°C; and the snow temperature, measured at a depth of about one centimetre, shall be between  2-4°C and -15°C.

It is recommended to avoid direct sunlight, large variations of sunlight or humidity, as well as wind.

The snow compaction index measured with a CTI penetrometer 1 shall be between 75 and 85.

_____________

1	See appendix of ASTM standard F1805-06 for details."

Annex 7, Appendix, the title, amend to read:

"Test Reports and Test Data for C1 and C2 tyres"

Paragraph 2.1., amend the table to read:

"

|  | At start of tests | At end of tests | Specification |
| --- | --- | --- | --- |
| weather |  |  |  |
| ambient temperature |  |  | -2 °C to -15 °C |
| snow temperature |  |  | 2-4 °C to -15 °C |
| CTI index |  |  | 75 to 85 |
| other |  |  |  |

"

_____________________

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.