|  | United Nations | ECE/TRANS/WP.29/GRRF/2013/20 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 July 2013 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-fifth session

Geneva, 17-19 September 2013

Item 7(c) of the provisional agenda

Tyres – Other business

Proposal for amendments to Regulation No. 75 (Tyres for motorcycles/mopeds)

Submitted by the expert from the European Tyre and Rim Technical Organisation*

The text reproduced below was prepared by the experts from the European Tyre and Rim Technical Organisation (ETRTO) to include ATV tyres in this Regulation. The document is based on the document ECE/TRANS/505/Rev.1/Add.74/Rev.2. The modifications to the existing text of the Regulation are underlined for new or struck through for deleted characters.

I.	Proposal

Paragraph 2.1.3., amend to read:

"2.1.3.	… moped, all terrain (ATV)."

Add new paragraph 2.16.4.

"1.16.4.	The prefix "AT" for all terrain (ATV) tyres (e.g. AT 18x11-8)"

Add new paragraph 2.34.

"2.34.		ATV (all terrain vehicle) tyre:  means a low pressure tyre designed primarily for the equipment of "quadricycles" (categories L6 and L7). They may also equip mopeds (categories L1 and L2) or agricultural vehicles."

Add new paragraph 3.1.15.

"3.1.15.	For all terrain (ATV) tyres, the symbol identifying the reference inflation pressure as follows:

| Symbol | Reference Inflation Pressure (kPa) |
| --- | --- |
|  | 25 |
|  | 35 |
|  | 45 |

"

Add new paragraph 4.1.16.

"4.1.16.	For all terrain (ATV) tyres, the symbol identifying the reference inflation pressure (see paragraph 3.1.15.)."

Annex 1, paragraph 5.2., amend to read:

"5.2.		… moped, all terrain (ATV) 2/"

Annex 1, add a new paragraph 5.6

"5.6.		For all terrain (ATV) tyres, the symbol (stars) identifying the reference inflation pressure."

Annex 5, add a new table

"Table 8: Tyres for all terrain vehicles (ATV).

Table 8 (1 of 3)

Tyres for all terrain vehicles (ATV)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT16x6-8 | 5 | 154 | 394 | 406 | 418 | 168 |
| AT16x7-7 | 5.5 | 177 | 392 | 406 | 420 | 193 |
| AT16x8-7 | 6.5 | 204 | 392 | 406 | 420 | 222 |
| AT18x7-7 | 5.5 | 177 | 440 | 457 | 474 | 193 |
| AT18x7-8 | 5.5 | 177 | 442 | 457 | 472 | 193 |
| AT18x8-7 | 6.5 | 204 | 440 | 457 | 474 | 222 |
| AT18x9-8 | 7 | 227 | 442 | 457 | 472 | 247 |
| AT18x9.5-8 | 7.5 | 240 | 442 | 457 | 472 | 262 |
| AT18x10-10 | 8 | 254 | 445 | 457 | 469 | 277 |
| AT18x10-8 | 8 | 254 | 442 | 457 | 472 | 277 |
| AT18x11-10 | 9 | 281 | 445 | 457 | 469 | 306 |
| AT18x11-8 | 9 | 281 | 442 | 457 | 472 | 306 |
| AT18x11-9 | 9 | 281 | 443 | 457 | 471 | 306 |
| AT19x6-10 | 5 | 154 | 469 | 483 | 497 | 168 |
| AT19x7-8 | 5.5 | 177 | 466 | 483 | 500 | 193 |
| AT19x8-8 | 6.5 | 204 | 466 | 483 | 500 | 222 |
| AT19x9-8 | 7 | 227 | 466 | 483 | 500 | 247 |
| AT19x10-9 | 8 | 254 | 468 | 483 | 498 | 277 |
| AT20x7-10 | 5.5 | 177 | 493 | 508 | 523 | 193 |
| AT20x7-8 | 5.5 | 177 | 490 | 508 | 526 | 193 |
| AT20x7-9 | 5.5 | 177 | 491 | 508 | 525 | 193 |
| AT20x8-10 | 6.5 | 204 | 493 | 508 | 523 | 222 |
| AT20x10-10 | 8 | 254 | 493 | 508 | 523 | 277 |
| AT20x10-8 | 8 | 254 | 490 | 508 | 526 | 277 |
| AT20x10-9 | 8 | 254 | 491 | 508 | 525 | 277 |
| AT20x11-10 | 9 | 281 | 493 | 508 | 523 | 306 |
| AT20x11-8 | 9 | 281 | 490 | 508 | 526 | 306 |
| AT20x11-9 | 9 | 281 | 491 | 508 | 525 | 306 |
| AT21x7-10 | 5.5 | 177 | 516 | 533 | 550 | 193 |
| AT21x8-9 | 6.5 | 204 | 515 | 533 | 551 | 222 |
| AT21x10-10 | 8 | 254 | 516 | 533 | 550 | 277 |
| AT21x10-8 | 8 | 254 | 513 | 533 | 553 | 277 |
| AT21x11-8 | 9 | 281 | 513 | 533 | 553 | 306 |
| AT21x11-9 | 9 | 281 | 515 | 533 | 551 | 306 |
| AT21x12-8 | 9.5 | 304 | 513 | 533 | 553 | 331 |
| AT22x7-10 | 5.5 | 177 | 541 | 559 | 577 | 193 |
| AT22x7-11 | 5.5 | 177 | 542 | 559 | 576 | 193 |
| AT22x7-12 | 5.5 | 177 | 544 | 559 | 574 | 193 |
| AT22x7-9 | 5.5 | 177 | 539 | 559 | 579 | 193 |
| AT22x8-10 | 6.5 | 204 | 541 | 559 | 577 | 222 |
| AT22x9-10 | 7 | 227 | 541 | 559 | 577 | 247 |

Table 8 (2 of 3)

Tyres for all terrain vehicles (ATV tyres)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT22x9-11 | 7 | 227 | 542 | 559 | 576 | 247 |
| AT22x9-8 | 7 | 227 | 538 | 559 | 580 | 247 |
| AT22x10-10 | 8 | 254 | 541 | 559 | 577 | 277 |
| AT22x10-8 | 8 | 254 | 538 | 559 | 580 | 277 |
| AT22x10-9 | 8 | 254 | 539 | 559 | 579 | 277 |
| AT22x11-10 | 9 | 281 | 541 | 559 | 577 | 306 |
| AT22x11-8 | 9 | 281 | 538 | 559 | 580 | 306 |
| AT22x11-9 | 9 | 281 | 539 | 559 | 579 | 306 |
| AT22x12-8 | 9,5 | 304 | 538 | 559 | 580 | 331 |
| AT22.5x10-8 | 8 | 254 | 550 | 572 | 594 | 277 |
| AT23x7-10 | 5.5 | 177 | 564 | 584 | 604 | 193 |
| AT23x8-10 | 6.5 | 204 | 564 | 584 | 604 | 222 |
| AT23x8-11 | 6.5 | 204 | 566 | 584 | 602 | 222 |
| AT23x8-12 | 6.5 | 204 | 567 | 584 | 601 | 222 |
| AT23x10-10 | 8 | 254 | 564 | 584 | 604 | 277 |
| AT23x10-12 | 8 | 254 | 567 | 584 | 601 | 277 |
| AT23x10-8 | 8 | 254 | 561 | 584 | 607 | 277 |
| AT23x11-9 | 9 | 281 | 563 | 584 | 605 | 306 |
| AT23x12-9 | 9,5 | 304 | 563 | 584 | 605 | 331 |
| AT24x8-11 | 6.5 | 204 | 590 | 610 | 630 | 222 |
| AT24x8-12 | 6.5 | 204 | 592 | 610 | 628 | 222 |
| AT24x9-11 | 7 | 227 | 590 | 610 | 630 | 247 |
| AT24x9-12 | 7 | 227 | 592 | 610 | 628 | 247 |
| AT24x10-11 | 8 | 254 | 590 | 610 | 630 | 277 |
| AT24x11-10 | 9 | 281 | 589 | 610 | 631 | 306 |
| AT24x11.5-10 | 9 | 290 | 589 | 610 | 631 | 316 |
| AT24x12-10 | 9,5 | 304 | 589 | 610 | 631 | 331 |
| AT24x13-9 | 10.5 | 330 | 587 | 610 | 633 | 360 |
| AT25x8-10 | 6.5 | 204 | 612 | 635 | 658 | 222 |
| AT25x8-12 | 6.5 | 204 | 615 | 635 | 655 | 222 |
| AT25x8-13 | 6.5 | 204 | 617 | 635 | 653 | 222 |
| AT25x10-10 | 8 | 254 | 612 | 635 | 658 | 277 |
| AT25x10-12 | 8 | 254 | 615 | 635 | 655 | 277 |
| AT25x11-10 | 9 | 281 | 612 | 635 | 658 | 306 |
| AT25x11-12 | 9 | 281 | 615 | 635 | 655 | 306 |
| AT25x11-9 | 9 | 281 | 611 | 635 | 659 | 306 |
| AT25x11.5-9 | 9 | 290 | 611 | 635 | 659 | 316 |
| AT25x12-10 | 9,5 | 304 | 612 | 635 | 658 | 331 |
| AT25x12-9 | 9,5 | 304 | 611 | 635 | 659 | 331 |
| AT25x13-9 | 10.5 | 330 | 611 | 635 | 659 | 360 |

Table 8 (3 of 3)

Tyres for all terrain vehicles (ATV tyres)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT26x9-12 | 7 | 227 | 639 | 660 | 681 | 247 |
| AT26x10-12 | 8 | 254 | 639 | 660 | 681 | 277 |
| AT26x10.5-12 | 8.5 | 268 | 639 | 660 | 681 | 292 |
| AT26x12-10 | 9.5 | 304 | 636 | 660 | 684 | 331 |
| AT26x12-12 | 9.5 | 304 | 639 | 660 | 681 | 331 |
| AT26x12-14 | 9.5 | 304 | 642 | 660 | 678 | 331 |
| AT27x9-12 | 7 | 227 | 663 | 686 | 709 | 247 |
| AT27x11-12 | 9 | 281 | 663 | 686 | 709 | 306 |
| AT27x12-10 | 9.5 | 304 | 660 | 686 | 712 | 331 |
| AT27x12-12 | 9.5 | 304 | 663 | 686 | 709 | 331 |

(1)	Tyres in Radial construction are identified by the letter "R" in place of "-" (e.g. AT16x6 R 8). In case of Tyre Size designations not included in this table, refer to any edition of one of the following International Tyre Standards for the relevant dimensional data:

(a)	The European Tyre and Rim Technical Organisation (ETRTO) 'Standards Manual';

(b)	The Tire and Rim Association Inc. (TRA) 'Year Book';

(c)	The Japan Automobile Tire Manufacturers Association (JATMA) 'Year Book';

(d)	The Scandinavian Tyre and Rim Organisation (STRO) 'Data Book'.

In that case the nominated International Tyre Standard, to which the tyre conforms, shall be specified in the application for approval.

"

Annex 6, paragraph 1. and Annex 7, paragraph 1.2, add to the table:

"

|  | Tyre version | Speed Category | Pressure (bar) | Pressure (kPa) |
| --- | --- | --- | --- | --- |
| all terrain (ATV) |  | all | 0.25 | 25 |
|  |  | all | 0.35 | 35 |
|  |  | all | 0.45 | 45 |

"

Annex 8, add a new table for all terrain (ATV) tyres with speed symbol "F":

"

| Speed (km/h) | Variation in Load Carrying Capacity (%) |
| --- | --- |
| 50 and below | + 12 |
| 60 | + 7 |
| 70 | + 3 |
| 80 | 0 |
| 90 | - 5 |
| 100 | - 10 |
| 110 | - 15 |
| 120 | - 20 |
| 130 | - 25 |

"

II.	Justification

The addition of AT tyres into UN Regulation No. 75 would permit the type approval of tyres designed to equip quadricycle covered by the new Regulation (EU) No. 168/2013 of the European Parliament and of the Council of 15 January 2013 on the approval and market surveillance of two- or three-wheel vehicles and quadricycles 
(L category vehicles). These tyres may also equip some agricultural tractors (SbS vehicles) covered by the new EU Regulation.

________________

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.