|  | United Nations | ECE/TRANS/WP.29/GRRF/2013/21 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 5 July 2013 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-fifth session

Geneva, 17-20 September 2013

Item 7(c) of the provisional agenda

Tyres – Other business

Proposal for amendments to Regulation No. 75 (Tyres for motorcycles/mopeds)

Submitted by the expert from the European Tyre and Rim Technical Organisation*

The text reproduced below was prepared by the experts from the European Tyre and Rim Technical Organisation (ETRTO) to remove uncertainty in the test procedure of the second load/speed performance test to be performed with tyres suitable for speeds above 240 km/h for tyres identified by means of letter code "V" within the size designation. The document is based on the document ECE/TRANS/505/Rev.1/Add.74/Rev.2. The modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Annex 7: Procedure for load/speed performance tests

Paragraph 2.5.2., amend to read:

"2.5.2.		Initial tests speed: 30 km/h less than the maximum rated speed of the tyre the speed corresponding to the speed category symbol marked on the tyre (see paragraph 2.28.2. of this Regulation) if a 2.0 m diameter test drum is used, or 40 km/h less if a 1.7 m diameter test drum is used."

Paragraph 2.5.2.1., amend to read:

"2.5.2.1.		The initial test maximum speed to be considered for the second test in case of tyres suitable for speeds above 240 km/h for tyres identified by means of letter code "V" within the size designation (or 270 km/h for tyres identified by means of letter code "Z" within the size designation) is 30 km/h less than the maximum speed specified by the tyre manufacturer (see paragraph 4.1.15.) if a 2.0 m diameter test drum is used, or 40 km/h less if a 1.7 m diameter test drum is used."

Insert a new paragraph 2.5.6.1. to read:

"2.5.6.1.		The maximum test speed for the second test in case of tyres suitable for speeds above 240 km/h for tyres identified by means of letter code "V" within the size designation (or 270 km/h for tyres identified by means of letter code "Z" within the size designation) is the maximum speed specified by the tyre manufacturer (see paragraph 4.1.15.) if a 2.0 m diameter test drum is used, or 10 km/h less if a 1.7 m diameter test drum is used."

Paragraph 2.6.1., amend to read:

"2.6.1.		Twenty minutes to build up from zero to the initial test speed as specified in paragraph 2.5.2.1."

Paragraph 2.6.3., amend to read:

"2.6.3. 		Ten minutes to build up to the maximum test speed as specified in paragraph 2.5.6.1."

II.	Justification

1.	Paragraph 2.5.2: Correction of two typographical errors.

2.	Paragraph 2.5.2.1: The wording gives rise to uncertainty in the test procedure of the second load/speed performance test to be performed with tyres suitable for speeds above 240 km/h for tyres identified by means of letter code "V" within the size designation (or 270 km/h for tyres identified by means of letter code "Z" within the size designation). The different procedures result in different performance requirements for a tyre, depending on how the test is performed. Therefore, it is proposed to adapt the wording in such a way that the test procedure is clear.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.