|  |  | INF.8 |
| --- | --- | --- |

Economic Commission for Europe

Inland Transport Committee

Working Party on the Transport of Dangerous Goods

Ninety-seventh session											20 October 2014

Geneva, 3 (P.M.) –7 November 2014

Item 6 (a) of the provisional agenda

Proposals for amendments to annexes A and B of ADR:

Construction and approval of vehicles

Guideline concerning the certificate of approval according to 9.1.3 of ADR – Revision of the English version of ECE/TRANS/WP.15/2014/18

Transmitted by the Government of France

1.	During the process of translation of ECE/TRANS/WP.15/2014/18 from French into English some existing parts of the Guideline concerning the certificate of approval were re-translated when no amendment was foreseen for these parts.

2.	Therefore, the Government of France proposes in annex a revision of the English version of the proposed text based on existing text in ADR.

ANNEX

Guideline concerning the certificate of approval according to 9.1.3 of ADR

The numbered boxes of the form for the certificate of approval for vehicles carrying dangerous goods for which the model is shown in 9.1.3.5 should be filled in as follows:

1.  Certificate No.

A number to be entered by the issuing service.

2.  Vehicle manufacturer

To be taken from the registration document(s).

3.  Vehicle Identification No.

To be taken from the registration document(s) and be checked on the vehicle(s).

4.  Registration number

To be taken from the registration document(s).  If the vehicle is still not registered at the date of issuing the certificate of approval this box should stay empty for the time being until the vehicle is registered.

Or another procedure prescribed by national law.

5.  Name and business address of carrier, operator or owner

6.  Description of vehicle

According to footnote 1 of the certificate, the descriptions shall follow the definitions for power-driven vehicles and for trailers of categories N and O as defined in paragraph 2 of the Consolidated Resolution on the Construction of Vehicles (R.E.3) or in Directive 2007/46/EC.

Description of motor vehicles according to R.E.3

| Maximum mass (Mm) | Power-driven vehicles of category N | Véhicule à moteur de catégorie N | Kraftfahrzeuge der Klasse N |
| --- | --- | --- | --- |
| Mm < 3.5t | Category N1 | Catégorie N1 | Klasse N1 |
| 3.5t < Mm < 12t | Category N2 | Catégorie N2 | Klasse N2 |
| Mm > 12t | Category N3 | Catégorie N3 | Klasse N3 |

Description of motor vehicles according to Directive 2007/46/EC

| Maximum mass (Mm) | Motor vehicles of category N | Véhicules à moteur de catégorie N | Kraftfahrzeuge der Klasse N |
| --- | --- | --- | --- |
| Mm < 3.5t | Lorry N1 | Camion N1 | Lastkraftwagen N1 |
| 3.5t < Mm < 12t | Lorry N2 | Camion N2 | Lastkraftwagen N2 |
| Mm > 12t | Lorry N3 | Camion N3 | Lastkraftwagen N3 |
| Mm < 3.5t | Tractor N1 | Tracteur N1 | Zugmaschine N1 |
| 3.5t < Mm < 12t | Tractor N2 | Tracteur N2 | Zugmaschine N2 |
| Mm > 12t | Tractor N3 | Tracteur N3 | Zugmaschine N3 |
| Mm < 3.5t | Tractor for semi-trailer N1 | Tracteur de semi-remorque N1 | Sattelzugmaschine N1 |
| 3.5t < Mm < 12t | Tractor for semi-trailer N2 | Tracteur de semi-remorque N2 | Sattelzugmaschine N2 |
| Mm > 12t | Tractor for semi-trailer N3 | Tracteur de semi-remorque N3 | Sattelzugmaschine N3 |

Description of trailers

| Maximum Mass | Trailers | Remorques | Anhänger |
| --- | --- | --- | --- |
| Mm < 0.75t | Drawbar trailer O1/ Full trailer O1* | Remorque à timon d'attelage O1 | Deichselanhänger O1 |
| 0.75t < Mm < 3.5t | Drawbar trailer O2/ Full trailer O2* | Remorque à timon d'attelage O2 | Deichselanhänger O2 |
| 3.5t < Mm < 10t | Drawbar trailer O3/ Full trailer O3* | Remorque à timon d'attelage O3 | Deichselanhänger O3 |
| Mm > 10t | Drawbar trailer O4/ Full trailer O4* | Remorque à timon d'attelage O4 | Deichselanhänger O4 |
| Mm < 0.75t | Semi-trailer O1 | Semi-remorque O1 | Sattelanhänger O1 |
| 0.75t < Mm < 3.5t | Semi-trailer O2 | Semi-remorque O2 | Sattelanhänger O2 |
| 3.5t < Mm < 10t | Semi-trailer O3 | Semi-remorque O3 | Sattelanhänger O3 |
| Mm > 10t | Semi-trailer O4 | Semi-remorque O4 | Sattelanhänger O4 |
| Mm < 0.75t | Centre-axle trailer O1 | Remorque à essieu central O1 | Zentralachsanhänger O1 |
| 0.75t < Mm < 3.5t | Centre-axle trailer O2 | Remorque à essieu central O2 | Zentralachsanhänger O2 |
| 3.5t < Mm < 10t | Centre-axle trailer O3 | Remorque à essieu central O3 | Zentralachsanhänger O3 |
| Mm > 10t | Centre-axle trailer O4 | Remorque à essieu central O4 | Zentralachsanhänger O4 |

*  Full trailer is the wording of R.E.3.

According to 9.1.3.3 the certificate of approval for a vacuum-operated waste tank-vehicle shall bear in addition the following remark:  “vacuum-operated waste tank-vehicle”.

7.  Vehicle designation(s) according to 9.1.1.2 of ADR

To prevent unauthorized changes to the certificate all the designations which are not appropriate should be struck out.

More than one vehicle designation may be appropriate.  For example a vehicle which complies with the requirements for FL vehicles automatically fulfils the AT requirements.  Both designations are mentioned in the certificate.

The data in No. 7 in combination with the entries in No. 10 set the standard for the goods which may be carried by a vehicle.

8.  Endurance braking system

“Not applicable” should be marked on the certificates of vehicles for which the provisions for endurance braking systems are not applicable because of their low maximum mass or the low mass of the trailer in accordance with Note (b) in the table of 9.2.1.

In other cases the second line of No. 8 shall be marked and the appropriate value shall be entered.  In some countries the registration/in service maximum permissible mass exceeds 44 tonnes, but according to 2.3.1.5 of ECE Regulation No. 13, Annex 5, the value of 44t is considered to be sufficient even if the total maximum mass of the combination is higher than 44t.  (See footnote 4 of the certificate.)

Example:  The maximum mass of a combination is 50 tonnes (according to national law).  The effectiveness of the endurance braking system is sufficient for a maximum permissible mass of 44t.  In accordance with 2.3.1.5 of ECE Regulation No. 13, Annex 5, the combination may be operated at 50t.

9.  Description of the fixed tank(s)/battery-vehicle

To be taken from the type approval of the tank, the last inspection certificate of the tank or the tank plate.

10.	Dangerous goods authorized for carriage

For vehicles other than EX/II, EX/III vehicles or vehicles with fixed tank or battery-vehicles no entry is required in No. 10.  These vehicles (e.g. tractors for semi-trailer) may be used for the carriage of the goods according to the vehicle designations under No. 7.

10.1	In accordance with 9.3.7.3 for the electrical equipment in the load compartments of EX/II and EX/III vehicles, compliance with IP65 is required if the vehicle is intended for the carriage of explosives of compatibility group J.  For other explosives the electrical equipment in the load compartment shall comply with IP 54.

10.2	For tank-vehicles and battery-vehicles one of two possibilities shall be chosen by marking the appropriate:

- Either a reference to the tank-code in No. 9.5 and any special provision TC and TE in No. 9.6; or

- The list of substances by Class, UN number and, if necessary, packing group and proper shipping name.

11.	Remarks

Space for remarks (for example, the date of the next required inspection of the tank).

12.	Valid until

Enter the day of expiry as well as the place and date of issue.  The certificate shall be stamped and signed by the issuing service.

13.	Extensions of validity

As for No. 12 above.