|  | United Nations | ECE/TRANS/WP.11/2014/14 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 9 July 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

Working Party on the Transport of Perishable Foodstuffs

Seventieth session

Geneva, 7-10 October 2014

Item 5 (a) and 6 of the provisional agenda

Proposals of amendments to the ATP: pending proposals

ATP Handbook

Interpretation of external surface area measurement for panel vans

Transmitted by the United Kingdom

I.	Introduction

1.	This topic was originally raised at the 66th session of WP.11 and lead to some misunderstandings over the issues experienced. The problem leads to some van structures receiving lower K values than they otherwise might be entitled to.

2.	The existing text of the ATP is not practical for the measurement of the external surface area of insulated vans for all shapes and sizes of vehicle. The smooth contours of a van body make it difficult to establish the most accurate point from which to take measurements. For example, the width of the floor is often different from the ceiling and the front can be narrower than the rear where a side door is installed.

3.	Measurement of the external surfaces of a van cannot account for unfilled voids within the structure. Panel voids might also include steel structures that bridge the external heat to the outer edge of the insulated wall panels.

4.	The point is illustrated in the diagram below. The grey shading represents the main insulated panel and the black voids represent varying thicknesses of foam infill. The question is whether the external surface should be considered as the outer edge of the grey part or whether it should include the voids and be loosely considered as the external metal surface.

II.	Proposed amendment

5.	The following text should be added to annex 1, appendix 2, paragraph 1.2 as follows:

"For calculating the mean surface area of the body of a panel van, the competent authorities' appointed experts shall select from one or a combination of the following three methods.

Method A.	The manufacturer shall provide drawings and calculations of the inside and outside surfaces.

The surface areas Se and Si are determined taking into consideration the projected surface areas of specific design features of the irregularities of its surface such as curves, corrugations, wheel boxes, etc.

Method B.	The manufacturer shall provide drawings and the competent authority shall use the calculations according to the schemes and formulae in the ATP Handbook (using either figures 1, 2 or 3 along with figures 4 and 5).

Si = (((WI x LI) + (WI x LI) + (Wi x Wi)) x 2)

Se = (((WE x LE) + (WE x LE) + (We x We)) x 2)

Where:

WI is the Y axis of the internal surface area

LI is the X axis of the internal surface area

Wi is the Z axis of the internal surface area

WE is the Y axis of the external surface area

LE is the X axis of the external surface area

We is the Y axis of the external surface area

Method C.	If neither of the above is acceptable to the experts, the internal surface shall be measured according to the figures and formulae in method B.

The K value shall then be calculated based on the internal surface area, taking the insulation thickness as nil. From this K value, the average insulation thickness is calculated from the assumption that λ for the insulation has a value of 0.025 W/m·K.

d = Si x ΔT x λ / W

Once the thickness of the insulation has been estimated, the external surface area is calculated and the mean surface area is determined. The final K value is derived from successive iteration.

An example of the calculation methods shall be given in the ATP Handbook."

III.	Test report

6.	To reflect the panel van dimensions in the test report, the maximum internal length and height along with the top and bottom width shall be included in Model No 1 A of the ATP.

7.	The external dimensions shall be the internal dimensions with the insulation thickness added.

IV.	ATP Handbook

8.	It is proposed to include the following drawings in the ATP Handbook with examples.

# Figure 1

# Figure 2

# Figure 3

# Figure 4

# Figure 5

V.	Examples

9.	Using a van recently tested in the United Kingdom, the three methods are demonstrated using figures 1 to 5 above.

10.	The manufacturer in this case was able to supply the drawing of the insulated panel van but was unable to provide calculations of the internal and external surface areas.

VI.	Using figures 1, 4 and 5

VII.	Using figures 2, 4 and 5

VIII.	Using figures 3, 4 and 5

IX.	Discussion

11.	Method A is shown to give a flattering figure due to the manufacturer’s overestimation of the external surface area. Methods B and C give very close K values. A spreadsheet will be submitted in an informal document that gives the method for calculation of the mean insulation surface area from the internal surface area.

12.	Below is a summary of the results:

# Results excluding wheel arches

# Results including wheel arches

X.	Impact

13.	There may be a financial impact for industry, in that vans overall become slightly more expensive as the less expensive inferior versions are eliminated. As can be seen the iteration method is slightly harsher on the manufacturer than when the precise drawing are available.

14.	Improved K values generally reduce carbon emissions. However, in some instances the higher measured K value might result in the necessity for a larger cooling unit, which would increase carbon emissions.

15.	The proposal would ensure that measurements made on panel vans are consistent between different experts and testing stations, avoiding unrealistic test results. This would benefit manufacturers / body assemblers who would be reassured that the K value is not dependent on which test station or which expert conducts the test.