Proposal to amend UN regulation No. 48 concerning the activation of side marker lamps for turning heavy goods vehicles

Note:	The text reproduced below was prepared by the expert from Germany. The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

- Proposal

Paragraph 6.5.7., amend to read:

"6.5.7.	Electrical connections

Direction-indicator lamps shall switch on independently of the other lamps. All direction-indicator lamps on one side of a vehicle shall be switched on and off by means of one control and shall flash in phase.

On M1 and N1 vehicles less than 6 m in length, with an arrangement complying with paragraph 6.5.5.2. above, the amber side-marker lamps, when mounted, shall also flash at the same frequency (in phase) with the direction-indicator lamps.

The side marker lamps mounted on N2, N3, O3 and O4 vehicles shall flash at the same frequency (in phase) with the direction-indicator lamps.“

Paragraph 6.18.7., amend to read:

"6.18.7.	Electrical connections

On M1 and N1 category vehicles less than 6 m in length amber side-marker lamps may be wired to flash, provided that this flashing is in phase and at the same frequency with the direction-indicator lamps at the same side of the vehicle.

On N2, N3, O3 and O4 category vehicles the compulsory side marker lamps shall flash, provided that this flashing is in phase and at the same frequency with the direction-indicator lamps at the same side of the vehicle.

For all other categories of vehicles: no individual specification.

II.	Justification

This proposal is introducing flashing side marker lamps for vehicles of categories N2, N3, O3 and O4. The objective of this provision is to improve the safety for vulnerable road users by informing them about the intention of a heavy goods vehicle making a turn.

It is a simple measure that will improve the safety level especially for cyclists and pedestrians located in the blind spot area of a heavy goods vehicle.