|  | United Nations | ECE/TRANS/WP.29/GRRF/2014/24 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 July 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-eighth session

Geneva, 16-19 September 2014

Item 7(d) of the provisional agenda

Tyres – Other business

Proposal for amendments to Regulation No. 106 (Tyres for agricultural vehicles)

Submitted by International Motorcycle Manufacturers Association*

The text reproduced below was prepared by the experts from the International Motorcycle Manufacturers Association (IMMA). This is a counter-proposal to European Tyre and Rim Technical Organisation (ETRTO) proposal to amend Regulation No. 75 and replaces documents ECE/TRANS/WP.29/GRRF/2013/22 and ECE/TRANS/WP.29/GRRF/2013/20 to consider the comments made at the last GRRF session in February 2014. Modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 1., amend to read:

"1.		Scope

This Regulation covers new pneumatic tyres designed primarily, but not only, for agricultural and forestry vehicles (power-driven vehicles in category T), agricultural machines (power-driven and trailed) and agricultural trailers, and identified by speed category symbols corresponding to speeds of 90 km/h (speed symbol G) and below for AT tyres and by speed category symbols corresponding to 65 km/h (speed symbol "D") and below for other tyres."

Add a new paragraph 2.42., to read:

"2.42.		"AT (all terrain) tyre" means a low pressure tyre designed primarily, but not only, for the equipment of agricultural or forestry all-terrain vehicles or side-by-side vehicles and their trailers.

Add a new paragraph 3.1.14., to read:

"3.1.14.	For all terrain (AT) tyres, the symbol identifying the reference inflation pressure as follows:

| Symbol | Reference Inflation Pressure (kPa) |
| --- | --- |
|  | 25 |
|  | 35 |
|  | 45 |

"

Add a new paragraph 4.1.16., to read:

"4.1.16.	For all terrain (AT) tyres, the symbol identifying the reference inflation pressure (see paragraph 3.1.14.) "

Annex 1,

Add a new paragraph 5.8., to read:

"5.8.		For all terrain (AT) tyres, the symbol (stars) identifying the reference inflation pressure."

Annex 5,

Table 7 (1 and 2 of 4), complete rows for the following tyre size designations, to be deleted:

AT21x7–10, AT22x9–8, AT23x7–10, AT23x8–11, AT24x8–11, AT24x9–11, AT24x10–11, AT25x8–12, AT25x11–9 and AT25x11–10.

Annex 5,

Add a new table 8, to read:

"Table 8

All terrain tyres

| Tyre size 1 | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT16x6-8 | 5 | 154 | 394 | 406 | 418 | 168 |
| AT16x7-7 | 5.5 | 177 | 392 | 406 | 420 | 193 |
| AT16x8-7 | 6.5 | 204 | 392 | 406 | 420 | 222 |
| AT18x7-7 | 5.5 | 177 | 440 | 457 | 474 | 193 |
| AT18x7-8 | 5.5 | 177 | 442 | 457 | 472 | 193 |
| AT18x8-7 | 6.5 | 204 | 440 | 457 | 474 | 222 |
| AT18x9-8 | 7 | 227 | 442 | 457 | 472 | 247 |
| AT18x9.5-8 | 7.5 | 240 | 442 | 457 | 472 | 262 |
| AT18x10-10 | 8 | 254 | 445 | 457 | 469 | 277 |
| AT18x10-8 | 8 | 254 | 442 | 457 | 472 | 277 |
| AT18x11-10 | 9 | 281 | 445 | 457 | 469 | 306 |
| AT18x11-8 | 9 | 281 | 442 | 457 | 472 | 306 |
| AT18x11-9 | 9 | 281 | 443 | 457 | 471 | 306 |
| AT19x6-10 | 5 | 154 | 469 | 483 | 497 | 168 |
| AT19x7-8 | 5.5 | 177 | 466 | 483 | 500 | 193 |
| AT19x8-8 | 6.5 | 204 | 466 | 483 | 500 | 222 |
| AT19x9-8 | 7 | 227 | 466 | 483 | 500 | 247 |
| AT19x10-9 | 8 | 254 | 468 | 483 | 498 | 277 |
| AT20x7-10 | 5.5 | 177 | 493 | 508 | 523 | 193 |
| AT20x7-8 | 5.5 | 177 | 490 | 508 | 526 | 193 |
| AT20x7-9 | 5.5 | 177 | 491 | 508 | 525 | 193 |
| AT20x8-10 | 6.5 | 204 | 493 | 508 | 523 | 222 |
| AT20x10-10 | 8 | 254 | 493 | 508 | 523 | 277 |
| AT20x10-8 | 8 | 254 | 490 | 508 | 526 | 277 |
| AT20x10-9 | 8 | 254 | 491 | 508 | 525 | 277 |
| AT20x11-10 | 9 | 281 | 493 | 508 | 523 | 306 |
| AT20x11-8 | 9 | 281 | 490 | 508 | 526 | 306 |
| AT20x11-9 | 9 | 281 | 491 | 508 | 525 | 306 |
| AT21x7-10 | 5.5 | 177 | 516 | 533 | 550 | 193 |
| AT21x8-9 | 6.5 | 204 | 515 | 533 | 551 | 222 |
| AT21x10-10 | 8 | 254 | 516 | 533 | 550 | 277 |
| AT21x10-8 | 8 | 254 | 513 | 533 | 553 | 277 |
| AT21x11-8 | 9 | 281 | 513 | 533 | 553 | 306 |
| AT21x11-9 | 9 | 281 | 515 | 533 | 551 | 306 |
| AT21x12-8 | 9.5 | 304 | 513 | 533 | 553 | 331 |
| AT22x7-10 | 5.5 | 177 | 541 | 559 | 577 | 193 |
| AT22x7-11 | 5.5 | 177 | 542 | 559 | 576 | 193 |
| AT22x7-12 | 5.5 | 177 | 544 | 559 | 574 | 193 |
| AT22x7-9 | 5.5 | 177 | 539 | 559 | 579 | 193 |
| AT22x8-10 | 6.5 | 204 | 541 | 559 | 577 | 222 |
| AT22x9-10 | 7 | 227 | 541 | 559 | 577 | 247 |
| AT22x9-11 | 7 | 227 | 542 | 559 | 576 | 247 |
| AT22x9-8 | 7 | 227 | 538 | 559 | 580 | 247 |
| AT22x10-10 | 8 | 254 | 541 | 559 | 577 | 277 |
| AT22x10-8 | 8 | 254 | 538 | 559 | 580 | 277 |
| AT22x10-9 | 8 | 254 | 539 | 559 | 579 | 277 |
| AT22x11-10 | 9 | 281 | 541 | 559 | 577 | 306 |
| AT22x11-8 | 9 | 281 | 538 | 559 | 580 | 306 |
| AT22x11-9 | 9 | 281 | 539 | 559 | 579 | 306 |
| AT22x12-8 | 9,5 | 304 | 538 | 559 | 580 | 331 |
| AT22.5x10-8 | 8 | 254 | 550 | 572 | 594 | 277 |
| AT23x7-10 | 5.5 | 177 | 564 | 584 | 604 | 193 |
| AT23x8-10 | 6.5 | 204 | 564 | 584 | 604 | 222 |
| AT23x8-11 | 6.5 | 204 | 566 | 584 | 602 | 222 |
| AT23x8-12 | 6.5 | 204 | 567 | 584 | 601 | 222 |
| AT23x10-10 | 8 | 254 | 564 | 584 | 604 | 277 |
| AT23x10-12 | 8 | 254 | 567 | 584 | 601 | 277 |
| AT23x10-8 | 8 | 254 | 561 | 584 | 607 | 277 |
| AT23x11-9 | 9 | 281 | 563 | 584 | 605 | 306 |
| AT23x12-9 | 9,5 | 304 | 563 | 584 | 605 | 331 |
| AT24x8-11 | 6.5 | 204 | 590 | 610 | 630 | 222 |
| AT24x8-12 | 6.5 | 204 | 592 | 610 | 628 | 222 |
| AT24x9-11 | 7 | 227 | 590 | 610 | 630 | 247 |
| AT24x9-12 | 7 | 227 | 592 | 610 | 628 | 247 |
| AT24x10-11 | 8 | 254 | 590 | 610 | 630 | 277 |
| AT24x11-10 | 9 | 281 | 589 | 610 | 631 | 306 |
| AT24x11.5-10 | 9 | 290 | 589 | 610 | 631 | 316 |
| AT24x12-10 | 9,5 | 304 | 589 | 610 | 631 | 331 |
| AT24x13-9 | 10.5 | 330 | 587 | 610 | 633 | 360 |
| AT25x8-10 | 6.5 | 204 | 612 | 635 | 658 | 222 |
| AT25x8-12 | 6.5 | 204 | 615 | 635 | 655 | 222 |
| AT25x8-13 | 6.5 | 204 | 617 | 635 | 653 | 222 |
| AT25x10-10 | 8 | 254 | 612 | 635 | 658 | 277 |
| AT25x10-12 | 8 | 254 | 615 | 635 | 655 | 277 |
| AT25x11-10 | 9 | 281 | 612 | 635 | 658 | 306 |
| AT25x11-12 | 9 | 281 | 615 | 635 | 655 | 306 |
| AT25x11-9 | 9 | 281 | 611 | 635 | 659 | 306 |
| AT25x11.5-9 | 9 | 290 | 611 | 635 | 659 | 316 |
| AT25x12-10 | 9,5 | 304 | 612 | 635 | 658 | 331 |
| AT25x12-9 | 9,5 | 304 | 611 | 635 | 659 | 331 |
| AT25x13-9 | 10.5 | 330 | 611 | 635 | 659 | 360 |
| AT26x9-12 | 7 | 227 | 639 | 660 | 681 | 247 |
| AT26x10-12 | 8 | 254 | 639 | 660 | 681 | 277 |
| AT26x10.5-12 | 8.5 | 268 | 639 | 660 | 681 | 292 |
| AT26x12-10 | 9.5 | 304 | 636 | 660 | 684 | 331 |
| AT26x12-12 | 9.5 | 304 | 639 | 660 | 681 | 331 |
| AT26x12-14 | 9.5 | 304 | 642 | 660 | 678 | 331 |
| AT27x9-12 | 7 | 227 | 663 | 686 | 709 | 247 |
| AT27x11-12 | 9 | 281 | 663 | 686 | 709 | 306 |
| AT27x12-10 | 9.5 | 304 | 660 | 686 | 712 | 331 |
| AT27x12-12 | 9.5 | 304 | 663 | 686 | 709 | 331 |

1	Tyres in Radial construction are identified by the letter "R" in place of "-" (e.g. AT16x6 R 8). In case of Tyre Size designations not included in this table, refer to any edition of one of the following International Tyre Standards for the relevant dimensional data:

(a)	The European Tyre and Rim Technical Organisation (ETRTO) 'Standards Manual';

(b)	The Tire and Rim Association Inc. (TRA) 'Year Book';

(c)	The Japan Automobile Tire Manufacturers Association (JATMA) 'Year Book';

(d)	The Scandinavian Tyre and Rim Organisation (STRO) 'Data Book'.

In that case the nominated International Tyre Standard, to which the tyre conforms, shall be specified in the application for approval.

"

Annex 7,

Add a new Part E for all terrain (AT) tyres with speed symbol "F", to read:

" Part E: AT tyres

Applicable to tyres classified with categories of use: agricultural or forestry all-terrain vehicles and side-by-side vehicles (see paragraph 2.42. of this Regulation)

# Variation of load carrying capacity (per cent) for tyres marked with speed category symbol F

| Speed (km/h) | Variation in Load Carrying Capacity (%) |
| --- | --- |
| 50 and below | + 12 |
| 60 | + 7 |
| 70 | + 3 |
| 80 | 0 |
| 90 | - 5 |

"

Annex 9,

Paragraph 1.1., amend to read:

"1.1.	This test procedure is applicable for new tyres marked with speed category symbol "D" and, in case of AT tyres with speed category symbol D, E, F, G."

Paragraph 3.2., amend to read:

"3.2.	Test drum speed: 20/km/h see table in paragraph 3.4. below"

Paragraph 3.3.1., amend to read:

"3.3.1.	The mass corresponding to load index marked on the tyre in case of tyres marked with speed symbol D."

Paragraph 3.4., amend to read:

"3.4.	Load/speed test programme

| Tyre speed category symbol | Test step | Percentage of the test load | Duration (hours) |
| --- | --- | --- | --- |
| D | 1 | 66 per cent | 7 |
|  | 2 | 84 per cent | 16 |
|  | 3 | 101 per cent | 24 |

| Load index | Tyre speed category symbol | Test-drum speed |  | Load placed on the wheel as a percentage of the load corresponding to the load index |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Radial-ply min-1 | Diagonal (bias-ply) min-1 | 7 h. | 16 h. | 24 h. |
| 122 or more | D E F G | 62.5 75 100 125 | 62.5 75 100 100 | 66 % | 84 % | 101 % |
| 121 or less | D E F G | 62.5 75 100 125 | 62.5 75 100 125 |  |  |  |

"

II.	Justification

1.	This proposal enables the type approval of tyres designed to equip all-terrain vehicles and side-by-side vehicles covered by the new EU Regulation No. 167/2013 of the European Parliament and of the Council of 5 February 2013 on the approval and market surveillance of agricultural and forestry vehicles. It also enables the type approval of tyres designed to equip those quadricycles categorised as all-terrain quads and side-by-side buggies and covered by the new EU Regulation No. 168/2013 of the European Parliament and of the Council of 15 January 2013 on the approval and market surveillance of two- or three-wheel vehicles and quadricycles (L category vehicles).

2.	In order to cover the speeds between 65 and 90 km/h, relevant requirements for the load/speed tests have been copied from Regulation No. 54.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

?	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.2, para. 2. - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html