|  | United Nations | ECE/TRANS/WP.29/2015/2 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 22 December 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

165th session

Geneva, 10-13 March 2015

Item 4.6.1 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRB

Proposal for Supplement 2 to the 04 series of amendments to Regulation No. 41 (Noise emissions of motorcycles)

Submitted by the Working Party on Noise*

The text reproduced below was adopted by the Working Party on Noise (GRB) at its sixtieth session (ECE/TRANS/WP.29/GRB/58, para. 4). It is based on Annex II to ECE/TRANS/WP.29/GRB/58. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration.

Paragraph 2.13., the table, amend to read:

| Symbol | Units | Explanation | Reference |
| --- | --- | --- | --- |
| AA' | – | virtual line on the test track | Annex 4 – Figure 1 |
| awot | m/s2 | calculated acceleration | Annex 3 – 1.4.2. |
| awot,ref | m/s2 | prescribed reference acceleration | Annex 3 – 1.3.3.3.1.2. |
| aurban | m/s2 | prescribed target acceleration | Annex 3 – 1.3.3.3.1.2. |
| BB' | – | virtual line on the test track | Annex 4 – Figure 1 |
| CC' | – | virtual line on the test track | Annex 4 – Figure 1 |
| k | – | gear weighting factor | Annex 3 – 1.4.3. |
| kp | – | partial power factor | Annex 3 – 1.4.4. |
| L | dB(A) | sound pressure level | Annex 3 – 1.4.6. |
| L wot(i) | dB(A) | L at wot condition |  |
| lPA | m | pre-acceleration length | Annex 3 – 1.3.3.1.1. |
| mkerb | kg | kerb mass of the vehicle | 2.6. |
| mt | kg | test mass of the vehicle | Annex 3 – 1.3.2.2. |
| n | min-1 | measured engine speed |  |
| nPP’ | min-1 | engine speed at PP’ | Annex 7 – 2.6. |
| nidle | min-1 | engine speed at idle | – |
| nwot(i) | min-1 | nPP' measured at Lwot(i) detection | Annex 7 – 2.6. |
| PP' | – | virtual line on the test track | Annex 4 – Figure 1 |
| PMR | – | power-to-mass ratio index | 2.9. |
| Pn | kW | rated maximum net power | 2.7. |
| S | min-1 | rated engine speed | 2.8. |
| v | km/h | measured vehicle speed | – |
| vmax | km/h | maximum speed | 2.10. |
| vtest | km/h | prescribed test speed | Annex 3 – 1.3.3.1.1. |

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.