|  | United Nations | ECE/TRANS/WP.29/2015/8 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 23 December 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

165th session

Geneva, 10-13 March 2015

Item 4.7.3 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRRF

Proposal for Supplement 15 to Regulation No. 75 (Tyres for motorcycles / mopeds)

Submitted by the Working Party on Brakes and Running Gear*

The text reproduced below was adopted by the Working Party on Brakes and Running Gear (GRRF) at its seventy-eighth session (ECE/TRANS/WP.29/GRRF/78, para. 36). It is based on Annex 5 to the report. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Executive Committee AC.1 for consideration.

Title of the Regulation, amend to read:

"Regulation No. 75

Uniform provisions concerning the approval of pneumatic tyres for L-category vehicles."

Paragraph 1., amend to read (including the insertion new footnotes 1 and 2):

"1.	Scope

This Regulation applies to new pneumatic tyres for vehicles of category L.1, 2

However, it does not apply to tyre types designed exclusively for the "off-road" use, which are marked "NHS" (Not for Highway Service) and to tyre types designed exclusively for competitions.

1 	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.3, para. 2. - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html

2 	This Regulation defines requirements for tyres as a component. It does not limit their installation on any categories of vehicles."

Paragraph 2.1.3., amend to read:

"2.1.3.	… moped, all terrain (AT)."

Add a new paragraph 2.16.4., to read:

"2.16.4.	The prefix "AT" for all terrain (AT) tyres (e.g. AT 18x11-8)"

Add a new paragraph 2.34., to read:

"2.34.		AT (all terrain) tyre:  means a low pressure tyre designed primarily for the equipment of categories L6 and L7."

Add a new paragraph 3.1.15., to read:

"3.1.15.	For all terrain (AT) tyres, the symbol identifying the reference inflation pressure as follows:

| Symbol | Reference Inflation Pressure (kPa) |
| --- | --- |
|  | 25 |
|  | 35 |
|  | 45 |

"

Add a new paragraph 4.1.16., to read:

"4.1.16.	For all terrain (AT) tyres, the symbol identifying the reference inflation pressure (see paragraph 3.1.15.)."

Annex 1, paragraph 5.2., amend to read:

"5.2.		…/moped/all terrain (AT) 2 "

Annex 1, insert a new paragraph 5.6., to read:

"5.6.		For all terrain (AT) tyres, the symbol (stars) identifying the reference inflation pressure."

Annex 5, add a new Table 8, to read:

"Table 8: Tyres for all terrain vehicles (AT)

# Table 8 (1 of 3)

# Tyres for all terrain vehicles (AT)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT16x6-8 | 5 | 154 | 394 | 406 | 418 | 168 |
| AT16x7-7 | 5.5 | 177 | 392 | 406 | 420 | 193 |
| AT16x8-7 | 6.5 | 204 | 392 | 406 | 420 | 222 |
| AT18x7-7 | 5.5 | 177 | 440 | 457 | 474 | 193 |
| AT18x7-8 | 5.5 | 177 | 442 | 457 | 472 | 193 |
| AT18x8-7 | 6.5 | 204 | 440 | 457 | 474 | 222 |
| AT18x9-8 | 7 | 227 | 442 | 457 | 472 | 247 |
| AT18x9.5-8 | 7.5 | 240 | 442 | 457 | 472 | 262 |
| AT18x10-10 | 8 | 254 | 445 | 457 | 469 | 277 |
| AT18x10-8 | 8 | 254 | 442 | 457 | 472 | 277 |
| AT18x11-10 | 9 | 281 | 445 | 457 | 469 | 306 |
| AT18x11-8 | 9 | 281 | 442 | 457 | 472 | 306 |
| AT18x11-9 | 9 | 281 | 443 | 457 | 471 | 306 |
| AT19x6-10 | 5 | 154 | 469 | 483 | 497 | 168 |
| AT19x7-8 | 5.5 | 177 | 466 | 483 | 500 | 193 |
| AT19x8-8 | 6.5 | 204 | 466 | 483 | 500 | 222 |
| AT19x9-8 | 7 | 227 | 466 | 483 | 500 | 247 |
| AT19x10-9 | 8 | 254 | 468 | 483 | 498 | 277 |
| AT20x7-10 | 5.5 | 177 | 493 | 508 | 523 | 193 |
| AT20x7-8 | 5.5 | 177 | 490 | 508 | 526 | 193 |
| AT20x7-9 | 5.5 | 177 | 491 | 508 | 525 | 193 |
| AT20x8-10 | 6.5 | 204 | 493 | 508 | 523 | 222 |
| AT20x10-10 | 8 | 254 | 493 | 508 | 523 | 277 |
| AT20x10-8 | 8 | 254 | 490 | 508 | 526 | 277 |
| AT20x10-9 | 8 | 254 | 491 | 508 | 525 | 277 |
| AT20x11-10 | 9 | 281 | 493 | 508 | 523 | 306 |
| AT20x11-8 | 9 | 281 | 490 | 508 | 526 | 306 |
| AT20x11-9 | 9 | 281 | 491 | 508 | 525 | 306 |
| AT21x7-10 | 5.5 | 177 | 516 | 533 | 550 | 193 |
| AT21x8-9 | 6.5 | 204 | 515 | 533 | 551 | 222 |
| AT21x10-10 | 8 | 254 | 516 | 533 | 550 | 277 |
| AT21x10-8 | 8 | 254 | 513 | 533 | 553 | 277 |
| AT21x11-8 | 9 | 281 | 513 | 533 | 553 | 306 |
| AT21x11-9 | 9 | 281 | 515 | 533 | 551 | 306 |
| AT21x12-8 | 9.5 | 304 | 513 | 533 | 553 | 331 |
| AT22x7-10 | 5.5 | 177 | 541 | 559 | 577 | 193 |
| AT22x7-11 | 5.5 | 177 | 542 | 559 | 576 | 193 |
| AT22x7-12 | 5.5 | 177 | 544 | 559 | 574 | 193 |
| AT22x7-9 | 5.5 | 177 | 539 | 559 | 579 | 193 |
| AT22x8-10 | 6.5 | 204 | 541 | 559 | 577 | 222 |
| AT22x9-10 | 7 | 227 | 541 | 559 | 577 | 247 |

# Table 8 (2 of 3)

# Tyres for all terrain vehicles (AT tyres)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT22x9-11 | 7 | 227 | 542 | 559 | 576 | 247 |
| AT22x9-8 | 7 | 227 | 538 | 559 | 580 | 247 |
| AT22x10-10 | 8 | 254 | 541 | 559 | 577 | 277 |
| AT22x10-8 | 8 | 254 | 538 | 559 | 580 | 277 |
| AT22x10-9 | 8 | 254 | 539 | 559 | 579 | 277 |
| AT22x11-10 | 9 | 281 | 541 | 559 | 577 | 306 |
| AT22x11-8 | 9 | 281 | 538 | 559 | 580 | 306 |
| AT22x11-9 | 9 | 281 | 539 | 559 | 579 | 306 |
| AT22x12-8 | 9,5 | 304 | 538 | 559 | 580 | 331 |
| AT22.5x10-8 | 8 | 254 | 550 | 572 | 594 | 277 |
| AT23x7-10 | 5.5 | 177 | 564 | 584 | 604 | 193 |
| AT23x8-10 | 6.5 | 204 | 564 | 584 | 604 | 222 |
| AT23x8-11 | 6.5 | 204 | 566 | 584 | 602 | 222 |
| AT23x8-12 | 6.5 | 204 | 567 | 584 | 601 | 222 |
| AT23x10-10 | 8 | 254 | 564 | 584 | 604 | 277 |
| AT23x10-12 | 8 | 254 | 567 | 584 | 601 | 277 |
| AT23x10-8 | 8 | 254 | 561 | 584 | 607 | 277 |
| AT23x11-9 | 9 | 281 | 563 | 584 | 605 | 306 |
| AT23x12-9 | 9,5 | 304 | 563 | 584 | 605 | 331 |
| AT24x8-11 | 6.5 | 204 | 590 | 610 | 630 | 222 |
| AT24x8-12 | 6.5 | 204 | 592 | 610 | 628 | 222 |
| AT24x9-11 | 7 | 227 | 590 | 610 | 630 | 247 |
| AT24x9-12 | 7 | 227 | 592 | 610 | 628 | 247 |
| AT24x10-11 | 8 | 254 | 590 | 610 | 630 | 277 |
| AT24x11-10 | 9 | 281 | 589 | 610 | 631 | 306 |
| AT24x11.5-10 | 9 | 290 | 589 | 610 | 631 | 316 |
| AT24x12-10 | 9,5 | 304 | 589 | 610 | 631 | 331 |
| AT24x13-9 | 10.5 | 330 | 587 | 610 | 633 | 360 |
| AT25x8-10 | 6.5 | 204 | 612 | 635 | 658 | 222 |
| AT25x8-12 | 6.5 | 204 | 615 | 635 | 655 | 222 |
| AT25x8-13 | 6.5 | 204 | 617 | 635 | 653 | 222 |
| AT25x10-10 | 8 | 254 | 612 | 635 | 658 | 277 |
| AT25x10-12 | 8 | 254 | 615 | 635 | 655 | 277 |
| AT25x11-10 | 9 | 281 | 612 | 635 | 658 | 306 |
| AT25x11-12 | 9 | 281 | 615 | 635 | 655 | 306 |
| AT25x11-9 | 9 | 281 | 611 | 635 | 659 | 306 |
| AT25x11.5-9 | 9 | 290 | 611 | 635 | 659 | 316 |
| AT25x12-10 | 9,5 | 304 | 612 | 635 | 658 | 331 |
| AT25x12-9 | 9,5 | 304 | 611 | 635 | 659 | 331 |
| AT25x13-9 | 10.5 | 330 | 611 | 635 | 659 | 360 |

# Table 8 (3 of 3)

# Tyres for all terrain vehicles (AT tyres)

| Tyre size (1) | Measuring rim width code | Section width (mm) | Overall diameter (mm) |  |  | Maximum overall width (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Dmin | D | Dmax |  |
| AT26x9-12 | 7 | 227 | 639 | 660 | 681 | 247 |
| AT26x10-12 | 8 | 254 | 639 | 660 | 681 | 277 |
| AT26x10.5-12 | 8.5 | 268 | 639 | 660 | 681 | 292 |
| AT26x12-10 | 9.5 | 304 | 636 | 660 | 684 | 331 |
| AT26x12-12 | 9.5 | 304 | 639 | 660 | 681 | 331 |
| AT26x12-14 | 9.5 | 304 | 642 | 660 | 678 | 331 |
| AT27x9-12 | 7 | 227 | 663 | 686 | 709 | 247 |
| AT27x11-12 | 9 | 281 | 663 | 686 | 709 | 306 |
| AT27x12-10 | 9.5 | 304 | 660 | 686 | 712 | 331 |
| AT27x12-12 | 9.5 | 304 | 663 | 686 | 709 | 331 |

(1)	Tyres in Radial construction are identified by the letter "R" in place of "-" (e.g. AT16x6 R 8). In case of Tyre Size designations not included in this table, refer to any edition of one of the following International Tyre Standards for the relevant dimensional data:

(a)	The European Tyre and Rim Technical Organisation (ETRTO) 'Standards Manual';

(b)	The Tire and Rim Association Inc. (TRA) 'Year Book';

(c)	The Japan Automobile Tire Manufacturers Association (JATMA) 'Year Book'; or

(d)	The Scandinavian Tyre and Rim Organisation (STRO) 'Data Book'.

In that case the nominated International Tyre Standard, to which the tyre conforms, shall be specified in the application for approval."

Annex 6, paragraph 1., the table, add the following tyres to the table:

"

|  | Tyre version | Speed Category | Pressure (bar) | Pressure (kPa) |
| --- | --- | --- | --- | --- |
| All terrain (AT) |  | all | 0.25 | 25 |
|  |  | all | 0.35 | 35 |
|  |  | all | 0.45 | 45 |

"

Annex 7, paragraph 1.2., the table, add the following tyres to the table:

"

|  | Tyre version | Speed Category | Inflation pressure |  |
| --- | --- | --- | --- | --- |
|  |  |  | (bar) | (kPa) |
| All terrain (AT) |  | all | 0.25 | 25 |
|  |  | all | 0.35 | 35 |
|  |  | all | 0.45 | 45 |

"

Annex 8, add a new table, to read:

# "Table for all terrain tyres (AT) with speed symbol "F"

| Speed (km/h) | Variation in Load Carrying Capacity (%) |
| --- | --- |
| 50 and below | + 12 |
| 60 | + 7 |
| 70 | + 3 |
| 80 | 0 |
| 90 | - 5 |
| 100 | - 10 |
| 110 | - 15 |
| 120 | - 20 |
| 130 | - 25 |

"

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.