|  | United Nations | ECE/TRANS/WP.29/2015/16 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 26 December 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

165th session

Geneva, 10-13 March 2015

Item 4.9.3 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRE

Proposal for Supplement 8 to the 04 series of amendments to Regulation No. 19 (Front fog lamps)

Submitted by the Working Party on Lighting and Light-Signalling*

The text reproduced below was adopted by the Working Party on Lighting and Light-Signalling (GRE) at its seventy-second session (ECE/TRANS/WP.29/GRE/72, para. 39). It is based on ECE/TRANS/WP.29/GRE/2014/18, not amended. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration.

Insert a new paragraph 3.5.3., to read:

"3.5.3.	If the LED module(s) are non-replaceable, the markings for LED module(s) are not required."

Annex 1, item 10.3., amend to read:

"10.3.	LED module(s): yes/no2  and for each LED module a statement whether it is replaceable or not: yes/no2 "

Annex 5, paragraph 1.2.1.1., amend to read;

"1.2.1.1.	Test mixture

1.2.1.1.1.	For front fog lamps with the outside lens in glass:

The mixture of water and a polluting agent to be applied to the front fog lamp shall be composed of:

(a)	9 parts by weight of silica sand with a particle size of 0-100 µm,

(b)	1 part by weight of vegetal carbon dust produced from beech wood with a particle size of 0-100 µm,

(c)	0.2 part by weight of NaCMC4,

(d)	5 parts by weight of sodium chloride (pure at 99 per cent), and

(e)	An appropriate quantity of distilled water with a conductivity of S < l µS/m.

The mixture must not be more than 14 days old.

1.2.1.1.2.	For front fog lamp with outside lens in plastic material:

The mixture of water and polluting agent to be applied to the front fog lamp shall be composed of:

(a)	9 parts by weight of silica sand with a particle size of 0-100 µm,

(b)	1 part by weight of vegetal carbon dust produced from beech wood with a particle size of 0-100 µm,

(c)	0.2 part by weight of NaCMC4,

(d)	5 parts by weight of sodium chloride (pure at 99 per cent),

(e)	13 parts by weight of distilled water with a conductivity of S < 1 µS/m, and

(f)		2 ± 1 parts by weight of surface-actant. 5

The mixture must not be more than 14 days old."

Annex 12, paragraph 4.6., amend to read:

"4.6.	UV-radiation

The UV-radiation of a low-UV-type LED module or light-generator shall be such that:

……………

(For definitions of the other symbols see paragraph 4.5.1. above)

This value shall be calculated using intervals of one nanometre. The UV-radiation shall be weighted according to the values as indicated in the UV table below.

|  | S() |  |  | S() |  |  | S() |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 250 | 0.430 |  | 305 | 0.060 |  | 355 | 0.000 16 |
| 255 | 0.520 |  | 310 | 0.015 |  | 360 | 0.000 13 |
| 260 | 0.650 |  | 315 | 0.003 |  | 365 | 0.000 11 |
| 265 | 0.810 |  | 320 | 0.001 |  | 370 | 0.000 09 |
| 270 | 1.000 |  | 325 | 0.000 50 |  | 375 | 0.000 077 |
| 275 | 0.960 |  | 330 | 0.000 41 |  | 380 | 0.000 064 |
| 280 | 0.880 |  | 335 | 0.000 34 |  | 385 | 0.000 053 |
| 285 | 0.770 |  | 340 | 0.000 28 |  | 390 | 0.000 044 |
| 290 | 0.640 |  | 345 | 0.000 24 |  | 395 | 0.000 036 |
| 295 | 0.540 |  | 350 | 0.000 20 |  | 400 | 0.000 030 |
| 300 | 0.300 |  |  |  |  |  |  |

UV Table

Values according to …………………….. other values should be interpolated."

Annex 12, paragraph 4.7.2., amend to read:

"4.7.2.	Colour

The colour of the light emitted, measured after 1 minute and measured after photometric stability has been obtained, as described in paragraph 4.7.1.3. of this annex, shall be within the required colour boundaries in both instances."

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.