|  | United Nations | ECE/TRANS/WP.29/2015/25/Corr.2 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 16 January 2015 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

165th session

Geneva, 10-13 March 2015

Item 4.9.12 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRE

Proposal for Supplement 17 to the 01 series of amendments to Regulation No. 53 (Installation of lighting and light-signalling devices for L3 vehicles)

Note by the secretariat

Corrigendum

Page 5, paragraph 6.4.4., replace to read:

"6.4.4.		Geometric visibility

For category S1 device specified in Regulation No. 7 or stop lamp specified in Regulation No. 50

Horizontal angle:	45° to left and to right for a single lamp;

45° outwards and 10° inwards for each pair of lamps;

Vertical angle: 	15° above and below the horizontal.

However, where a lamp is mounted below 750 mm (measured according to the provisions of paragraph 5.7.), the downward angle of 15° may be reduced to 5°.

For category S3 device specified in Regulation No. 7

Horizontal angle:	10° to the left and to the right of the longitudinal axis of the vehicle.

Vertical angle:	10° above and 5° below the horizontal."