|  | United Nations | ECE/TRANS/WP.29/2015/59 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 10 April 2015 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

166th session

Geneva, 23-26 June 2015

Item 4.8.1 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRB

Proposal for Supplement 1 to the 07 series of amendments to Regulation No. 9 (Noise of three-wheeled vehicles)

Submitted by the Working Party on Noise*

The text reproduced below was adopted by the Working Party on Noise (GRB) at its sixty-first session (ECE/TRANS/WP.29/GRB/59, para. 17). It is based on Annex VII to ECE/TRANS/WP.29/GRB/59. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration.

Insert new paragraphs 11.7. and 11.8., to read:

"11.7. 	As from the official date of entry into force of Supplement 1 to the 07 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approval according to Supplement 1 to 07 series of amendments to the regulation.

11.8. 	As from 60 months after the date of entry into force of Supplement 1 to the 07 series of amendments to this Regulation, Contracting Parties applying this Regulation shall grant type approvals only if the vehicle type to be approved meets the requirements of this Regulation as amended by Supplement 1 to the 07 series of amendments to this Regulation."

Annex 1,

Item 19., to be deleted.

Items 20. to 29., renumber as 19. to 28., respectively.

Annex 3,

Paragraph 2.2., amend to read:

"2.2.	The test site shall consist of a central acceleration track surrounded by a substantially level test area. The test track shall be level; the track surface shall be dry and so designed that tyre noise remains low.

On the test site, free sound field conditions shall be maintained to within ±1 dB between the sound source placed in the middle of the acceleration section and the microphone. This condition shall be deemed to be met if there are no large sound-reflecting objects such as fences, rocks, bridges or buildings within 50 m of the centre of the acceleration section. The surface of the test track shall conform to the requirements of Annex 4 to this Regulation or to ISO10844:2014.

No obstacle likely to affect the sound field shall be close to the microphone and no one shall come between the microphone and the sound source. The observer taking the measurements shall take up position so as to avoid influencing the metre readings."

Annex 4,

After the title, insert the reference to a new footnote 1 and a new footnote 1, to read:

"1 	The specifications for the test site reproduced in this annex are valid until the end of the period indicated in paragraph 11.8."

Paragraph 1, the reference to footnote 1 and footnote 1, renumber as footnote 2.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.