|  | United Nations | ECE/TRANS/WP.29/2015/60 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 10 April 2015 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

166th session

Geneva, 23-26 June 2015

Item 4.8.2 of the provisional agenda

1958 Agreement – Consideration of draft amendments
to existing Regulations submitted by GRB

Proposal for Supplement 4 to the original series of amendments to Regulation No. 28 (Audible warning devices)

Submitted by the Working Party on Noise*

The text reproduced below was adopted by the Working Party on Noise (GRB) at its sixty-first session (ECE/TRANS/WP.29/GRB/59, para. 4). It is based on ECE/TRANS/WP.29/GRB/2014/4, as amended by Annex II to ECE/TRANS/WP.29/GRB/59. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration.

Paragraph 14.4., amend to read:

"14.4.		The A-weighted sound pressure level emitted by the device(s) fitted on the vehicle shall be measured at a distance of 7 m in front of the vehicle, the latter being placed on an open site , on ground as smooth as possible, and, in case of devices supplied with direct current, with its engine stopped. The test voltage as specified in paragraph 6.2.3. shall be supplied by:

(a)	The vehicle battery only; or

(b)	The vehicle battery with the vehicle engine warmed-up and at idle; or

(c)	With an external power source supply connected to the AWD terminal(s)."

Paragraph 14.8., subparagraph (b), replace "93 dB(A)" with "87 dB(A)".

Annex 2,

After item 5., insert a new item 6. to read:

"6. 	Power supply used: Vehicle battery only/Battery with vehicle engine at idle/External power supply2 	"

Items 6. (former) to 15., renumber to become items 7. to 16., respectively.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.