|  | United Nations | ECE/TRANS/WP.29/2015/66 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 10 April 2015 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

166th session

Geneva, 23-26 June 2015

Item 4.9.1 of the provisional agenda

1958 Agreement – Consideration of draft amendments

to existing Regulations submitted by GRRF

Proposal for Supplement 20 to Regulation No. 54 (Tyres for commercial vehicles and their trailers)

Submitted by the Working Party on Brakes and Running Gear*

The text reproduced below was adopted by the Working Party on Brakes and Running (GRRF) at its seventy-ninth session (ECE/TRANS/WP.29/GRRF/79, para. 31). It is based on ECE/TRANS/WP.29/GRRF/2015/5 and ECE/TRANS/WP.29/GRRF/2015/6, not amended. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee AC.1 for consideration at their June 2015 session.

Paragraph 3.1.13., amend to read:

"3.1.13.	The prefix "LT" before the tyre size designation, or the suffix "C" or "LT" after the rim diameter marking referred to in paragraph 2.17.1.3., and, if applicable, after the tyre to rim fitment configuration referred to in paragraph 2.17.1.4., or the suffix "LT" after the service description."

Annex 5, part II, table B, amend to read:

"Table B

- Tyres for light commercial vehicles (high flotation tyres)

- Diagonal and radial

| Tyre-size designation1 | Measuring rim width code | Nominal rim diameter d (mm) | Outer diameter D (mm)2 |  | Section width S (mm)3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Normal | Snow |  |
| 9-15LT | 8.00 | 381 | 744 | 755 | 254 |
| 10-15LT | 8.00 | 381 | 773 | 783 | 264 |
| 11-15LT | 8.00 | 381 | 777 | 788 | 279 |
| 24x7.50-13LT | 6.00 | 330 | 597 | 604 | 191 |
| 27x8.50-14LT | 7.00 | 356 | 674 | 680 | 218 |
| 28x8.50-15LT | 7.00 | 381 | 699 | 705 | 218 |
| 29x9.50-15LT | 7.50 | 381 | 724 | 731 | 240 |
| 30x9.50-15LT | 7.50 | 381 | 750 | 756 | 240 |
| 31x10.50-15LT | 8.50 | 381 | 775 | 781 | 268 |
| 31x11.50-15LT | 9.00 | 381 | 775 | 781 | 290 |
| 31x12.50R15LT | 10.00 | 381 | 775 | 781 | 318 |
| 31x13.50-15LT | 11.00 | 381 | 775 | 781 | 345 |
| 31x15.50-15LT | 12.00 | 381 | 775 | 781 | 390 |
| 32x11.50-15LT | 9.00 | 381 | 801 | 807 | 290 |
| 33x9.50 R15LT | 7.50 | 381 | 826 | 832 | 240 |
| 33x10.50R15LT | 8.50 | 381 | 826 | 832 | 268 |
| 33x10.50R17LT | 8.50 | 432 | 826 | 832 | 268 |
| 33x10.50R18LT | 8.50 | 457 | 826 | 832 | 268 |
| 33x11.50R18LT | 9.00 | 457 | 826 | 832 | 290 |
| 33x12.50-15LT | 10.00 | 381 | 826 | 832 | 318 |
| 33x12.50R17LT | 10.00 | 432 | 826 | 832 | 318 |
| 33x12.50R18LT | 10.00 | 457 | 826 | 832 | 318 |
| 33x12.50R20LT | 10.00 | 508 | 826 | 832 | 318 |
| 33x12.50R22LT | 10.00 | 559 | 826 | 832 | 318 |
| 33x13.50R15LT | 11.00 | 381 | 826 | 832 | 345 |
| 33x15.50R15LT | 12.00 | 381 | 826 | 832 | 390 |
| 34x10.50R17LT | 8.50 | 432 | 851 | 858 | 268 |
| 34x12.50R18LT | 10.00 | 457 | 851 | 858 | 318 |
| 35x12.50-15LT | 10.00 | 381 | 877 | 883 | 318 |
| 35x12.50R17LT | 10.00 | 432 | 877 | 883 | 318 |
| 35x12.50R18LT | 10.00 | 457 | 877 | 883 | 318 |
| 35x12.50R20LT | 10.00 | 508 | 877 | 883 | 318 |
| 35x12.50R22LT | 10.00 | 559 | 877 | 883 | 318 |
| 35x13.50R15LT | 11.00 | 381 | 877 | 883 | 345 |
| 35x13.50R18LT | 11.00 | 457 | 877 | 883 | 345 |
| 35x13.50R20LT | 11.00 | 508 | 877 | 883 | 345 |
| 35x14.50R15LT | 12.00 | 381 | 877 | 883 | 372 |
| 36x13.50R18LT | 11.00 | 457 | 902 | 908 | 345 |
| 36x14.50R15LT | 12.00 | 381 | 902 | 908 | 372 |
| 36x14.50R17LT | 12.00 | 432 | 902 | 908 | 372 |
| 36x14.50R18LT | 12.00 | 457 | 902 | 908 | 372 |
| 36x15.50R15LT | 12.00 | 381 | 902 | 908 | 390 |
| 37x12.50-15LT | 10.00 | 381 | 928 | 934 | 318 |
| 37x12.50 R17LT | 10.00 | 432 | 928 | 934 | 318 |
| 37x12.50R18LT | 10.00 | 457 | 928 | 934 | 318 |
| 37x12.50R20LT | 10.00 | 508 | 928 | 934 | 318 |
| 37x12.50R22LT | 10.00 | 559 | 928 | 934 | 318 |
| 37x13.50R15LT | 11.00 | 381 | 928 | 934 | 345 |
| 37x13.50R17LT | 11.00 | 432 | 928 | 934 | 345 |
| 37x13.50R18LT | 11.00 | 457 | 928 | 934 | 345 |
| 37x13.50R20LT | 11.00 | 508 | 928 | 934 | 345 |
| 37x13.50R22LT | 11.00 | 559 | 928 | 934 | 345 |
| 37x13.50R24LT | 11.00 | 610 | 928 | 934 | 345 |
| 37x14.50-15LT | 12.00 | 381 | 928 | 934 | 372 |
| 38x13.50R17LT | 11.00 | 432 | 953 | 959 | 345 |
| 38x13.50R20LT | 11.00 | 508 | 953 | 959 | 345 |
| 38x13.50R24LT | 11.00 | 610 | 953 | 959 | 345 |
| 38x14.50R17LT | 12.00 | 432 | 953 | 959 | 372 |
| 38x14.50R18LT | 12.00 | 457 | 953 | 959 | 372 |
| 38x14.50R20LT | 12.00 | 508 | 953 | 959 | 372 |
| 38x15.50R15LT | 12.00 | 381 | 953 | 959 | 390 |
| 38x15.50R17LT | 12.00 | 432 | 953 | 959 | 390 |
| 38x15.50R18LT | 12.00 | 457 | 953 | 959 | 390 |
| 38x15.50R20LT | 12.00 | 508 | 953 | 959 | 390 |
| 39x13.50R17LT | 11.00 | 432 | 978 | 985 | 345 |
| 40x13.50R17LT | 11.00 | 432 | 1004 | 1010 | 345 |
| 40x13.50R20LT | 11.00 | 508 | 1004 | 1010 | 345 |
| 40x14.50R17LT | 12.00 | 432 | 1004 | 1010 | 372 |
| 40x14.50R18LT | 12.00 | 457 | 1004 | 1010 | 372 |
| 40x14.50R20LT | 12.00 | 508 | 1004 | 1010 | 372 |
| 40x15.50R20LT | 12.00 | 508 | 1004 | 1010 | 390 |
| 40x15.50R22LT | 12.00 | 559 | 1004 | 1010 | 390 |
| 40x15.50R24LT | 12.00 | 610 | 1004 | 1010 | 390 |
| 42x14.50R17LT | 12.00 | 432 | 1055 | 1061 | 372 |
| 42x14.50R20LT | 12.00 | 508 | 1055 | 1061 | 372 |
| 8.00-16.5LT | 6.00 | 419 | 720 | 730 | 203 |
| 8.75-16.5LT | 6.75 | 419 | 748 | 759 | 222 |
| 9.50-16.5LT | 6.75 | 419 | 776 | 787 | 241 |
| 10-16.5LT | 8.25 | 419 | 762 | 773 | 264 |
| 12-16.5LT | 9.75 | 419 | 818 | 831 | 307 |
| 30x9.50-16.5LT | 7.50 | 419 | 750 | 761 | 240 |
| 31x10.50-16.5LT | 8.25 | 419 | 775 | 787 | 266 |
| 33x12.50-16.5LT | 9.75 | 419 | 826 | 838 | 315 |
| 35x12.50 R16.5LT | 10.00 | 419 | 877 | 883 | 318 |
| 37x12.50-16.5LT | 9.75 | 419 | 928 | 939 | 315 |
| 37x14.50-16.5LT | 11.25 | 419 | 928 | 939 | 365 |
| 1 Tyres in Radial construction are identified by the letter "R" in place of "-" (e.g. 24x7.50 R 13LT). 2 Coefficient 'b' for the calculation of Dmax: 1.07. 3 Overall width may exceed this value up to +7 per cent. |  |  |  |  |  |

"

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.