|  | United Nations | ECE/TRANS/WP.29/GRRF/2015/5 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 December 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-ninth session

Geneva, 16-20 February 2015

Item 7(c) of the provisional agenda

Tyres – Regulation No. 54

Proposal for an amendment to Regulation No. 54 (Tyres for commercial vehicles and their trailers)

Submitted by the experts from the European Tyre and Rim Technical Organisation*

The text reproduced below was prepared by the experts from the European Tyre and Rim Technical Organisation (ETRTO) in order to add new tyres for light commercial vehicles (high flotation tyres) in Annex 5 of UN Regulation No. 54. The modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Annex 5, part II, table B, amend to read:

"Table B

- Tyres for light commercial vehicles (high flotation tyres)

- Diagonal and radial

| Tyre-size designation1 | Measuring rim width code | Nominal rim diameter d (mm) | Outer diameter D (mm)2 |  | Section width S (mm)3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Normal | Snow |  |
| 9-15LT | 8.00 | 381 | 744 | 755 | 254 |
| 10-15LT | 8.00 | 381 | 773 | 783 | 264 |
| 11-15LT | 8.00 | 381 | 777 | 788 | 279 |
| 24x7.50-13LT | 6.00 | 330 | 597 | 604 | 191 |
| 27x8.50-14LT | 7.00 | 356 | 674 | 680 | 218 |
| 28x8.50-15LT | 7.00 | 381 | 699 | 705 | 218 |
| 29x9.50-15LT | 7.50 | 381 | 724 | 731 | 240 |
| 30x9.50-15LT | 7.50 | 381 | 750 | 756 | 240 |
| 31x10.50-15LT | 8.50 | 381 | 775 | 781 | 268 |
| 31x11.50-15LT | 9.00 | 381 | 775 | 781 | 290 |
| 31x12.50R15LT | 10.00 | 381 | 775 | 781 | 318 |
| 31x13.50-15LT | 11.00 | 381 | 775 | 781 | 345 |
| 31x15.50-15LT | 12.00 | 381 | 775 | 781 | 390 |
| 32x11.50-15LT | 9.00 | 381 | 801 | 807 | 290 |
| 33x9.50 R15LT | 7.50 | 381 | 826 | 832 | 240 |
| 33x10.50R15LT | 8.50 | 381 | 826 | 832 | 268 |
| 33x10.50R17LT | 8.50 | 432 | 826 | 832 | 268 |
| 33x10.50R18LT | 8.50 | 457 | 826 | 832 | 268 |
| 33x11.50R18LT | 9.00 | 457 | 826 | 832 | 290 |
| 33x12.50-15LT | 10.00 | 381 | 826 | 832 | 318 |
| 33x12.50R17LT | 10.00 | 432 | 826 | 832 | 318 |
| 33x12.50R18LT | 10.00 | 457 | 826 | 832 | 318 |
| 33x12.50R20LT | 10.00 | 508 | 826 | 832 | 318 |
| 33x12.50R22LT | 10.00 | 559 | 826 | 832 | 318 |
| 33x13.50R15LT | 11.00 | 381 | 826 | 832 | 345 |
| 33x15.50R15LT | 12.00 | 381 | 826 | 832 | 390 |
| 34x10.50R17LT | 8.50 | 432 | 851 | 858 | 268 |
| 34x12.50R18LT | 10.00 | 457 | 851 | 858 | 318 |
| 35x12.50-15LT | 10.00 | 381 | 877 | 883 | 318 |
| 35x12.50R17LT | 10.00 | 432 | 877 | 883 | 318 |
| 35x12.50R18LT | 10.00 | 457 | 877 | 883 | 318 |
| 35x12.50R20LT | 10.00 | 508 | 877 | 883 | 318 |

| Tyre-size designation1 | Measuring rim width code | Nominal rim diameter d (mm) | Outer diameter D (mm)2 |  | Section width S (mm)3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Normal | Snow |  |
| 35x12.50R22LT | 10.00 | 559 | 877 | 883 | 318 |
| 35x13.50R15LT | 11.00 | 381 | 877 | 883 | 345 |
| 35x13.50R18LT | 11.00 | 457 | 877 | 883 | 345 |
| 35x13.50R20LT | 11.00 | 508 | 877 | 883 | 345 |
| 35x14.50R15LT | 12.00 | 381 | 877 | 883 | 372 |
| 36x13.50R18LT | 11.00 | 457 | 902 | 908 | 345 |
| 36x14.50R15LT | 12.00 | 381 | 902 | 908 | 372 |
| 36x14.50R17LT | 12.00 | 432 | 902 | 908 | 372 |
| 36x14.50R18LT | 12.00 | 457 | 902 | 908 | 372 |
| 36x15.50R15LT | 12.00 | 381 | 902 | 908 | 390 |
| 37x12.50-15LT | 10.00 | 381 | 928 | 934 | 318 |
| 37x12.50 R17LT | 10.00 | 432 | 928 | 934 | 318 |
| 37x12.50R18LT | 10.00 | 457 | 928 | 934 | 318 |
| 37x12.50R20LT | 10.00 | 508 | 928 | 934 | 318 |
| 37x12.50R22LT | 10.00 | 559 | 928 | 934 | 318 |
| 37x13.50R15LT | 11.00 | 381 | 928 | 934 | 345 |
| 37x13.50R17LT | 11.00 | 432 | 928 | 934 | 345 |
| 37x13.50R18LT | 11.00 | 457 | 928 | 934 | 345 |
| 37x13.50R20LT | 11.00 | 508 | 928 | 934 | 345 |
| 37x13.50R22LT | 11.00 | 559 | 928 | 934 | 345 |
| 37x13.50R24LT | 11.00 | 610 | 928 | 934 | 345 |
| 37x14.50-15LT | 12.00 | 381 | 928 | 934 | 372 |
| 38x13.50R17LT | 11.00 | 432 | 953 | 959 | 345 |
| 38x13.50R20LT | 11.00 | 508 | 953 | 959 | 345 |
| 38x13.50R24LT | 11.00 | 610 | 953 | 959 | 345 |
| 38x14.50R17LT | 12.00 | 432 | 953 | 959 | 372 |
| 38x14.50R18LT | 12.00 | 457 | 953 | 959 | 372 |
| 38x14.50R20LT | 12.00 | 508 | 953 | 959 | 372 |
| 38x15.50R15LT | 12.00 | 381 | 953 | 959 | 390 |
| 38x15.50R17LT | 12.00 | 432 | 953 | 959 | 390 |
| 38x15.50R18LT | 12.00 | 457 | 953 | 959 | 390 |
| 38x15.50R20LT | 12.00 | 508 | 953 | 959 | 390 |
| 39x13.50R17LT | 11.00 | 432 | 978 | 985 | 345 |
| 40x13.50R17LT | 11.00 | 432 | 1004 | 1010 | 345 |
| 40x13.50R20LT | 11.00 | 508 | 1004 | 1010 | 345 |
| 40x14.50R17LT | 12.00 | 432 | 1004 | 1010 | 372 |
| 40x14.50R18LT | 12.00 | 457 | 1004 | 1010 | 372 |
| 40x14.50R20LT | 12.00 | 508 | 1004 | 1010 | 372 |
| 40x15.50R20LT | 12.00 | 508 | 1004 | 1010 | 390 |
| 40x15.50R22LT | 12.00 | 559 | 1004 | 1010 | 390 |
| 40x15.50R24LT | 12.00 | 610 | 1004 | 1010 | 390 |
| 42x14.50R17LT | 12.00 | 432 | 1055 | 1061 | 372 |
| 42x14.50R20LT | 12.00 | 508 | 1055 | 1061 | 372 |
| 8.00-16.5LT | 6.00 | 419 | 720 | 730 | 203 |
| 8.75-16.5LT | 6.75 | 419 | 748 | 759 | 222 |
| 9.50-16.5LT | 6.75 | 419 | 776 | 787 | 241 |
| 10-16.5LT | 8.25 | 419 | 762 | 773 | 264 |
| 12-16.5LT | 9.75 | 419 | 818 | 831 | 307 |
| 30x9.50-16.5LT | 7.50 | 419 | 750 | 761 | 240 |
| 31x10.50-16.5LT | 8.25 | 419 | 775 | 787 | 266 |
| 33x12.50-16.5LT | 9.75 | 419 | 826 | 838 | 315 |
| 35x12.50 R16.5LT | 10.00 | 419 | 877 | 883 | 318 |
| 37x12.50-16.5LT | 9.75 | 419 | 928 | 939 | 315 |
| 37x14.50-16.5LT | 11.25 | 419 | 928 | 939 | 365 |

1	Tyres in Radial construction are identified by the letter "R" in place of "-" (e.g. 24x7.50 R 13LT).

2	Coefficient 'b' for the calculation of Dmax: 1.07.

3	Overall width may exceed this value up to +7 per cent. "

II.	Justification

These tyres are commonly used in some of the countries covered by the 1958 Agreement.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.