|  | United Nations | ECE/TRANS/WP.29/GRRF/2015/15 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 December 2014 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Seventy-ninth session

Geneva, 16-20 February 2015

Item 7(d) of the provisional agenda

Tyres – Regulation No. 106

Proposal for a corrigendum to Regulation No. 106 (Tyres for agricultural vehicles)

Submitted by the experts from the European Tyre and Rim Technical Organisation *

The text reproduced below was prepared by the experts of the European Tyre and Rim Technical Organisation (ETRTO), introducing corrigendum to UN Regulation No. 106. The modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Annex 5,

Table 5, amend to read:

# "Table 5 (1 of 3)

# Agricultural implement tyres - Normal section sizes

| Tyre size designation | Theoretical rim width code (A1) | Nominal section width (S1) (mm) | Overall diameter (D) (mm) |  | Nominal rim diameter (d) (mm) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | (*) |  |
| 4.00 - 16 IMP | 3 | 114 | 618 |  | 406 |
| 4.00 – 18 IMP | 3 | 112 | 672 | 688 | 457 |
| 4.00 4.50 – 19 IMP | 3 | 114 124 | 694 720 | 733 | 483 |
| 4.00 - 21 IMP | 3 | 112 | 765 |  | 533 |
| … |  |  |  |  |  |
| 4.50 - 16 IMP | 3 | 123 | 647 |  | 406 |
| 4.50 - 19 IMP | 3 | 124 | 720 | 733 | 483 |
| 4.80 – 8 IMP | 3,75 | 121 | 423 | 449 | 203 |
| … |  |  |  |  |  |

# Table 5 (2 of 3)

# Agricultural implement tyres - Normal section sizes

| Tyre size designation | Theoretical rim width code (A1) | Nominal section width (S1) (mm) | Overall diameter (D) (mm) |  | Nominal rim diameter (d) (mm) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | (*) |  |
| …. |  |  |  |  |  |
| 6.0 - 12 IMP | 5 | 155 | 569 |  | 305 |
| 6.00 - 12 IMP | 5 | 152 | 579 |  | 305 |
| 6.00 – 16 IMP | 4 | 158 | 712 | 729 | 406 |
| 6.00 – 19 IMP | 4,5 | 169 | 810 |  | 483 |
| 6.00 - 20 IMP | 4,5 | 169 | 830 |  | 508 |
| …. |  |  |  |  |  |

# Table 5 (3 of 3)

# Agricultural implement tyres - Normal section sizes

| Tyre size designation | Theoretical rim width code (A1) | Nominal section width (S1) (mm) | Overall diameter (D) (mm) |  | Nominal rim diameter (d) (mm) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | (*) |  |
| ….. |  |  |  |  |  |
| 8 - 16 IMP | 6 | 211 | 795 |  | 406 |
| 8.00 – 6 IMP | 7 | 203 | 452 |  | 152 |
| 8.00 - 12 IMP | 5 | 214 | 710 |  | 305 |
| 8.00 – 16 IMP | 6 | 206 | 808 |  | 406 |
| 8.00 - 19 IMP | 6 | 214 | 888 |  | 483 |
| …. |  |  |  |  |  |

"

# Table 6, amend to read

# "Table 6 (2 of 3)

# Agricultural implement tyres - Low section sizes

| Tyre size designation | Theoretical rim width code (A1) | Nominal section width (S1) (mm) | Overall diameter (D) (mm) |  | Nominal rim diameter (d) (mm) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | (*) |  |
| …… |  |  |  |  |  |
| 15.0/55 - 17 IMP | 13 | 391 | 850 | 872 | 432 |
| 10.5/65 - 16 IMP | 9 | 274 | 755 |  | 406 |
| 11.0/60 - 16 IMP | 9 | 281 | 742 |  | 406 |
| ……. |  |  |  |  |  |
| 10.0/75 - 12 IMP | 9 | 264 | 685 |  | 305 |
| 10.0/75 - 15.3 IMP | 9 | 258 264 | 785 760 | 780 | 389 |
| 10.0/75 - 15.3 IMP | 9 | 264 | 760 | 780 | 389 |
| 10.0/75 - 16 IMP | 9 | 264 | 805 |  | 406 |
| …. |  |  |  |  |  |

# "

II.	Justification

1.	With revision 1 Amendment 1 (supplement 7), 1 line of table 5 were incorrectly amended: 4.50 – 19 IMP and 1 line of table 6: 10.0/75 – 15.3 IMP.

2.	Revision 1 deleted 3 lines from table 5 compared to the original version: 6.00 – 
19 IMP, 8.00 – 6 IMP and 8.00 – 16 IMP.

3.	This was not the intention of the working group that drafted the extension to UN Regulation No. 90 and is different to brake linings where the approval of all parts contained within a Test Group are covered by a single certificate.

4.	In table 6, the Nominal rim diameter is incorrect: 406.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2012–2016 (ECE/TRANS/224, para. 94 and ECE/TRANS/2012/12, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.