Proposal for an amendment to Regulation No. 54 (Tyres for commercial vehicles and their trailers).

The text reproduced below was prepared by the expert from France, introducing an amendment to the UN Regulation N° 54. The modifications to the existing text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.		Proposal

3. Markings

Paragraph 3.1.5, Amend to read:

3.1.5.	The inscription M+S or M.S or M&S if the tyre is classified in the category of use "snow tyre"; or if the tyre is classified in the category of use “special use tyre” when declared by the tyre manufacturer at paragraph 4.1.3. as complying also with the definition given in §2.2.2.

Paragraph 3.1.12, Amend to read:

3.1.12.	The inscription "MPT" (or alternatively "ML" or "ET") and /or "POR" if the tyre is classified in the category of use "special use tyre". In addition, they may also bear the inscription M+S or M.S or M&S.

ET means Extra Tread, ML stands for Mining and Logging, MPT means Multi-Purpose Truck and POR means Professional Off Road 6/;

4. Application for approval

Paragraph 4.1.3, Amend to read:

4.1.3.		Category of use (normal tyre, snow tyre, special use tyre);

4.1.3.1.	For the tyres belonging to the category of use “special use tyre” those which may bear the inscription M+S or M.S or M&S.

6. Specifications

Paragraph 6.1.5.3.3, Amend to read:

6.1.5.3.3. For snow tyres of the category of use “snow tyre” the outer diameter (Dmax) established in conformity with the above may be exceeded by 1 per cent.

II.	Justification

1.	Current UN Regulation N° 54 does not consider the case where a tyre designed for a “special” application can also fulfil the definition of a “snow” tyre. In this case, if declared by the tire manufacturer, the tyre has to be marked with the inscription M+S or M.S or M&S while remaining in the category of use “special”.