| Submitted by the expert from CLEPA | Informal document GRSP-57-03 (57th GRSP, 18-22 May 2015, agenda item 7) |
| --- | --- |

Proposal of amendments to ECE/TRANS/WP.29/GRSP/2015/16 - Simplified test method description for Annex 14 - Proposal for Supplement 6 to the 06 series of amendments to Regulation No. 16 (Safety-belts).

I.	Proposal

Annex 14, paragraph 2.2.3., amend to read:

"2.2.3.	Results

…

A simplified, adapted method could be, e.g. the measurement of the speed at the chest reference point when its forward displacement is at 300 mm, the use of a reference chest speed measured at 300 mm forward displacement carried out in a test without an airbag in front of it, during the conformity of production test to be considered in the control of conformity control plan."

II.	Justification

- The simplified test method is mainly used for system homologation. An increasing number of this kind of approvals and respective need for COP testing can be observed.

- In the view of the industry and after discussion with some technical services, the proposed text should be replacing the current version in GRSP/2015/16. France, Nederland and CLEPA did prepare the document GRSP/2015/16 and CLEPA would appreciate further clarification of the text.

- The speed of the reference point of the chest should be determined at the conformity test.

- The important detail, to compare this determined speed in COP test with some reference chest speed defined in a control plan is missing in the published version of the GRSP/2015/16.

- This defined reference speed of the chest at 300mm forward displacement can be defined either

  - In dynamic test performed with and without the airbag or other system homologation support material.

  - In simulation based on historical data or expertise.

  - In any other engineering method, which the related technical service can follow in the regular review of the COP procedure