| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/824 22 January 2002 ENGLISH Original: ENGLISH and FRENCH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

DRAFT SUPPLEMENT 4 TO REGULATION No. 39

(Speedometers)

Note:  The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its nineteenth session, following the recommendation by WP.29 at its one-hundred-and-twenty-fifth session.  It is based on document TRANS/WP.29/2001/56, not amended (TRANS/WP.29/815, para. 130).

___________

GE.02-

Paragraph 1., amend to read (including the addition of a new footnote 1/):

“1.		SCOPE

This Regulation applies to the approval of vehicles of categories L, M and N. 1/

__________________

1/ As defined in annex 7 of the Consolidated Resolution on the Construction of Vehicles (R.E.3) (document TRANS/WP.29/78/Rev.1/Amend.2)."

Paragraph 2.5., the reference to  footnote */ and footnote */, renumber as footnote 2/.

Paragraph 4.4.1., the reference to footnote 1/ and footnote 1/ (former), renumber as footnote 3/.

Paragraph 5.1.5., should be deleted.

Annex 3, paragraph 2., amend to read:

“........

In the case of vehicles of categories M and N:

0 < (V1 – V2) < 0.1 V2 + 6 km/h;

In the case of vehicles of categories L3, L4 and L5:

0 < (V1 – V2) < 0.1 V2 + 8 km/h;

In the case of vehicles of categories L1 and L2:

0 < (V1 – V2) < 0.1 V2 + 4 km/h.”

________