| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/851 8 May 2002 ENGLISH Original: ENGLISH and FRENCH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

### World Forum for Harmonization of Vehicle Regulations (WP.29)

DRAFT SUPPLEMENT 3 TO THE 02 SERIES OF AMENDMENTS

TO REGULATION No. 78

(Braking of motorcycles)

Note:   The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twentieth session, following the recommendation by WP.29 at its one-hundred-and-twenty-sixth session.  It is based on document TRANS/WP.29/2002/5, not amended (TRANS/WP.29/841, para. 145).

____________

# Annex 3,

Paragraphs 1.1.1. and 1.1.2.,amend to read:

"1.1.1.	The performance prescribed for braking devices shall be based on the stopping distance and/or the mean fully developed deceleration.  The performance of a braking device shall be determined by measuring the stopping distance in relation to the initial speed of the vehicle and/or measuring the mean fully developed deceleration during the test.

1.1.2.	The stopping distance shall be the distance covered by the vehicle from the moment when the driver begins to actuate the control of the braking system until the moment when the vehicle stops; the initial vehicle speed, v1, shall be the speed at the moment when the driver begins to actuate the control of the braking system; the initial speed shall not be less than 98 per cent of the prescribed speed for the test in question.  The mean fully developed deceleration, (dm), shall be calculated as the deceleration averaged with respect to distance over the interval vb to ve according to the following formula:

dm=vb2− ve225.92(se− sb)       m/s2

Where:

dm	 = mean fully developed deceleration

v1	 = as defined above

vb	 = vehicle speed at 0.8 v1 in km/h

ve	 = vehicle speed at 0.1 v1 in km/h

sb	 = distance travelled between v1 and vb in metres

se	 = distance travelled between v1 and ve in metres

The speed and distance shall be determined using instrumentation having an accuracy of  1 per cent at the prescribed speed for the test.  The "dm" may be determined by other methods than the measurement of speed and distance; in this case, the accuracy of the "dm" shall be within  3 per cent."

Paragraph 1.4.1., amend to read:

"1.4.1.	General

The limits prescribed for minimum performance are those laid down hereunder for each category of vehicle; the vehicle shall satisfy both the prescribed stopping distance and the prescribed mean fully developed deceleration for the relevant vehicle category, but it may not be necessary to measure both parameters."

Paragraph 2.1.1., amend to read (footnote */ not amended):

"2.1.1.	Prescribed speed	v = 40 km/h */ for categories L1 and L2,

v = 60 km/h */ for categories L3 and L4."

Paragraph 2.2.1., amend to read (footnote */ not amended):

"2.2.1.	Prescribed speed	v = 40 km/h */ for categories L1 and L2,

v = 60 km/h */ for categories L3, L4 and L5."

____________