UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/918

###### 17 April 2003

- ENGLISH

- Original: ENGLISH and FRENCH

### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

# DRAFT SUPPLEMENT 3 TO REGULATION No. 98

(Headlamps with gas-discharge light sources)

Note:	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-third session, following the recommendation by WP.29 at its one-hundred-and-twenty-ninth session.  It is based on document TRANS/WP.29/2003/11, not amended (TRANS/WP.29/909, para. 123).

__________

Annex 4, paragraph 1.1.1.1., add a new subparagraph (d) to read:

"(d)	In the case of a passing beam designed to provide bend lighting with the addition of a light source, this light source shall be switched on for 1 minute, and switched off for 9 minutes during the activation of the passing beam only (see annex 4 - appendix 1)."

Insert new Annex 4 - appendix 1, to read:

"Annex 4 - Appendix 1

OVERVIEW OF OPERATIONAL PERIODS

CONCERNING TEST FOR STABILITY OF PHOTOMETRIC PERFORMANCE

| Abbreviations: P: | passing beam lamp |
| --- | --- |
| D: | driving beam lamp (D1 + D2 means two driving beams) |
| F: | front fog lamp |
| : | means a cycle of 15 minutes off and 5 minutes lit. |
| : | means a cycle of 9 minutes off and 1 minutes lit |

All following grouped headlamps and front fog lamps together with the added marking symbols are given as examples and are not exhaustive.

1.	P or D or F (HC or HR or B)

| P, D or F |  |  |
| --- | --- | --- |
| Additional light source of bend light |  |  |

0	         6	        12h

2.	P+F (HC B)  or P+D (HCR)

| Additional light source of bend light |  |  |
| --- | --- | --- |
| D or F |  |  |
| P |  |  |

0	         6	        12h

3.	P+F (HC B/)  or HC/B or P+D (HC/R)

| D or F |  |  |
| --- | --- | --- |
| P |  |  |
| Additional light source of bend light |  |  |

0	        6	        12h

_________"