UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/921

###### 17 April 2003

- ENGLISH

- Original: ENGLISH and FRENCH

### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

# DRAFT SUPPLEMENT 1 TO REGULATION No. 108

(Retreaded pneumatic tyres)

Note:	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-third session, following the recommendation by WP.29 at its one-hundred-and-twenty-ninth session.  It is based on document TRANS/WP.29/2003/7, not amended (TRANS/WP.29/909, para. 126).

__________

Paragraph 1.2., amend to read:

“ .... capability below 120 km/h or above 300 km/h.”

Paragraph 2.26., and its footnote 5/, amend to read:

“ .......

..... of Australia (TRAA) 4/:

"Standards Manual"

- The Associaçiao Latino Americana de Pneus e Aros (ALAPA) 5/:

"Manual de Normas Technicas"

(g)	The Scandinavian ............

________________

..........

.......................... Australia

5/ ALAPA, Av. Paulista, 2444-12° andar, conj. 124, 01310 Sao Paulo, SP Brazil

6/ .........”

Paragraph 2.34.2., the table, amend to read:

“

| Speed symbol | Corresponding speed (km/h) |
| --- | --- |
| ..... | ..... |
| V | 240 |
| W | 270 |
| Y | 300 |

”

Paragraph 2.35.2., amend  to read:

"2.35.2.	for speeds greater than 210 km/h but not exceeding 300 km/h, the maximum load rating shall not exceed the percentage of the value associated with the load capacity index of the tyre, given in the table below, with reference to the speed capability of the vehicle to which the tyre is to be fitted.

| Tyre speed symbol | Maximum speed – km/h | Maximum load rating - % |
| --- | --- | --- |
| V | 210 215 220 225 230 235 240 | 100,0 98,5 97,0 95,5 94,0 92,5 91,0 |
| W | 240 250 260 270 | 100 95 90 85 |
| Y | 270 280 290 300 | 100 95 90 85 |

For intermediate maximum speeds a linear interpolation of the maximum load rating is permissible."

Insert a new paragraph 6.1.1., to read:

"6.1.1.	High speed tyres which have only the inscription "ZR" within the tyre size designation and do not bear a service description shall not be retreaded."

Paragraph 6.6.10., amend to read:

" .... and the maximum shall be 300 km/h (“Y” speed symbol)."

Annex 7,

Paragraph 1.2., the table, add a new row with "W and Y" for "Speed Category" and the values

"3.2" for "Radial Tyres Standard" and "3.6" for "Radial Tyres Reinforced".

Paragraph 2.2.2., amend to read:

"2.2.2.	the maximum load rating associated with a maximum speed (see paragraph 2.35.2. of this Regulation) of:

- 240 km/h in the case of tyres of Speed symbol "V",

- 270 km/h in the case of tyres of Speed symbol "W",

- 300 km/h in the case of tyres of Speed symbol "Y"."

____________