UNITED

- NATIONS

Distr.

Economic and Social	GENERAL

Council					TRANS/WP.29/927

23 July 2003

ENGLISH

- Original:  ENGLISH and FRENCH

# ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

DRAFT SUPPLEMENT 10 TO REGULATION No. 4

(Rear registration plate illumination)

Note :	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-fourth session, following the recommendation of WP.29 at its one-hundred-and-thirtieth session.  It is based on documents TRANS/WP.29/2003/20 and Corr.1 (French only), not amended (TRANS/WP.29/926, para. 95).

__________

Paragraphs 5. to 5.2.1., replace by the following text (footnote 3/ not modified):

"5.	GENERAL SPECIFICATIONS

Each device shall satisfy the provisions of paragraph 9. 3/

5.1.	The devices for the illumination of rear registration plates shall be so constructed that the whole surface of the plate will be visible within the angles given in annex 4.

  - All measurements shall be made with the standard filament lamp of the category prescribed by the manufacturer, the supply voltage being so regulated as to produce the reference luminous flux.  All measurements on the devices with non-replaceable light sources shall be made at 6.75 V, 13.5 V or 28.0 V respectively.

  - In the case of light sources supplied by a special power supply, the above test voltages shall be applied to the input terminals of that power supply.  The test laboratory may require from manufacturer the special power supply needed to supply the light sources."

Paragraph 5.3. (former), renumber as paragraph 5.4.

Paragraph 7., amend to read:

"......intended to be illuminated by the device concerned.

When the device has one outer edge of the illuminating surface that is parallel to the surface of the registration plate, the extremity of the illuminating surface of the device which is furthest from the surface of the plate is the middle point of the edge of the illuminating surface, which is parallel to the plate and is furthest from the surface of the plate.

The device must be so designed that no light is emitted ......"

Paragraph 8., amend to read (including a new footnote 4/):

"8.	Luminance measurements shall be made on a diffuse colourless surface with known diffuse reflection factor. 4/  The diffuse colourless surface shall have the dimensions of the registration plate or the dimension exceeding one measuring point.  Its centre shall be placed in the centre of the positions of the measuring points.

This diffuse colourless surface(s) shall be placed in the position normally occupied by the registration plate and 2 mm in front of its holder.

Luminance measurements shall be made perpendicularly to the surface of the diffuse colourless surface with the tolerance of 5° in each direction at the points shown in annex 3 to this Regulation, each point representing a circular area of 25 mm in diameter.

The measured luminance shall be corrected for the diffuse reflection factor 1.0.

_________________________

4/	CIE Publication No 17 – 1970, paragraph 45-20-040."

__________