UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/962

###### 23 January 2004

- ENGLISH

- Original: ENGLISH and FRENCH

### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

# DRAFT SUPPLEMENT 5 TO THE 05 SERIES OF AMENDMENTS

# TO REGULATION No. 14

(Safety-belt anchorages)

Note :	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-fifth session, following the recommendation by WP.29 at its one-hundred-and-thirty-first session.  It is based on document TRANS/WP.29/2003/82, as amended (TRANS/WP.29/953, para. 103).

____________

Paragraph 5.3.3., amend to read:

"5.3.3.	However, for outboard seating positions, other than front, of vehicles of category N1, shown in annex 6 and marked with the symbol Ø, two lower anchorages are allowed, where there exists a passage ........... "

Annex 6,

The table, amend to read:

"

| VEHICLE | FORWARD FACING SEATING POSITIONS |  |  |  | REAR |
| --- | --- | --- | --- | --- | --- |
| CATEGORY | OUTBOARD |  | CENTRE |  | FACING |
|  | FRONT | OTHER | FRONT | OTHER |  |
| M1 | 3 | 3 | 3 | 3 | 2 |
| M2 ≤ 3.5 tonnes | 3 | 3 | 3 | 3 | 2 |
| M3 & M2 > 3.5 tonnes | 3  | 3 or 2 ╬ | 3 or 2 ╬ | 3 or 2 ╬ | 2 |
| N1 | 3 | 3 or 2 Ø | 3 or 2 * | 2 | 2 |
| N2 & N3 | 3 | 2 | 3 or 2 * | 2 | 2 |

"

Insert new paragraphs 14.6. to 14.11., to read:

"14.6.	As from the official date of entry into force of Supplement 5 to the 05 series of amendments, no Contracting Party applying this Regulation shall refuse to grant ECE approvals under this Regulation as modified by Supplement 5 to the 05 series of amendments."

- 14.7.	For vehicles not affected by Supplement 5 to the 05 series of amendments to this Regulation the existing approvals shall remain valid, if they had been granted in compliance with the 05 series of amendments, up to its Supplement 3.

14.8.	As from 20 February 2005 for vehicles of category M1, Contracting Parties applying this Regulation shall grant ECE approvals only if the requirements of this Regulation, as amended by Supplement 5 to the 05 series of amendments, are satisfied.

14.9.	As from 20 February 2007 for vehicles of category M1, Contracting Parties applying this Regulation may refuse to recognize approvals which were not granted in accordance with Supplement 5 to the 05 series of amendments to this Regulation.

14.10.	As from 16 July 2006 for vehicles of category N, Contracting Parties applying this Regulation shall grant approval only if the vehicle type satisfies the requirements of this Regulation as amended by the Supplement 5 to the 05 series of amendments.

14.11.	As from 16 July 2008 for vehicles of category N, Contracting Parties applying this Regulation may refuse to recognize approvals not granted in accordance with Supplement 5 to the 05 series of amendments to this Regulation."

____________