UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/974

###### 4 February 2004

- ENGLISH

- Original: ENGLISH AND FRENCH

# ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

DRAFT SUPPLEMENT 8 TO THE 02 SERIES OF AMENDMENTS

TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Note :	The text reproduced below was adopted by the Administrative Committee (AC.1) of the amended 1958 Agreement at its twenty-fifth session, following the recommendation by WP.29 at its one-hundred-and-thirty-first session.  It is based on document TRANS/WP.29/2003/61, as amended (TRANS/WP.29/953, para. 116).

__________

Paragraphs 2.7.1., amend to read :

"2.7.1.	Light source  */

        - "Light source" means one or more elements for visible radiation, which may be assembled with one or more transparent envelopes and with a base for mechanical and electrical connection.

A light source may also be constituted by the extreme outlet of a light-guide, as part of a distributed lighting or light-signalling system not having a built-in outer lens;

2.7.1.1.1.	"Replaceable light source" means a light source which is designed to be inserted in and removed from the holder of its device without tool;

        - "Non-replaceable light source" means a light source which can only be replaced by replacement of the device to which this light source is fixed;

in case of a light source module:  a light source which can only be replaced by replacement of the light source module to which this light source is fixed;

2.7.1.1.3.	"Light source module" means an optical part of a device which is specific to that device, is containing one or more non-replaceable light sources, and is only removable from its device with the use of tool(s);

2.7.1.1.4.	"Filament light source" (filament lamp) means a light source where the element for visible radiation is one or more heated filaments producing thermal radiation;

2.7.1.1.5.	"Gas-discharge light source" means a light source where the element for visible radiation is a discharge arc producing electro-luminescence / fluorescence;

2.7.1.1.6.	"Light-emitting diode" (LED) means a light source where the element for visible radiation is one or more solid state junctions producing injection-luminescence / fluorescence;

2.7.1.2.	"Electronic light source control gear" means one or more components between supply and light source to control voltage and/ or electrical current of the light source;

        - "Ballast" means an electronic light source control gear between supply and light source to stabilise the electrical current of a gas-discharge light source;

2.7.1.2.2.	"Ignitor" means an electronic light source control gear to start the arc of a gas-discharge light source.

__________

*/	For clarification see annex 10

Paragraph 6.7.1., amend to read :

"6.7.1.		Presence

Devices of S1 or S2 categories : 	mandatory on all categories of vehicles.

Devices of S3 category : 	mandatory on M1 and N1 categories of vehicles, except for chassis-cabs and those N1 category vehicles with open cargo space;

optional on other categories of vehicles"

_______________