Informal Document No._10____

Submission from the United Kingdom regarding supplementary amendments to those contained in document TRANS/WP.29/GRRF/2001/4 – Agenda Item 6.3

Amend proposal in 2001/4 for a new paragraph 3.1.11 to read:

3.1.11	In the case of temporary use spare tyres, the words “TEMPORARY USE ONLY” in upper case characters at least 12,7mm high.

Add further new paragraph 3.1.11.1 to read:

3.1.11.1	In addition, in the case of “T” type temporary use spare tyres, the legend

“INFLATE TO 420kPa (4,2Bar – 60PSI)” in upper and lower case characters as shown and at least 12,7mm high.

Justification:  The addition of size requirements for the marking and the inflation pressure details for “T” type temporary use spare tyres will complete the marking requirements and will align with proposals in the draft Global Technical Regulations for Tyres and with paragraph 4.2.2 of the industry global proposal TRANS/WP.29/GRRF/1999/7 otherwise referred to as

GTS 2000.