# Informal document N°10

(51st GRRF, 4-8 February 2002

agenda item 1.5 )

PROPOSAL FOR A DRAFT AMENDMENT

TO ECE-R48

#### Proposal

2.7.12.	“Stop lamp” means the lamp used to indicate to other road users to the rear of the vehicle that the vehicle may be decelerating.

6.7.7.	… the engine to operate.

The stop lamps may be activated by the operation of any other device or function which is capable of generating a retarding force.

#### Justification

GRRF-50 requested OICA to prepare a new proposal for amendment to ECE-R48 on stop lamp activation, taking into consideration the comments from the GRRF experts.

In this new proposal:

- The definition does not contain any functioning prescription.

- The stop lamps must be illuminated by the action of the driver for slowing down the vehicle. GRRF-50 generally agreed that the activation of the brakes by ACC is assimilated to an activation of the brakes by the driver.

- There is no mention of any deceleration figure.

- The very low decelerations (e.g. engine brake on M1 vehicles) are not considered.

- The dynamic driving help systems may activate the stop lamps when generating a retarding force.

##### Information

For information, here are the current prescriptions of ECE-R48.02 for the respective paragraphs.

2.7.12.	“Stop lamp” means the lamp used to indicate to other road-users to the rear of the vehicle that its driver is applying the service brake.

The stop lamps may be activated by the application of a retarder or a similar device;

6.7.7.	Electrical connections

Must light up when the service brake is applied. The stop lamps need not function if the device which starts and/or stops the engine is in a position which makes it impossible for the engine to operate.

The stop lamps may be activated by the application of a retarder or a similar device.

____________