Proposal for Supplement 4 to the 07 series of amendments to

Regulation No. 14 (Safety-belt anchorages)

I.		Proposal

Paragraph 5.1.5., amend to read:

"5.1.5. 		The angles α1 and α2 are respectively the angles between a horizontal plane

through the point H1 and planes perpendicular to the median vertical

longitudinal plane of the seat and passing through the R-point and
the points L1 and L2.

If the seat is adjustable, this requirement shall be fulfilled also for the H-points of all normal driving or riding position, as indicated by the vehicle manufacturer.

II.		Justification

The current text of Supplement 3 to the 07 series of amendments to Regulation No. 14 (Safety-belt anchorages) contains a scientific error regarding the establishing of the angles α1 and α2.

Furthermore an erroneous reference to a point “H1” is made which should read “R-point”.

The Corrigendum rectifies these errors.